Working Draft	T13

American National	Project 1699D

Standard

Revision 1d

June 20, 2005

Information Technology -

AT Attachment - 8 ATA/ATAPI Command Set

(ATA8-ACS)

This is a draft proposed American National Standard of Accredited Standards Committee INCITS.  As such

this is not a completed standard.  The T10 Technical Committee may modify this document as a result of

comments received during public review and its approval as a standard.  Use of the information contained

here in is at your own risk.

Permission is granted to members of INCITS, its technical committees, and their associated task groups to reproduce this document for the purposes of INCITS standardization activities without further permission, provided this notice is included.  All other rights are reserved.  Any commercial or for-profit replication or republication is prohibited.

T13 Technical Editor:

Curtis E.  Stevens

Western Digital

20511 Lake Forest Drive

Lake Forest, CA 92630

USA

Tel:     949-672-7933

E-Mail: Curtis.Stevens@WDC.com

Reference number

ISO/IEC ****** :200x

ANSI INCITS.*** - xxxx

Printed July, 30, 2026 7:39PM

Points of Contact:

T13 Chair					T13 Vicechair

Dan Colgrove					Jim Hatfield

Hitachi Global Storage Technologies		Seagate Technology

2903 Carmelo Dr				389 Disc Drive

Henderson, NV 89502				Longmont CO 80503

Tel:	702-614-6119				Tel:	720-684-2120

Fax:	702-614-7955				Fax:	720-684-2711

INCITS Secretariat

NCITS Secretariat 				Tel:	202-737-8888

1250 Eye Street, NW  Suite 200 		Fax:	202-638-4922

Washington, DC 20005

Email:  INCITS@ITIC.ORG

T13 Reflector

See the T13 Web Site at http://www.t13.org for reflector information.

T13 Web Site

http://www.t13.org

T13 Anonymous FTP Site

ftp.t13.org

Document Distribution

INCITS Online Store		http://www.techstreet.com/incits.html

managed by Techstreet 	Telephone: 1-734-302-7801

1327 Jones Drive 		or 1-800-699-9277

Ann Arbor, MI 48105 		Facsimile: 1-734-302-7811

or

Global Engineering 		http://global.ihs.com/

15 Inverness Way East 		Telephone: 1-303-792-2181

Englewood, CO 80112-5704 	or 1-800-854-7179

Facsimile: 1-303-792-2192

DOCUMENT STATUS

| Revision History |  |  |
| --- | --- | --- |
| Revision | Date | Description |
| 0 | August 17, 2004 | - Initial revision created from ATA/ATAPI-7 Volume 1 (1532D Rev 4b). - Removed 3 volume structure and changed abstract to reflect a command set document - Removed sections on signal, bit, and timing conventions - Removed clause 5, I/O register descriptions - Kept informative annex’s A-C |
| 1 | September 7, 2004 | - Restructured all commands to conform to format agreed to in e04139r4. - Added a section defining status bits - Added a section defining error bits - Added a section describing protocols - Added a section describing interrupt reason - Incorporated single log table e04143r0 table. Did not add wording to status that host VS pages are common between SMART and GPL. - Removed prohibited statements from command definitions. These reside in the feature set definitions. - [Editors Note: Fix N/A vs Reserved] - [Editors Note: Do a consistency pass for Body Text] - [Editors Note: Do a consistency pass on mandatory and optional statements] - [Editors Note: Add section and diagrams to make clear the realationship of this doc to other documents.] - [Editors Note: Under Error Outputs, error bit 0 is sometimes obsolete, and sometimes N/A. I do not think this inconsistent documentation is correct. Bit 1 should always be obsolete, N/A, or is it time for reserved] |
| 1a | October 6, 2004 | - Replaced duplicate normal outputs tables with a see clause to the first usage. - Replaced many error outputs with a see clause. - [Editors Note: Fix N/A vs Reserved] - [Editors Note: Do a consistency pass for Body Text] - [Editors Note: Do a consistency pass on mandatory and optional statements] - [Editors Note: Add section and diagrams to make clear the realationship of this doc to other documents.] - [Editors Note: Under Error Outputs, error bit 0 is sometimes obsolete, and sometimes N/A. I do not think this inconsistent documentation is correct. Bit 1 should always be obsolete, N/A, or is it time for reserved] |
| 1b | February 17, 2005 | - Stripped unused informative references - Changed the definition of ordered and unordered lists to make the numeric form ordered. - Changed all the lists to conform to the changes in #2 - Reserved 5Ch-5Fh for TCG. This was done by added command headers and a reserved statement in the command section. Also maerked the commands with T in the command matrix table. - Moved Normal and Error Outputs to their own section. Commands now have a hotlink to the tables. - Moved IDENTIFY (PACKET) DATA to its own section (Input Data) - Changed more of the titles to be consistent with the front cover. - Deleted unused definitions - Changed the general feature set to only apply to non-packet devices - Changed the packet feature set to include all the commands in the packet feature set. - The command prohibitions are now found in the feature set description and not with each command. - Integrated e04127r0 – This makes SATA signatures reserved with no description. |
| 1c | April 15, 2005 | - Added proposal e04143r1 – Notes that the Host Vendor Specific pages are common to both SMART READ LOG and READ LOG EXTENDED - Added e04130r2 – Tightens the definition of SMART first polling time. Also adds a field that enables longer times. - Added proposal e05103r0 – Changes the features register to log page specific for read and write log commands - Added e04153r1 – Historical annex of command documentation to Annex B - Added e05102r1 as amended – Reserves some set features and DCO fields as vendor specific - Added back in FEATURE SET Clause infront of the description |
| 1d | June 20, 2005 | - Added code 1Dh to IDENTFY DEVICE for ATA/ATAPI-7 Table 16. - Added placeholder for reporting alignment - Updated SET FEATURES table 41 to include reserved entries instead of just saying all other entries are reserved. - Partially Incorporated e04129r5. Assigned Set Features 0Bh and 8Bh for the enable and disable capability. Had to make several modifications to the proposal to fill in missing pieces. Unable to fill in IDENTIFY DEVICE info, data is incomplete. Stopped incorporation |

American National Standard

for Information Technology 

AT Attachment - 8 ATA/ATAPI Command Set (ATA8-ACS)

Secretariat

Information Technology Industry Council

Approved mm dd yy

American National Standards Institute, Inc.

Abstract

This standard specifies the AT Attachment command set between host systems and storage devices.  It provides a common command set for systems manufacturers, system integrators, software suppliers, and suppliers of intelligent storage devices.  It includes the Packet Command feature set implemented by devices commonly known as ATAPI devices.

This standard maintains a high degree of compatibility with the AT Attachment Interface with Packet Interface - 7 (ATA/ATAPI-7) volume 1, INCITS 397-2004, and while providing additional functions, is not intended to require changes to presently installed devices or existing software.

ANSI

INCITS.   -200x

| American National Standard | Approval of an American National Standard requires verification by ANSI that the requirements for due process, consensus, and other criteria for approval have been met by the standards developer. Consensus is established when, in the judgment of the ANSI Board of Standards Review, substantial agreement has been reached by directly and materially affected interests. Substantial agreement means much more than a simple majority, but not necessarily unanimity. Consensus requires that all views and objections be considered, and that effort be made towards their resolution. The use of American National Standards is completely voluntary; their existence does not in any respect preclude anyone, whether he has approved the standards or not, from manufacturing, marketing, purchasing, or using products, processes, or procedures not conforming to the standards. The American National Standards Institute does not develop standards and will in no circumstances give interpretation on any American National Standard. Moreover, no person shall have the right or authority to issue an interpretation of an American National Standard in the name of the American National Standards Institute. Requests for interpretations should be addressed to the secretariat or sponsor whose name appears on the title page of this standard. CAUTION NOTICE: This American National Standard may be revised or withdrawn at any time. The procedures of the American National Standards Institute require that action be taken periodically to reaffirm, revise, or withdraw this standard. Purchasers of American National Standards may receive current information on all standards by calling or writing the American National Standards Institute. |
| --- | --- |
|  | CAUTION: The developers of this standard have requested that holders of patents that may be required for the implementation of the standard, disclose such patents to the publisher. However, neither the developers nor the publisher have undertaken a patent search in order to identify which, if any, patents may apply to this standard. As of the date of publication of this standard, following calls for the identification of patents that may be required for the implementation of the standard, no such claims have been made. No further patent search is conducted by the developer or the publisher in respect to any standard it processes. No representation is made or implied that licenses are not required to avoid infringement in the use of this standard. |

Published by

American National Standards Institute

11 West 42nd Street, New York, New York  10036

Copyright © 2004 by Information Technology Industry Council (ITI)

All rights reserved.

No part of this publication may by reproduced in any

form, in an electronic retrieval system or otherwise,

without prior written permission of ITI, 1250 Eye Street NW, Suite 200,

Washington, DC 20005.

Printed in the United States of America

# Contents

Page

Contents	viii

Tables	xiii

Figures	xv

Foreword	xvi

Introduction	xvii

1	Scope	1

2	Normative references	3

2.1	Approved references	3

2.1.1	ANSI References	3

2.1.2	ISO References	3

2.2	References under development	3

2.3	Other references	4

3	Definitions, abbreviations, and conventions	5

3.1	Definitions and abbreviations	5

3.2	Conventions	8

3.2.1	Precedence	8

3.2.2	Lists	8

3.2.3	Keywords	8

3.2.4	Numbering	9

3.2.5	State diagram conventions	9

3.2.6	Byte ordering for data transfers	11

3.2.7	Byte, word and  DWORD Relationships	12

4	General operational requirements	13

4.1	Command delivery	13

4.2	Data transfer command sector addressing	13

4.2.1	Definitions and value ranges of IDENTIFY DEVICE data words	13

4.2.2	Addressing constraints and error reporting	13

4.3	General feature set	13

4.4	The PACKET feature set	14

4.4.1	Identification of PACKET Command feature set devices	15

4.4.2	PACKET Command feature set resets	15

4.4.3	The PACKET command	15

4.5	Power Management feature set	15

4.5.1	Power management commands	16

4.5.2	Standby timer	16

4.5.3	Power modes	16

4.6	Advanced Power Management feature set	19

4.7	Security Mode feature set	19

4.7.1	Security mode initial setting	20

4.7.2	User password lost	20

4.7.3	Attempt limit for SECURITY UNLOCK command	20

4.7.4	Security mode states	20

4.8	SMART (Self-monitoring, analysis, and reporting technology) feature set	26

4.8.1	Device SMART data structure	26

4.8.2	On-line data collection	26

4.8.3	Off-line data collection	26

4.8.4	Threshold exceeded condition	26

4.8.5	SMART feature set commands	26

4.8.6	SMART operation with power management modes	27

4.8.7	SMART device error log reporting	27

4.9	Host Protected Area feature set	27

4.9.1	BIOS determination of SET MAX security exension status	29

4.9.2	BIOS locking SET MAX	29

4.10	CompactFlash Association (CFA) feature set	31

4.11	Removable Media Status Notification and Removable Media feature sets	32

4.11.1	Removable Media Status Notification feature set	32

4.11.2	Removable Media feature set	33

4.12	Power-Up In Standby feature set	34

4.13	Automatic Acoustic Management (AAM) feature set	34

4.14	48-bit Address feature set	35

4.15	Device Configuration Overlay feature set	36

4.16	Media Card Pass Through Command feature set	39

4.17	Streaming feature set	40

4.17.1	Streaming commands	40

4.17.1.1	Urgent bit	40

4.17.1.2	Flush to Disk bit	41

4.17.1.3	Not Sequential bit	41

4.17.1.4	Read Continuous bit	41

4.17.1.5	Write Continuous bit	41

4.17.1.6	Handle Streaming Error bit	41

4.17.2	Streaming Logs	41

4.18	General Purpose Logging feature set	42

4.19	Overlapped feature set	42

4.20	Queued feature set	43

4.21	Long Physical Sector Feature Set  for Non-Packet Devices	44

4.22	Long Logical Sector Feature Set  for Non-Packet Devices	45

4.23	Devices Implementing the Long Physical Sector Feature Set and the Long Logical Feature Sector Set	46

4.24	Write-Read-Verify Feature Set	47

5	ATA Protocols	48

5.1	Overview	48

5.2	Power-On	48

5.3	Hardware Reset	48

5.4	Software Reset	48

5.5	Non-data Command Protocol	48

5.6	PIO data-in Command Protocol	48

5.7	PIO data-out Command Protocol	48

5.8	DMA Command Protocol	48

5.9	Packet Command Protocol	48

5.10	DMA Queued Command Protocol	48

5.11	Execute Device Diagnostic Command Protocol	48

5.12	Device Reset Command Protocol	48

5.13	Ultra DMA data-in Command Protocol	48

5.14	Ultra DMA Data-Out Command Protocol	48

6	Status and Error bits	48

6.1	Overview	48

6.2	Status Bits	48

6.3	Error Bits	49

6.4	Interrupt Reason Bits	50

7	Command Descriptions	50

7.1	Overview	50

7.2	CFA ERASE SECTORS - C0h, non-data	54

7.3	CFA REQUEST EXTENDED ERROR CODE - 03h, non-data	55

7.4	CFA TRANSLATE SECTOR - 87h, PIO data-in	56

7.5	CFA WRITE MULTIPLE WITHOUT ERASE - CDh, PIO data-out	57

7.6	CFA WRITE SECTORS WITHOUT ERASE - 38h, PIO data-out	58

7.7	CHECK MEDIA CARD TYPE - D1h, Non-data	59

7.8	CHECK POWER MODE - E5h, Non-data	60

7.9	CONFIGURE STREAM - 51h, Non-data	61

7.10	DEVICE CONFIGURATION	62

7.10.1	DEVICE CONFIGURATION RESTORE - B1h/C0h, Non-data	63

7.10.2	DEVICE CONFIGURATION FREEZE LOCK - B1h/C1h, Non-data	64

7.10.3	DEVICE CONFIGURATION IDENTIFY - B1h/C2h, PIO Data-in	65

7.10.4	DEVICE CONFIGURATION SET - B1h/C3h, PIO Data Out	68

7.11	DEVICE RESET - 08h, Device reset	73

7.12	DOWNLOAD MICROCODE - 92h, PIO Data-out	74

7.13	EXECUTE DEVICE DIAGNOSTIC - 90h, Device diagnostic	75

7.14	FLUSH CACHE - E7h, Non-data	76

7.15	FLUSH CACHE EXT - EAh, Non-data	77

7.16	GET MEDIA STATUS - DAh, Non-data	78

7.17	IDENTIFY DEVICE - ECh, PIO Data-in	79

7.18	IDENTIFY PACKET DEVICE - A1h, PIO Data-in	102

7.19	IDLE - E3h, Non-data	115

7.20	IDLE IMMEDIATE - E1h, Non-data	116

7.21	MEDIA EJECT - EDh, Non-data	118

7.22	MEDIA LOCK - DEh, Non-data	119

7.23	MEDIA UNLOCK - DFh, Non-data	120

7.24	NOP - 00h, Non-data	121

7.25	PACKET - A0h, Packet	122

7.26	READ BUFFER - E4, PIO data-in	126

7.27	READ DMA - C8h, DMA	127

7.28	READ DMA EXT - 25h, DMA	128

7.29	READ DMA QUEUED - C7h, DMA Queued	129

7.30	READ DMA QUEUED EXT- 26h, DMA Queued	131

7.31	READ LOG EXT - 2Fh, PIO data-in	133

7.32	READ MULTIPLE - C4h, PIO data-in	145

7.33	READ MULTIPLE EXT - 29h, PIO data-in	146

7.34	READ NATIVE MAX ADDRESS - F8h, Non-data	147

7.35	READ NATIVE MAX ADDRESS EXT - 27h, Non-data	148

7.36	READ SECTOR(S) - 20h, PIO data-in	149

7.37	READ SECTOR(S) EXT - 24h, PIO data-in	150

7.38	READ STREAM DMA EXT - 2Ah, DMA	151

7.39	READ STREAM EXT - 2Bh, PIO data-in	154

7.40	READ VERIFY SECTOR(S) - 40h, Non-data	156

7.41	READ VERIFY SECTOR(S) EXT - 42h, Non-data	157

7.42	SECURITY DISABLE PASSWORD - F6h, PIO data-out	158

7.43	SECURITY ERASE PREPARE - F3h, Non-data	159

7.44	SECURITY ERASE UNIT - F4h, PIO data-out	160

7.45	SECURITY FREEZE LOCK - F5h, Non-data	161

7.46	SECURITY SET PASSWORD - F1h, PIO data-out	162

7.47	SECURITY UNLOCK - F2h, PIO data-out	163

7.48	SERVICE - A2h, Packet or DMA queued	164

7.49	SET FEATURES - EFh, Non-data	165

7.49.2	Description	165

7.49.3	Enable/disable 8-bit PIO data transfer	167

7.49.4	Enable/disable write cache	167

7.49.5	Set transfer mode	167

7.49.6	Enable/disable advanced power management	168

7.49.7	Enable/disable Power-Up In Standby feature set	168

7.49.8	Power-Up In Standby feature set device spin-up	168

7.49.9	Enable/disable CFA power mode 1	169

7.49.10	Enable/Disable Write Read Verify feature set	169

7.49.11	Enable/disable Media Status Notification	169

7.49.12	Enable/disable Automatic Acoustic Management	170

7.49.13	Set Maximum Host Interface Sector Times	171

7.49.14	Enable/disable read look-ahead	171

7.49.15	Enable/disable release interrupt	171

7.49.16	Enable/disable SERVICE interrupt	171

7.49.17	Enable/disable reverting to defaults	171

7.50	SET MAX	172

7.50.1	SET MAX Overview	172

7.50.2	SET MAX ADDRESS - F9h	173

7.50.3	SET MAX SET PASSWORD - F9h/01h, PIO data-out	174

7.50.5	SET MAX LOCK - F9h/02h, Non-data	175

7.50.6	SET MAX UNLOCK - F9h/03h, PIO data-out	176

7.51	SET MAX ADDRESS EXT - 37h, Non-data	178

7.52	SET MULTIPLE MODE - C6h, Non-data	179

7.53	SLEEP - E6h, Non-data	180

7.54	SMART	181

7.54.1	Overview	181

7.54.2	SMART DISABLE OPERATIONS - B0h/D9h, Non-data	182

7.54.3	SMART ENABLE/DISABLE ATTRIBUTE AUTOSAVE - B0h/D2h, Non-data	183

7.54.4	SMART ENABLE OPERATIONS - B0h/D8h, Non-data	184

7.54.5	SMART EXECUTE OFF-LINE IMMEDIATE - B0h/D4h, Non-data	185

7.54.6	SMART READ DATA - B0h/D0h, PIO data-in	189

7.54.7	SMART READ LOG - B0h/D5h	193

7.54.8	SMART RETURN STATUS - B0h/DAh, Non-data	201

7.54.9	SMART WRITE LOG - D6h, PIO data-out	202

7.55	STANDBY - E2h, Non-data	203

7.56	STANDBY IMMEDIATE - E0h, Non-data	204

7.57	TRUSTED RECEIVE – 5Ch	205

7.58	TRUSTED RECEIVE DMA – 5Dh	205

7.59	TRUSTED SEND – 5Eh	205

7.60	TRUSTED SEND DMA – 5Fh	205

7.61	WRITE BUFFER - E8h, PIO data-out	206

7.62	WRITE DMA - CAh, DMA	207

7.63	WRITE DMA EXT - 35h, DMA	208

7.64	WRITE DMA FUA EXT - 3Dh, DMA	209

7.65	WRITE DMA QUEUED - CCh, DMA queued	210

7.66	WRITE DMA QUEUED EXT - 36h, DMA queued	212

7.67	WRITE DMA QUEUED FUA EXT - 3Eh, DMA queued	214

7.68	WRITE LOG EXT - 3Fh, PIO data-out	215

7.69	WRITE MULTIPLE - C3h, PIO data-out	216

7.70	WRITE MULTIPLE EXT - 39h, PIO data-out	217

7.71	WRITE MULTIPLE FUA EXT - CEh, PIO data-out	218

7.72	WRITE SECTOR(S) - 30h, PIO data-out	219

7.73	WRITE SECTOR(S) EXT - 34h, PIO data-out	220

7.74	WRITE STREAM DMA EXT - 3Ah, DMA	221

7.75	WRITE STREAM EXT - 3Bh, PIO data-out	223

8	Normal & Error Outputs	224

8.1	Overview	224

8.2	Normal Outputs	224

8.3	Error Outputs	231

Annex A.	Bibliography (Informative)	243

Annex B.	Command Set summary (Informative)	243

Annex C.	Design and programming considerations for large physical sector devices (informative)	258

C.1	Introduction	258

C.2	Physical sectors	258

C.3	Unaligned write	258

C.4	SET MAX	259

C.5	Software compatibility	259

# Tables

Page

Table 1 - PACKET delivered command sets	2

Table 2 - Byte order	11

Table 3 - Byte order	11

Table 4 - Security mode command actions	24

Table 5 - 48-bit addresses	35

Table 6 - 28-bit addresses	36

Table 7 - Media Card type references	40

Table 8 - Long Logical Sector Function	45

Table 9 - Extended error codes	55

Table 10 - CFA TRANSLATE SECTOR Information	56

Table 11 - Device Configuration Overlay Features register values	62

Table 12 - Device Configuration Identify data structure	66

Table 13 - Device Configuration Overlay data structure	70

Table 14 - Diagnostic codes	75

Table 15 - IDENTIFY DEVICE data	81

Table 16 - Minor version number	92

Table 17 - IDENTIFY PACKET DEVICE data	103

Table 18 - Automatic Standby timer periods	115

Table 19 - NOP Subcommand Code	121

Table 20 - General Purpose Log Directory	133

Table 21 - Extended Comprehensive SMART error log	133

Table 22 - Extended Error log data structure	134

Table 23 - Command data structure	135

Table 24 - Error data structure	135

Table 25 - State field values	136

Table 26 - Extended Self-test log data structure	137

Table 27 - Extended Self-test log descriptor entry	137

Table 28 - Read Stream Error Log	138

Table 29 - Error Log Entry	139

Table 30 - Write Stream Error Log	139

Table 31 - Streaming Performance Parameters Log	140

Table 32 - Sector Time Array Entry (Linearly Interpolated)	140

Table 33 - Position Array Entry (Linearly Interpolated)	140

Table 34 - Access Time Array Entry (Linearly Interpolated)	140

Table 35 -  Delayed LBA log	141

Table 36 - Log address definition	143

Table 37 - Security password content	158

Table 38 - SECURITY ERASE UNIT password	160

Table 39 - SECURITY SET PASSWORD data content	162

Table 40 - Identifier and security level bit interaction	162

Table 41 - SET FEATURES register definitions	166

Table 42 - Transfer mode values	167

Table 43 - Advanced power management levels	168

Table 44 – Write-Read-Verify Sector Counts	169

Table 44 - Automatic acoustic management levels	170

Table 45 - SET MAX Features register values	172

Table 46 - SET MAX SET PASSWORD data content	174

Table 47 - SMART Feature register values	181

Table 48 - SMART EXECUTE OFF-LINE IMMEDIATE Subcommands	185

Table 49 - Device SMART data structure	189

Table 50 - Off-line data collection status byte values	190

Table 51 - Self-test execution status values	190

Table 52 - SMART Log Directory	193

Table 53 - SMART summary error log sector	194

Table 54 - Error log data structure	194

Table 55 - Command data structure	195

Table 56 - Error data structure	195

Table 57 - State field values	195

Table 58 - Comprehensive error log	196

Table 59 - Self-test log data structure	197

Table 60 - Self-test log descriptor entry	197

Table 61 - Selective self-test log	198

Table 62 - Selective self-test feature flags	199

Table 63 – Extended Error Code	224

Table 64 – Device Fault w/Error Bit Defined Normal Output	224

Table 65 – Normal Output	225

Table 66 – Media Card Type Normal Output	225

Table 67 – Check Power Mode	226

Table 68 – Stream Normal	226

Table 69 – Device Reset and Execute Diagnostics	227

Table 70 – IDLE Unload	227

Table 71 – ATAPI Normal Output	228

Table 72 – Queued Normal Output	228

Table 73 – SETMAX Normal Output	229

Table 74 – SMART Off-Line Immediate Normal Output	229

Table 75 – SMART Return Status Normal Output	230

Table 76 –Error Status	231

Table 77 –Abort Only Error Status	231

Table 78 –Abort w/bit 3 Error Status	232

Table 79 – Stream Error	232

Table 80 – Write Error	233

Table 81 –Media Status	233

Table 82 –Media Eject	234

Table 83 –Media Lock	234

Table 84 – Read DMA Error	235

Table 85 –Read Log Ext Error	236

Table 86 –Read PIO Error	237

Table 87 –Read Stream Error	238

Table 88 –Setmax Error	238

Table 89 –SMART Error	239

Table 90 –Writre Error	240

Table 91 –Writre Stream DMA Error	241

Table 92 –Writre Stream Error	242

Table 93 - Command Matrix	243

Table 94 - Command codes (sorted by command code)	244

Table 95- Command codes (sorted by command)	247

Table 96 - Historical Command Assignments	249

Table 97 - Historical SET FEATURE Code Assignments	256

# Figures

Page

Figure 1 - ATA document relationships	1

Figure 2 - State diagram convention	9

Figure 3 - Byte, word and  DWORD relationships	12

Figure 4 - Power management state diagram	17

Figure 5 - Security mode state diagram	21

Figure 6 - SET MAX security state diagram	30

Figure 7 - Device Configuration Overlay state diagram	38

Figure 8 - Long Logical and long Physical Sector Example	44

Figure 9 - Selective self-test test span example	187

Figure 10 - Unaligned Write Example	259

# Foreword

(This foreword is not part of this standard.)

Requests for interpretation, suggestions for improvement and addenda, or defect reports are welcome.

They should be sent to the INCITS Secretariat, ITI, 1250 Eye Street, NW, Suite 200, Washington, DC

20005-3922.

This standard was processed and approved for submittal to ANSI by InterNational Committee for

Information Technology Standards (INCITS).  Committee approval of this standard does not necessarily

imply that all committee members voted for approval.  At the time it approved this standard, INCITS had the

following members:

Karen Higginbottom, Chair

David Michael, Vice-chair

Monica Vago, Secretary

Technical Committee T13 on ATA Interfaces, that reviewed this standard, had the following members and additional participants:

Dan Colgrove, Chairman

Jim Hatfield, Vice-Chairman

Mark Overby,  Secretary

[Editors Note: Insert T13 Membership List Here]

# Introduction

This standard encompasses the following:

Clause 1 describes the scope.

Clause 2 provides normative references for the entire standard.

Clause 3 provides definitions, abbreviations, and conventions used within the entire standard.

Clause 4 describes the general operating requirements of the command layer.

Clause 5 describes the ATA protocols used by the commands in this standard

Clause 6 describes status and error bits

Clause 7 describes commands

AMERICAN NATIONAL STANDARD                                                         INCITS ***-nnnn

American National Standard

for Information Systems 

Information Technology 

AT Attachment - 8 ATA/ATAPI Command Set (ATA8-ACS)

## Scope

[Editors Note: This section needs to be updated.  Need to indicate that this is an independent standard and its relationship to other standards.]

This standard specifies the AT Attachment Interface between host systems and storage devices.  It provides a common attachment interface for systems manufacturers, system integrators, software suppliers, and suppliers of intelligent storage devices.

Volume 1 defines the register delivered commands used by devices implementing the standard.  Volume 2 defines the connectors and cables for physical interconnection between host and storage device, the electrical and logical characteristics of the interconnecting signals, and the protocols for the transporting commands, data, and status over the interface for the parallel interface.  Volume 3 defines the connectors and cables for physical interconnection between host and storage device, the electrical and logical characteristics of the interconnecting signals, and the protocols for the transporting commands, data, and status over the interface for the serial interface.  Figure 1 shows the relationship of these documents.  For devices implementing the PACKET command feature set, additional command layer standards are listed in Table 1 and described in Clause 2.

Figure 1 - ATA document relationships 1

Table 1 - PACKET delivered command sets 1

Standard

SCSI Primary Commands (SPC)

SCSI Primary Commands - 2 (SPC-2)

SCSI Primary Commands - 3 (SPC-3)

SCSI Block Commands (SBC-2)

SCSI Stream Commands (SSC)

Multimedia Commands (MMC) [Editors Note: Do we really need to list all these?]

Multimedia Commands - 2 (MMC-2)

Multimedia Commands - 3 (MMC-3)

Multimedia Commands - 4 (MMC-4)

ATAPI for Removable Media (SFF8070I)

ATA Packet Interface (ATAPI) for Streaming Tape QIC-157 revision D

This standard maintains compatibility with the AT Attachment with Packet Interface - 7 standard (ATA/ATAPI-7), INCITS 361-2002 [Editors Note: Need number], and while providing additional functions, is not intended to require changes to presently installed devices or existing software.

## Normative references

The following standards contain provisions that, through reference in the text, constitute provisions of this standard.  At the time of publication, the editions indicated were valid.  All standards are subject to revision, and parties to agreements based on this standard are encouraged to investigate the possibility of applying the most recent editions of the standards listed below.

Copies of the following documents can be obtained from ANSI:  Approved ANSI standards, approved and draft international and regional standards (ISO, IEC, CEN/CENELEC, ITUT), and approved and draft foreign standards (including BSI, JIS, and DIN).  For further information, contact ANSI Customer Service Department at 212-642-4900 (phone), 212-302-1286 (fax), or via the World Wide Web at http://www.ansi.org.

Additional availability contact information is provided below as needed.

### Approved references

The following approved ANSI standards, approved international and regional standards (ISO, IEC, CEN/CENELEC, ITUT), may be obtained from the international and regional organizations who control them.

#### ANSI References

SCSI-3 Block Commands (SBC)		[ANSI INCITS 306-1998]

SCSI-3 Primary Commands (SPC)	[ANSI X3.301-1997]

SCSI-3 Streaming Commands (SSC)	[ANSI INCITS 335-2000]

Multimedia Commands (MMC)		[ANSI X3.304-1997]

Multimedia Commands - 2 (MMC-2)	[ANSI INCITS 333-2000]

Multimedia Commands - 3 (MMC-3)	[ANSI INCITS 360-2002]

Protected Area Run Time Interface Extensions (PARTIES)	[ANSI INCITS 346-2001]

SCSI Primary Commands - 2 (SPC-2)	[ANSI INCITS 351-2001]

AT Attachment with Packet Interface Extension (ATA/ATAPI-4), [ANSI INCITS.317-1998]

#### ISO References

Control and Status Register (CSR) Architecture for microprocessor buses	[ISO/IEC 13213:1994]

To obtain copies of these documents, contact Global Engineering or INCITS.  Additional information may be available at http://www.t10.org and http://www.t13.org.

### References under development

At the time of publication, the following referenced standards were still under development.  For information on the current status of the document, or regarding availability, contact the relevant standards body or other organization as indicated.

SCSI Block Commands - 2 (SBC-2)	[T10/1417-D]

SCSI Primary Commands - 3 (SPC-3)	[T10/1416-D]

ATAPI for Rewritable Media 		[SFF8070i]

Multimedia Commands - 4 (MMC-4)	[T10/1545D]

For more information on the current status of the T10 documents, contact INCITS.  To obtain copies of T10 or SFF documents, contact Global Engineering.

### Other references

The following standards and specifications are also referenced.

PC Card Standard, February 1995, PCMCIA (68-pin Connector)

For the PC Card Standard published by the Personal Computer Memory Card International Association, contact PCMCIA at 408-433-2273 or http://www.pc-card.org.

CompactFlash Association Specification, Revision 1.4

For the CompactFlash Association Specification published by the CompactFlash Association, contact the CompactFlash Association at http://www.compactflash.org.

ATA Packet Interface (ATAPI) for Streaming Tape QIC-157 revision D

For QIC specifications published by Quarter-Inch Cartridge Drive Standards, Inc., contact them at 805 963-3853 or http://www.qic.org.

## Definitions, abbreviations, and conventions

### Definitions and abbreviations

For the purposes of this standard, the following definitions apply:

|  | ASCII Character: Designates 8-bit value that is encoded using the ASCII Character set. |
| --- | --- |
|  | acoustics: Measurement of airborne noise emitted by information technology and telecommunications equipment [ISO 7779:1999(E)] |
|  | ATA (AT Attachment): ATA defines the physical, electrical, transport, and command protocols for the internal attachment of storage devices to host systems. |
|  | ATA-1 device: A device that complied with ANSI X3.221-1994, the AT Attachment Interface for Disk Drives. ANSI X3.221-1994 has been withdrawn. |
|  | ATA-2 device: A device that complied with ANSI X3.279-1996, the AT Attachment Interface with Extensions. ANSI X3.279-1996 has been withdrawn. |
|  | ATA-3 device: A device that complies with ANSI X3.298-1997, the AT Attachment-3 Interface. ANSI X3.298-1997 has been withdrawn. |
|  | ATA/ATAPI-4 device: A device that complies with ANSI INCITS 317-1998, AT Attachment Interface with Packet Interface Extensions-4. |
|  | ATA/ATAPI-5 device: A device that complies with ANSI INCITS 340-2000, the AT Attachment with Packet Interface-5. |
|  | ATA/ATAPI-6 device: A device that complies with ANSI INCITS 361-2002, the AT Attachment with Packet Interface-6. |
|  | ATA/ATAPI-7 device: A device that complies with this standard. |
|  | ATAPI (AT Attachment Packet Interface) device: A device implementing the Packet Command feature set. |
|  | AU (Allocation Unit): The minimum number of logically contiguous sectors on the media as used in the Streaming feature set. An Allocation Unit may be accessed with one or more requests. |
|  | AV (Audio-Video): Audio-Video applications use data that is related to video images and/or audio. The distinguishing characteristic of this type of data is that accuracy is of lower priority than timely transfer of the data. |
|  | bus release: This is the state that allows the host to select the other device or deliver another queued command (e.g. For devices implementing overlap, the term bus release is the act of clearing both DRQ and BSY to zero before the action requested by the command is completed). [Editors Note: Need to rework this] |
|  | byte count: The value placed in the Byte Count register by the device to indicate the number of bytes to be transferred during this DRQ assertion when executing a PACKET PIO data transfer command. [Editors Note: Need to rework this] |
|  | byte count limit: The value placed in the Byte Count register by the host as input to a PACKET PIO data transfer command to specify the maximum byte count that may be transferred during a single DRQ assertion. [Editors Note: Need to rework this] |
|  | CFA (CompactFlash Association): The CompactFlash Association which created the specification for compact flash memory that uses the ATA interface. |
|  | check condition: For devices implementing the PACKET Command feature set, this indicates an error or exception condition has occurred. |
|  | CHS (cylinder-head-sector): An obsolete method of addressing the data on the device by cylinder number, head number, and sector number. |
|  | command aborted: Command completion with ERR set to one in the Status field and ABRT set to one in the Error field. |
|  | command acceptance: Positive acknoweledgement of a command being received by a device. See the appropriate transport standard for a definition of positive acknoweledgement. |
|  | Command Block: In a parallel implementation this is the interface registers used for delivering commands to the device or posting status from the device. In a serial implementation, the command block registers are FIS payload fields. |
|  | command completion: Command completion is the completion by the device of the action requested by the command or the termination of the command with an error, the placing of the appropriate error bits in the Error field, the placing of the appropriate status bits in the Status field. |
|  | command packet: A data structure transmitted to the device during the execution of a PACKET command that includes the command and command parameters. |
|  | command released: When a device supports overlap or queuing, a command is considered released when a bus release occurs before command completion. |
|  | Control Block: In a parallel implementation, interface registers used for device control and to post alternate status. In a serial interface implementation, the logical field of a FIS corresponding to the Device Register bits of a parallel implementation. |
|  | CRC (Cyclical Redundancy Check): A means used to check the validity of certain data transfers. |
|  | Cylinder High field: The name used for the LBA High register in previous ATA/ATAPI standards. |
|  | Cylinder Low field: The name used for the LBA Mid register in previous ATA/ATAPI standards. |
|  | Delayed LBA: Any sector for which the performance specified by the Streaming Performance Parameters log is not valid. |
|  | device: A storage peripheral. Traditionally, a device on the interface has been a hard disk drive, but any form of storage device may be placed on the interface provided the device adheres to this standard. |
|  | DMA (direct memory access) data transfer: A means of data transfer between device and host memory without host processor intervention. |
|  | don’t care: A term to indicate that a value is irrelevant for the particular function described. |
|  | DRQ data block: A unit of data words transferred during a single assertion of DRQ when using PIO data transfer. [Editors Note: All references to DRQ blocks should be moved to the parallel transport and deleted from here.] |
|  | FUA (Forced Unit Access): Forced Unit Access requires that user data be transferred to or from the device media before command completion even if caching is enabled. |
|  | Gen1 DWORD Time: The time it takes to transmit a 40 bit encoded value at 1.5 Gb/Sec. |
|  | host: The computer system executing the software BIOS and/or operating system device driver controlling the device and the adapter hardware for the ATA interface to the device. |
|  | host adapter: The implementation of the host transport, link, and physical layers. |
|  | LBA (logical block address): The addressing of data on the device by the linear mapping of sectors. |
|  | logical sector: A uniquely addressable set of 256 words (512 bytes). |
|  | native max address: The highest address a device accepts in the factory default condition, that is, the highest address that is accepted by the SET MAX ADDRESS command. |
|  | overlap: A protocol that allows devices that require extended command time to perform a bus release so that commands may be executed by the other device (if present) on the bus. |
|  | packet delivered command: A command that is delivered to the device using the PACKET command via a command packet that contains the command and the command parameters. See also register delivered command. |
|  | physical sector: A group of contiguous logical sectors that are read from or written to the device media in a single operation. |
|  | PIO (programmed input/output) data transfer: PIO data transfers are performed using PIO commands and protocol. |
|  | queued: Command queuing allows the host to issue concurrent commands to the same device. Only commands included in the Overlapped feature set may be queued. In this standard, the queue contains all commands for which command acceptance has occurred but command completion has not occurred. |
|  | read command: A command that causes the device to transfer data from the device to the host (e.g., READ SECTOR(S), READ DMA, etc.). |
|  | sector: A uniquely addressable set of 256 words (512 bytes). |
|  | Sector Number field:The LBA Low register in previous ATA/ATAPI standards. |
|  | signature: A unique set of values placed in the Command Block registers by the device to allow the host to distinguish devices implementing the PACKET Command feature set from those devices not implementing the PACKET Command feature set. |
|  | SMART (Self-Monitoring, Analysis, and Reporting Technology): for prediction of device degradation and/or faults. |
|  | transport: The mechanism used to communicate with a device. See ATA8-APT and ATA8-AST. |
|  | unaligned write: A write command that does not start at the first logical sector of a physical sector or does not end at the last logical sector of a physical sector. |
|  | unit attention condition: A state that a device implementing the PACKET Command feature set maintains while the device has asynchronous status information to report to the host. |
|  | unrecoverable error: When the device sets either the ERR bit or the DF bit to one in the Status field at command completion. |
|  | VS (vendor specific): Bits, bytes, fields, and code values that are reserved for vendor specific purposes. These bits, bytes, fields, and code values are not described in this standard, and may vary among vendors. This term is also applied to levels of functionality whose definition is left to the vendor. NOTE  Industry practice could result in conversion of a Vendor Specific bit, byte, field, or code value into a defined standard value in a future standard. |
|  | write command: A command that causes the device to transfer data from the host to the device (e.g., WRITE SECTOR(S), WRITE DMA, etc.). |
|  | WWN (world wide name): A 64-bit worldwide unique name based upon a company’s IEEE identifier. (See IDENTIFY DEVICE Words (108:111) in Volume 1 Clause 6). |

### Conventions

Lowercase is used for words having the normal English meaning.  Certain words and terms used in this standard have a specific meaning beyond the normal English meaning.  These words and terms are defined either in Clause 3 or in the text where they first appear.

The names of abbreviations, commands, fields, and acronyms used as signal names are in all uppercase (e.g., IDENTIFY DEVICE).  Fields containing only one bit are usually referred to as the "name" bit instead of the "name" field.  (See [Editors Note: Fix this reference] for the naming convention used for naming bits.)

Names of device registers begin with a capital letter (e.g., LBA Mid register).

The expression “word n” or “bit n” shall be interpreted as indicating the content of word n or bit n.

#### Precedence

If there is a conflict between text, figures, and tables, the precedence shall be tables, figures, then text.

#### Lists

Unordered lists, those lists describing a sequence, are of the form:

a)

b)

c)

Ordered list are of the form:

1)

2)

3)

#### Keywords

Several keywords are used to differentiate between different levels of requirements and optionality.

3.2.3.1 expected: A keyword used to describe the behavior of the hardware or software in the design models assumed by this standard.  Other hardware and software design models may also be implemented.

3.2.3.2 mandatory: A keyword indicating items to be implemented as defined by this standard.

3.2.3.3 may: A keyword that indicates flexibility of choice with no implied preference.

3.2.3.4 obsolete: A keyword indicating that the designated bits, bytes, words, fields, and code values that may have been defined in previous standards are not defined in this standard and shall not be reclaimed for other uses in future standards.  However, some degree of functionality may be required for items designated as “obsolete” to provide for backward compatibility.

Obsolete commands should not be used by the host.  Commands defined as obsolete may be command aborted by devices conforming to this standard.  However, if a device does not command abort an obsolete command, the minimum that is required by the device in response to the command is command completion.

3.2.3.5 optional: A keyword that describes features that are not required by this standard.  However, if any optional feature defined by the standard is implemented, the feature shall be implemented in the way defined by the standard.

3.2.3.6 prohibited: A keyword indicating that an item shall not be implemented by an implementation.

3.2.3.7 reserved: A keyword indicating reserved bits, bytes, words, fields, and code values that are set aside for future standardization.  Their use and interpretation may be specified by future extensions to this or other standards.  A reserved bit, byte, word, or field shall be cleared to zero, or in accordance with a future extension to this standard.  The recipient shall not check reserved bits, bytes, words, or fields.  Receipt of reserved code values in defined fields shall be treated as a command parameter error and reported by returning command aborted.

3.2.3.8 retired: A keyword indicating that the designated bits, bytes, words, fields, and code values that had been defined in previous standards are not defined in this standard and may be reclaimed for other uses in future standards.  If retired bits, bytes, words, fields, or code values are used before they are reclaimed, they shall have the meaning or functionality as described in previous standards.

3.2.3.9 shall: A keyword indicating a mandatory requirement.  Designers are required to implement all such mandatory requirements to ensure interoperability with other products that conform to this standard.

3.2.3.10 should: A keyword indicating flexibility of choice with a strongly preferred alternative.  Equivalent to the phrase “it is recommended”.

#### Numbering

Numbers that are not immediately followed by a lowercase "b" or "h" are decimal values.  Numbers that are immediately followed by a lowercase "b" (e.g., 01b) are binary values.  Numbers that are immediately followed by a lowercase "h" (e.g., 3Ah) are hexadecimal values.

#### State diagram conventions

State diagrams shall be as shown in Figure 2.

Figure 2 - State diagram convention2

Each state is identified by a state designator and a state name.  The state designator is unique among all states in all state diagrams in this document.  The state designator consists of a set of letters that are capitalized in the title of the figure containing the state diagram followed by a unique number.  The state name is a brief description of the primary action taken during the state, and the same state name may appear in other state diagrams.  If the same primary function occurs in other states in the same state diagram, they are designated with a unique letter at the end of the name.  Additional actions may be taken while in a state and these actions are described in the state description text.

In device command protocol state diagrams, the state of bits and signals that change state during the execution of this state diagram are shown under the state designator:state_name, and a table is included that shows the state of all bits and signals throughout the state diagram as follows:

v = bit value changes.

1 = bit set to one.

0 = bit cleared to zero.

x = bit is don’t care.

V = signal changes.

A = signal is asserted.

N = signal is negated.

R = signal is released.

X = signal is don’t care.

Each transition is identified by a transition label and a transition condition.  The transition label consists of the state designator of the state from which the transition is being made followed by the state designator of the state to which the transition is being made.  In some cases, the transition to enter or exit a state diagram may come from or go to a number of state diagrams, depending on the command being executed.  In this case, the state designator is labeled xx.  The transition condition is a brief description of the event or condition that causes the transition to occur and may include a transition action, indicated in italics, that is taken when the transition occurs.  This action is described fully in the transition description text.

Upon entry to a state, all actions to be executed in that state are executed.  If a state is re-entered from itself, all actions to be executed in the state are executed again.

Transitions from state to state shall be instantaneous.

#### Byte ordering for data transfers

Data is transferred in blocks using either PIO or DMA protocols.  PIO data transfers occur when the BSY bit is cleared to zero and the DRQ bit is set to one.  These transfers are usually 16-bit but CFA devices may implement 8-bit PIO transfers.  Data is transferred in blocks of one or more bytes known as a DRQ block.  DMA data transfers occur when the host asserts DMACK- in response to the device asserting DMARQ.  DMA transfers are always 16-bit.  Each assertion of DMACK- by the host defines a DMA data burst.  A DMA data burst is two or more bytes.

Assuming a DRQ block or a DMA burst of data contains "n" bytes of information, the bytes are labeled Byte(0) through Byte(n-1), where Byte(0) is first byte of the block, and Byte(n-1) is the last byte of the block.  Table 2 shows the order the bytes shall be presented in when such a block of data is transferred on the interface using 16-bit PIO and DMA transfers.  Table 3 shows the order the bytes shall be presented in when such a block or burst of data is transferred on the interface using 8-bit PIO.

Table 2 - Byte order 2

|  | DD 15 | DD 14 | DD 13 | DD 12 | DD 11 | DD 10 | DD 9 | DD 8 | DD 7 | DD 6 | DD 5 | DD 4 | DD 3 | DD 2 | DD 1 | DD0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First transfer | Byte (1) |  |  |  |  |  |  |  | Byte (0) |  |  |  |  |  |  |  |
| Second transfer | Byte (3) |  |  |  |  |  |  |  | Byte (2) |  |  |  |  |  |  |  |
| ........ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Last transfer | Byte (n-1) |  |  |  |  |  |  |  | Byte (n-2) |  |  |  |  |  |  |  |

Table 3 - Byte order 3

|  | DD 7 | DD 6 | DD 5 | DD 4 | DD 3 | DD 2 | DD 1 | DD0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First transfer | Byte (0) |  |  |  |  |  |  |  |
| Second transfer | Byte (1) |  |  |  |  |  |  |  |
| ........ |  |  |  |  |  |  |  |  |
| Last transfer | Byte (n-1) |  |  |  |  |  |  |  |

NOTE  The above description is for data on the interface.  Host systems and/or host adapters may cause the order of data as seen in the memory of the host to be different.

Some parameters are defined as a string of ASCII characters.  ASCII data fields shall contain only code values 20h through 7Eh.  For the string “Copyright”, the character “C” is the first byte, the character “o” is the second byte, etc.  When such fields are transferred, the order of transmission is:

the 1st character (“C”) is on DD(15:8) of the first word,

the 2nd character (“o”) is on DD(7:0) of the first word,

the 3rd character (“p”) is on DD(15:8) of the second word,

the 4th character (“y”) is on DD(7:0) of the second word,

the 5th character (“r”) is on DD(15:8) of the third word,

the 6th character (“i”) is on DD(7:0) of the third word,

the 7th character (“g”) is on DD(15:8) of the fourth word,

the 8th character (“h”) is on DD(7:0) of the fourth word,

the 9th character (“t”) is on DD(15:8) of the fifth word,

the 10th character (“space”) is on DD(7:0) of the fifth word,

etc.

Word (n:m) denotes a set of words, for example, words (103:100).

#### Byte, word and  DWORD Relationships

Figure 3 illustrates the relationship between bytes, words and  DWORDs for serial interface implementations.

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Byte |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Word |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Byte 1 |  |  |  |  |  |  |  | Byte 0 |  |  |  |  |  |  |  |  |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DWORD |
| Word 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Word 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Byte 3 |  |  |  |  |  |  |  | Byte 2 |  |  |  |  |  |  |  | Byte 1 |  |  |  |  |  |  |  | Byte 0 |  |  |  |  |  |  |  |  |

Figure 3 - Byte, word and  DWORD relationships 3

## General operational requirements

### Command delivery

Commands may be delivered in two forms.  For devices that do not implement the PACKET Command feature set, all commands and command parameters are delivered by writing the device Command Block registers.  Such commands are defined as register delivered commands.

Devices that implement the PACKET Command feature set use packet delivered commands as well as some register delivered commands.

All register delivered commands and the PACKET command are described in Clause 5.

NOTE  The content of command packets delivered during execution of the PACKET command are not described in this standard.  See Clause 1 for standards and specifications that define command packet content.

### Data transfer command sector addressing

For data transfer commands all addressing of data sectors recorded on the device's media is by a logical sector address.  There is no implied relationship between logical sector addresses and the actual physical location of the data sector on the media.  All devices shall support LBA translation.

In standards ATA/ATAPI-5 and earlier, a CHS translation was defined.  This translation is obsolete but if implemented it shall be implemented as defined in ATA/ATAPI-5.

#### Definitions and value ranges of IDENTIFY DEVICE data words

(Also See Clause 7.17)

- Words (61:60) shall contain the value one greater than the total number of user-addressable sectors in 28-bit addressing and shall not exceed 0FFFFFFFh.  The content of words (61:60) shall be greater than or equal to one and less than or equal to 268,435,455.

- Words (103:100) shall contain the value one greater than the total number of user-addressable sectors in 48-bit addressing and shall not exceed 0000FFFFFFFFFFFFh.

- The contents of words (61:60) and (103:100) may be affected by the host issuing a SET MAX ADDRESS or SET MAX ADDRESS EXT command.

- The contents of words (61:60) and (103:100) shall not be used to determine if 48-bit addressing is supported.  IDENTIFY DEVICE bit 10 word 83 indicates support for 48-bit addressing.

#### Addressing constraints and error reporting

Devices shall set IDNF to one or ABRT to one in the Error register and ERR to one in the Status register in response to any command where the requested LBA number is greater than or equal to the content of words (61:60) for a 28-bit addressing command or greater or equal to the contents of words (103:100) for a 48-bit addressing command.

### General feature set

The following General feature set commands are mandatory for all devices that are capable of both reading and writing their media and do not implement the PACKET feature set:

- EXECUTE DEVICE DIAGNOSTIC

- FLUSH CACHE

- IDENTIFY DEVICE

- READ DMA

- READ MULTIPLE

- READ SECTOR(S)

- READ VERIFY SECTOR(S)

- SET FEATURES

- SET MULTIPLE MODE

- WRITE DMA

- WRITE MULTIPLE

- WRITE SECTOR(S)

The following General feature set commands are mandatory for all devices that are capable of only reading their media and do not implement the PACKET command feature set:

- EXECUTE DEVICE DIAGNOSTIC

- IDENTIFY DEVICE

- READ DMA

- READ MULTIPLE

- READ SECTOR(S)

- READ VERIFY SECTOR(S)

- SET FEATURES

- SET MULTIPLE MODE

The following General feature set commands are optional for devices not implementing the PACKET command feature set:

- DOWNLOAD MICROCODE

- NOP

- READ BUFFER

- WRITE BUFFER

The following Packet feature set command is prohibited for use by devices not implementing the PACKET command feature set:

- DEVICE RESET

[Editors Note: Reset requirement need to be in each transport]

### The PACKET feature set

The following commands are mandatory for all devices implementing the PACKET feature set:

- PACKET

- DEVICE RESET

- EXECUTE DEVICE DIAGNOSTIC

- IDENTIFY DEVICE

- IDENTIFY PACKET DEVICE

- NOP

- READ SECTOR(S)

- SET FEATURES

The following General feature set commands are optional for all devices implementing the PACKET command feature set:

- FLUSH CACHE

The following General feature set commands are prohibited for use by devices implementing the PACKET command feature set.

- DOWNLOAD MICROCODE

- READ BUFFER

- READ DMA

- READ MULTIPLE

- READ VERIFY

- SET MULTIPLE MODE

- WRITE BUFFER

- WRITE DMA

- WRITE MULTIPLE

- WRITE SECTOR(S)

[Editors Note: Reset requirement need to be in each transport]

#### Identification of PACKET Command feature set devices

When executing a power-on, hardware, DEVICE RESET, or software reset, a device implementing the PACKET Command feature set shall perform the same reset protocol as other devices, but leaves the registers with a signature unique to PACKET Command feature set devices (See the appropriate transport documentation for a details).

In addition, the IDENTIFY DEVICE command shall not be executed but shall be command aborted and shall return a signature unique to devices implementing the PACKET Command feature set.  The IDENTIFY PACKET DEVICE command is used by the host to get identifying parameter information for a device implementing the PACKET Command feature set (See 827.17.5 and 7.18).

#### PACKET Command feature set resets

Devices implementing the PACKET Command feature set respond to power-on, hardware, and software resets as any other device except for the resulting contents in the device registers as described above.  However, software reset should not be issued while a PACKET command is in progress.  PACKET commands used by some devices do not terminate if a software reset is issued.

The DEVICE RESET command is provided to allow the device to be reset without affecting the other device on the bus.

#### The PACKET command

The PACKET command allows a host to send a command to the device via a command packet.  The command packet contains the command and command parameters that the device is to execute (See Clause 1).

Upon receipt of the PACKET command the device sets BSY to one and prepares to receive the command packet.  When ready, the device sets DRQ to one and clears BSY to zero.  The command packet is then transferred to the device by PIO transfer.  When the last word of the command packet is transferred, the device sets BSY to one, and clears DRQ to zero (See 7.25 as well as the appropriate transport documentation for a details).

[Editors Note: add a table the lists the feature sets and how they apply to both General and Packet devices]

### Power Management feature set

A device shall implement power management.  A device implementing the PACKET Command feature set may implement the power management as defined by the PACKET command set implemented by the device.  Otherwise, the device shall implement the Power Management feature set as described in this standard.

The Power Management feature set permits a host to modify the behavior of a device in a manner that reduces the power required to operate.  The Power Management feature set provides a set of commands and a timer that enable a device to implement low power consumption modes.  A register delivered command device that implements the Power Management feature set shall implement the following minimum set of functions: See also 4.6 and 4.12.

- A Standby timer

- CHECK POWER MODE command

- IDLE command

- IDLE IMMEDIATE command

- SLEEP command

- STANDBY command

- STANDBY IMMEDIATE command

A device that implements the PACKET Command feature set and implements the Power Management feature set shall implement the following minimum set of functions:

- CHECK POWER MODE command

- IDLE IMMEDIATE command

- SLEEP command

- STANDBY IMMEDIATE command

#### Power management commands

The CHECK POWER MODE command allows a host to determine if a device is currently in, going to or leaving Standby or Idle mode.  The CHECK POWER MODE command shall not change the power mode or affect the operation of the Standby timer.

The IDLE and IDLE IMMEDIATE commands move a device to Idle mode immediately from the Active or Standby modes.  The IDLE command also sets the Standby timer count and enables or disables the Standby timer.

The STANDBY and STANDBY IMMEDIATE commands move a device to Standby mode immediately from the Active or Idle modes.  The STANDBY command also sets the Standby timer count and enables or disables the Standby timer.

The SLEEP command moves a device to Sleep mode.  The device's interface becomes inactive at command completion of the SLEEP command.  A hardware or software reset or DEVICE RESET command is required to move a device out of Sleep mode.

#### Standby timer

The Standby timer provides a method for the device to automatically enter Standby mode from either Active or Idle mode following a host programmed period of inactivity.  If the Standby timer is enabled and if the device is in the Active or Idle mode, the device waits for the specified time period and if no command is received, the device automatically enters the Standby mode.

If the Standby timer is disabled, the device may automatically enter Standby mode.

#### Power modes

Figure 4 shows the set of mode transitions that shall be implemented.

Figure 4 - Power management state diagram4

PM0: Active:  This mode shall be entered when the device receives a media access command while in Idle or Standby mode.  This mode shall also be entered when the device is powered-up with the Power-Up In Standby feature not implemented or not enabled (See 4.12).

In Active mode the device is capable of responding to commands.  During the execution of a media access command a device shall be in Active mode.  Power consumption is greatest in this mode.

Transition PM0:PM0: When hardware reset, software reset, or DEVICE RESET command is received, the device shall make a transition to the PM0: Active mode when the reset protocol is completed.

Transition PM0:PM1: When an IDLE or IDLE IMMEDIATE command is received or when a vendor specific implementation determines a transition is required, then the device shall make a transition to the PM1:Idle mode.

Transition PM0:PM2: When a STANDBY or STANDBY IMMEDIATE command is received, the Standby timer expires, or a vendor specific implementation determines a transition is required, then the device shall make a transition to the PM2:Standby mode.

Transition PM0:PM3: When a SLEEP command is received, the device shall make a transition to the PM3:Sleep mode.

PM1: Idle:  This mode shall be entered when the device receives an IDLE or IDLE IMMEDIATE command.  Some devices may perform vendor specific internal power management and make a transition to the Idle mode without host intervention.

In Idle mode the device is capable of responding to commands but the device may take longer to complete commands than when in the Active mode.  Power consumption may be reduced from that of Active mode.

Transition PM1:PM0: When a media access is required, the device shall make a transition to the PM0:Active mode.

Transition PM1:PM1: When hardware reset, software reset, or DEVICE RESET command is received, the device shall make a transition to the PM1:Idle mode when the reset protocol is completed.

Transition PM1:PM2: When a STANDBY or STANDBY IMMEDIATE command is received, the Standby timer expires, or a vendor specific implementation determines a transition is required, then the device shall make a transition to the PM2:Standby mode.

Transition PM1:PM3: When a SLEEP command is received, the device shall make a transition to the PM3:Sleep mode.

PM2: Standby:  This mode shall be entered when the device receives a STANDBY command, a STANDBY IMMEDIATE command, or the Standby timer expires.  Some devices may perform vendor specific internal power management and make a transition to the Standby mode without host intervention.  This mode shall also be entered when the device is powered-up with the Power-Up In Standby feature implemented and enabled.

In Standby mode the device is capable of responding to commands but the device may take longer to complete commands than in the Idle mode.  The time to respond could be as long as 30 s.  Power consumption may be reduced from that of Idle mode.

Transition PM2:PM0: When a media access is required, the device shall make a transition to the PM0:Active mode.

Transition PM2:PM1: When an IDLE or IDLE IMMEDIATE command is received, or a vendor specific implementation determines a transition is required, then the device shall make a transition to the PM1:Idle mode.

Transition PM2:PM2: When hardware reset, software reset, or DEVICE RESET command is received, the device shall make a transition to the PM2:Standby mode when the reset protocol is completed.

Transition PM2:PM3: When a SLEEP command is received, the device shall make a transition to the PM3:Sleep mode.

PM3: Sleep:  This mode shall be entered when the device receives a SLEEP command.

In Sleep mode the device requires a hardware or software reset or a DEVICE RESET command to be activated.  The time to respond could be as long as 30 s.  Sleep mode provides the lowest power consumption of any mode.

In Sleep mode, the device's interface is not active.  The content of the Status register is invalid in this mode.

Transition PM3:PM2:, When hardware reset, software reset, or DEVICE RESET command is received the device shall make a transition to the PM2:Standby mode.

### Advanced Power Management feature set

The Advanced Power Management feature set is an optional feature set that allows the host to select a power management level.  The power management level is specified using a scale from the lowest power consumption setting of 01h to the maximum performance level of FEh.  Device performance may increase with increasing power management levels.  Device power consumption may increase with increasing power management levels.  A device may implement one power management method for two or more contiguous power management levels.  For example, a device may implement one power management method from level 80h to A0h and a higher performance, higher power consumption method from level A1h to FEh.  Advanced power management levels 80h and higher do not permit the device to spin down to save power.

The Advanced Power Management feature set uses the following functions:

- A SET FEATURES subcommand to enable Advanced Power Management

- A SET FEATURES subcommand to disable Advanced Power Management

Advanced Power Management is independent of the Standby timer setting.  If both Advanced Power Management and the Standby timer are set, the device will go to the Standby state when the timer times out or the device’s Advanced Power Management algorithm indicates that the Standby state should be entered.

The IDENTIFY DEVICE command indicates that Advanced Power Management is supported, whether Advanced Power Management is enabled, and the current advanced power management level if Advanced Power Management is enabled.

### Security Mode feature set

The optional Security Mode feature set is a password system that restricts access to user data stored on a device.  The system has two passwords, User and Master, and two security levels, High and Maximum.  The security system is enabled by sending a user password to the device with the SECURITY SET PASSWORD command.  When the security system is enabled, access to user data on the device is denied after a power cycle until the User password is sent to the device with the SECURITY UNLOCK command.

A Master password may be set in addition to the User password.  The purpose of the Master password is to allow an administrator to establish a password that is kept secret from the user, and which may be used to unlock the device if the User password is lost.  Setting the Master password does not enable the password system.

The security level is set to High or Maximum with the SECURITY SET PASSWORD command.  The security level determines device behavior when the Master password is used to unlock the device.  When the security level is set to High the device requires the SECURITY UNLOCK command and the Master password to unlock.  When the security level is set to Maximum the device requires a SECURITY ERASE PREPARE command and a SECURITY ERASE UNIT command with the masterpassword to unlock.  Execution of the SECURITY ERASE UNIT command erases all user data on the device.

The SECURITY FREEZE LOCK command prevents changes to passwords until a following power cycle.  The purpose of the SECURITY FREEZE LOCK command is to prevent password setting attacks on the security system.

A device that implements the Security Mode feature set shall implement the following minimum set of commands:

- SECURITY SET PASSWORD

- SECURITY UNLOCK

- SECURITY ERASE PREPARE

- SECURITY ERASE UNIT

- SECURITY FREEZE LOCK

- SECURITY DISABLE PASSWORD

Support of the Security Mode feature set is indicated in IDENTIFY DEVICE data word 82 and data word 128.

#### Security mode initial setting

When the device is shipped by the manufacturer, the state of the Security Mode feature shall be disabled.  The initial Master password value is not defined by this standard.

If the Master Password Revision Code feature is supported, the Master Password Revision Code shall be set to FFFEh by the manufacturer.

#### User password lost

If the User password sent to the device with the SECURITY UNLOCK command does not match the user password previously set with the SECURITY SET PASSWORD command, the device shall not allow the user to access data.

If the Security Level was set to High during the last SECURITY SET PASSWORD command, the device shall unlock if the Master password is received.

If the Security Level was set to Maximum during the last SECURITY SET PASSWORD command, the device shall not unlock if the Master password is received.  The SECURITY ERASE UNIT command shall erase all user data and unlock the device if the Master password matches the last Master password  previously set with the SECURITY SET PASSWORD command.

#### Attempt limit for SECURITY UNLOCK command

The device shall have an attempt limit counter.  The purpose of this counter is to defeat repeated trial attacks.  After each failed User or Master password SECURITY UNLOCK command, the counter is decremented.  When the counter value reaches zero the EXPIRE bit (bit 4) of IDENTIFY DEVICE data word 128 is set to one, and the SECURITY UNLOCK and SECURITY UNIT ERASE commands are command aborted until the device is powered off or hardware reset.  The EXPIRE bit shall be cleared to zero after power-on or hardware reset.  The counter shall be set to five after a power-on or hardware reset.

#### Security mode states

Figure 5 decribes security mode states and state transitions.

Figure 5 - Security mode state diagram 5

SEC0: Powered down/Security disabled:  This mode shall be entered when the device is powered-down with the Security Mode feature set disabled.

Transition SEC0:SEC1: When the device is powered-up, the device shall make a transition to the SEC1: Security disabled/not Frozen state.

SEC1: Security disabled/not Frozen:  This mode shall be entered when the device is powered-up or a hardware reset is received with the Security Mode feature set disabled or when the Security Mode feature set is disabled by a SECURITY DISABLE PASSWORD or SECURITY ERASE UNIT command.

In this state, the device is capable of responding to all commands (See Table 4 Unlocked column).

Transition SEC1:SEC0: When the device is powered-down, the device shall make a transition to the SEC0: Powered down/Security disabled state.

Transition SEC1:SEC1: When the device receives a hardware reset, the device shall make a transition to the SEC1: Security disabled/not Frozen state.

Transition SEC1:SEC2: When a SECURITY FREEZE LOCK command is received, the device shall make a transition to the SEC2: Security disabled/Frozen state.

Transition SEC1:SEC5: When a SECURITY SET PASSWORD command is received, the device shall make a transition to the SEC5: Unlocked/not frozen state

SEC2: Security disabled/Frozen:  This mode shall be entered when the device receives a SECURITY FREEZE LOCK command while in Security disabled/not Frozen state.

In this state, the device is capable of responding to all commands except those indicated in Table 4 Frozen column.

Transition SEC2:SEC0: When the device is powered-down, the device shall make a transition to the SEC0: Powered down/Security disabled state.

Transition SEC2:SEC1: When the device receives a hardware reset, the device shall make a transition to the SEC1: Security disabled/not Frozen state.

SEC3: Powered down/Security enabled:  This mode shall be entered when the device is powered-down with the Security Mode feature set enabled.

Transition SEC3:SEC4: When the device is powered-up, the device shall make a transition to the SEC4: Security enabled/locked state.

SEC4: Security enabled/Locked:  This mode shall be entered when the device is powered-up or a hardware reset is received with the Security Mode feature set enabled.

In this state, the device shall only respond to commands that do not access data in the user data area of the media (See Table 4 Locked column).

Transition SEC4:SEC3: When the device is powered-down, the device shall make a transition to the SEC3: Powered down/Security enabled state.

Transition SEC4:SEC4: When the device receives a hardware reset, the device shall make a transition to the SEC4: Security enabled/locked state.

Transition SEC4:SEC5: When a valid SECURITY UNLOCK command is received, the device shall make a transition to the SEC5: Unlocked/not Frozen state.

Transition SEC4:SEC1: When a SECURITY ERASE PREPARE command is received and is followed by a SECURITY ERASE UNIT command, the device shall make a transition to the SEC1: Security disabled/not Frozen state.

SEC5: Unlocked/not Frozen:  This mode shall be entered when the device receives a SECURITY SET PASSWORD command to enable the lock or a SECURITY UNLOCK command.

In this state, the device shall respond to all commands (See Table 4 Unlocked column).

Transition SEC5a:SEC1: When a valid SECURITY DISABLE PASSWORD command is received, the device shall make a transition to the SEC1: Security disabled/not Frozen state.

Transition SEC5b:SEC1: When a SECURITY ERASE PREPARE command is received and is followed by a SECURITY ERASE UNIT command, the device shall make a transition to the SEC1: Security disabled/not Frozen state.

Transition SEC5:SEC6: When a SECURITY FREEZE LOCK command is received, the device shall make a transition to the SEC6: Unlocked/Frozen state.

Transition SEC5:SEC3: When the device is powered-down, the device shall make a transition to the SEC3: Powered down/Security enabled state.

Transition SEC5:SEC4: When the device receives a hardware reset, the device shall make a transition to the SEC4: Security enabled/Locked state.

SEC6: Unlocked/ Frozen:  This mode shall be entered when the device receives a SECURITY FREEZE LOCK command while in Unlocked/Locked state.

In this state, the device is capable of responding to all commands except those indicated in Table 4 Frozen column.

Transition SEC6:SEC3: When the device is powered-down, the device shall make a transition to the SEC3: Powered down/Security enabled state.

Transition SEC6:SEC4: When the device receives a hardware reset, the device shall make a transition to the SEC4: Security enabled/Locked state.

Table 4 - Security mode command actions 4

| Command | Locked | Unlocked | Frozen |
| --- | --- | --- | --- |
| CFA ERASE SECTORS | Command aborted | Executable | Executable |
| CFA REQUEST EXTENDED ERROR CODE | Executable | Executable | Executable |
| CFA TRANSLATE SECTOR | Executable | Executable | Executable |
| CFA WRITE MULTIPLE WITHOUT ERASE | Command aborted | Executable | Executable |
| CFA WRITE SECTORS WITHOUT ERASE | Command aborted | Executable | Executable |
| CHECK MEDIA CARD TYPE | Command aborted | Executable | Executable |
| CHECK POWER MODE | Executable | Executable | Executable |
| CONFIGURE STREAM | Command aborted | Executable | Executable |
| DEVICE CONFIGURATION | Command aborted | Executable | Executable |
| DEVICE RESET | Executable | Executable | Executable |
| DOWNLOAD MICROCODE | Vendor Specific | Vendor Specific | Vendor Specific |
| EXECUTE DEVICE DIAGNOSTIC | Executable | Executable | Executable |
| FLUSH CACHE | Command aborted | Executable | Executable |
| FLUSH CACHE EXT | Command aborted | Executable | Executable |
| GET MEDIA STATUS | Command aborted | Executable | Executable |
| IDENTIFY DEVICE | Executable | Executable | Executable |
| IDENTIFY PACKET DEVICE | Executable | Executable | Executable |
| IDLE | Executable | Executable | Executable |
| IDLE IMMEDIATE | Executable | Executable | Executable |
| MEDIA EJECT | Command aborted | Executable | Executable |
| MEDIA LOCK | Command aborted | Executable | Executable |
| MEDIA UNLOCK | Command aborted | Executable | Executable |
| NOP | Executable | Executable | Executable |
| PACKET | Command aborted | Executable | Executable |
| READ BUFFER | Executable | Executable | Executable |
| READ DMA | Command aborted | Executable | Executable |
| READ DMA EXT | Command aborted | Executable | Executable |
| READ DMA QUEUED | Command aborted | Executable | Executable |
| READ DMA QUEUED EXT | Command aborted | Executable | Executable |
| READ LOG EXT | Command aborted | Executable | Executable |
| READ MULTIPLE | Command aborted | Executable | Executable |
| READ MULTIPLE EXT | Command aborted | Executable | Executable |
| READ NATIVE MAX ADDRESS | Executable | Executable | Executable |
| READ NATIVE MAX ADDRESS EXT | Executable | Executable | Executable |
| READ SECTOR(S) | Command aborted | Executable | Executable |
| READ SECTOR(S) EXT | Command aborted | Executable | Executable |
| READ STREAM DMA EXT | Command aborted | Executable | Executable |
| READ STREAM EXT | Command aborted | Executable | Executable |
| READ VERIFY SECTOR(S) | Command aborted | Executable | Executable |
| READ VERIFY SECTOR(S) EXT | Command aborted | Executable | Executable |
| SECURITY DISABLE PASSWORD | Command aborted | Executable | Command aborted |
| SECURITY ERASE PREPARE | Executable | Executable | Command aborted |
| SECURITY ERASE UNIT | Executable | Executable | Command aborted |
| SECURITY FREEZE LOCK | Command aborted | Executable | Executable |
| SECURITY SET PASSWORD | Command aborted | Executable | Command aborted |
| SECURITY UNLOCK | Executable | Executable | Command aborted |
| SERVICE | Command aborted | Executable | Executable |
| SET FEATURES | Executable | Executable | Executable |
| SET MAX ADDRESS | Command aborted | Executable | Executable |
| SET MAX ADDRESS EXT | Command aborted | Executable | Executable |
| SET MAX SET PASSWORD | Command aborted | Executable | Executable |
| SET MAX LOCK | Command aborted | Executable | Executable |
| SET MAX FREEZE LOCK | Command aborted | Executable | Executable |
| SET MAX UNLOCK | Command aborted | Executable | Executable |
| SET MULTIPLE MODE | Executable | Executable | Executable |
| SLEEP | Executable | Executable | Executable |
| SMART DISABLE OPERATIONS | Executable | Executable | Executable |
| SMART ENABLE/DISABLE AUTOSAVE | Executable | Executable | Executable |
| SMART ENABLE OPERATIONS | Executable | Executable | Executable |
| SMART EXECUTE OFF-LINE IMMEDIATE | Executable | Executable | Executable |
| SMART READ DATA | Executable | Executable | Executable |

(continued)

Table 4 - Security mode command actions (continued)

| Command | Locked | Unlocked | Frozen |
| --- | --- | --- | --- |
| SMART READ LOG | Executable | Executable | Executable |
| SMART RETURN STATUS | Executable | Executable | Executable |
| SMART WRITE LOG | Executable | Executable | Executable |
| STANDBY | Executable | Executable | Executable |
| STANDBY IMMEDIATE | Executable | Executable | Executable |
| WRITE BUFFER | Executable | Executable | Executable |
| WRITE DMA | Command aborted | Executable | Executable |
| WRITE DMA EXT | Command aborted | Executable | Executable |
| WRITE DMA FUA EXT | Command aborted | Executable | Executable |
| WRITE DMA QUEUED | Command aborted | Executable | Executable |
| WRITE DMA QUEUED EXT | Command aborted | Executable | Executable |
| WRITE DMA QUEUED FUA EXT | Command aborted | Executable | Executable |
| WRITE LOG EXT | Command aborted | Executable | Executable |
| WRITE MULTIPLE | Command aborted | Executable | Executable |
| WRITE MULTIPLE EXT | Command aborted | Executable | Executable |
| WRITE MULTIPLE FUA EXT | Command aborted | Executable | Executable |
| WRITE SECTOR(S) | Command aborted | Executable | Executable |
| WRITE SECTOR(S) EXT | Command aborted | Executable | Executable |
| WRITE STREAM DMA EXT | Command aborted | Executable | Executable |
| WRITE STREAM EXT | Command aborted | Executable | Executable |

(concluded)

### SMART (Self-monitoring, analysis, and reporting technology) feature set

The intent of self-monitoring, analysis, and reporting technology (the SMART feature set) is to protect user data and minimize the likelihood of unscheduled system downtime that may be caused by predictable degradation and/or fault of the device.  By monitoring and storing critical performance and calibration parameters, SMART feature set devices attempt to predict the likelihood of near-term degradation or fault condition.  Providing the host system the knowledge of a negative reliability condition allows the host system to warn the user of the impending risk of a data loss and advise the user of appropriate action.  Support of this feature set is indicated in the IDENTIFY DEVICE data.

Devices that implement the PACKET Command feature set shall not implement the SMART feature set as described in this standard.  Devices that implement the PACKET Command feature set and SMART shall implement SMART as defined by the command packet set implemented by the device.  This feature set is optional if the PACKET Command feature set is not supported.

#### Device SMART data structure

SMART feature set capability and status information for the device are stored in the device SMART data structure.  The off-line data collection capability and status data stored herein may be useful to the host if the SMART EXECUTE OFF-LINE IMMEDIATE command is implemented (See 7.54.5).

#### On-line data collection

Collection of SMART data in an “on-line” mode shall have no impact on device performance.  The SMART data that is collected or the methods by which data is collected in this mode may be different than those in the off-line data collection mode for any particular device and may vary from one device to another.

#### Off-line data collection

The device shall use off-line mode for data collection and self-test routines that have an impact on performance if the device is required to respond to commands from the host while performing that data collection.  This impact on performance may vary from device to device.  The data that is collected or the methods by which the data is collected in this mode may be different than those in the on-line data collection mode for any particular device and may vary from one device to another.

#### Threshold exceeded condition

This condition occurs when the device’s SMART reliability status indicates an impending degrading or fault condition.

#### SMART feature set commands

These commands use a single command code and are differentiated from one another by the value placed in the Features register (See 7.54).

If the SMART feature set is implemented, the following commands shall be implemented.

- SMART DISABLE OPERATIONS

- SMART ENABLE/DISABLE AUTOSAVE

- SMART ENABLE OPERATIONS

- SMART RETURN STATUS

If the SMART feature set is implemented, the following commands may be implemented.

- SMART EXECUTE OFF-LINE IMMEDIATE

- SMART READ DATA

- SMART READ LOG

- SMART WRITE LOG

- READ LOG EXT

- WRITE LOG EXT

#### SMART operation with power management modes

When used with a host that has implemented the Power Management feature set, a SMART enabled device should automatically save the device accumulated SMART data upon receipt of an IDLE IMMEDIATE, STANDBY IMMEDIATE, or SLEEP command or upon return to an Active or Idle mode from a Standby mode (See 7.54.6).

If a SMART feature set enabled device has been set to use the Standby timer, the device should automatically save the device accumulated SMART data prior to going from an Idle mode to the Standby mode or upon return to an Active or Idle mode from a Standby mode.

A device shall not execute any routine to automatically save the device accumulated SMART data while the device is in a Standby or Sleep mode.

#### SMART device error log reporting

Logging of reported errors is an optional SMART feature.  If error logging is supported by a device, it is indicated in byte 370 of the SMART READ DATA command response and bit 0 of word 84 of the IDENTIFY DEVICE response.  If error logging is supported, the device shall provide information on the last five errors that the device reported as described in the SMART READ LOG command (See 7.54.7).  The device may also provide additional vendor specific information on these reported errors.

If error logging is supported, it shall not be disabled when SMART is disabled.  Error log information shall be gathered when the device is powered-on except that logging of errors when in a reduced power mode is optional.  If errors are logged when in a reduced power mode, the reduced power mode shall not change.  Disabling SMART shall disable the delivering of error log information via the SMART READ LOG command.

If a device receives a firmware modification, all error log data shall be discarded and the device error count for the life of the device shall be reset to zero.

### Host Protected Area feature set

A reserved area for data storage outside the normal operating system file system is required for several specialized applications.  Systems may wish to store configuration data or save memory to the device in a location that the operating systems cannot change.  The optional Host Protected Area feature set allows a portion of the device to be reserved for such an area when the device is initially configured.  A device that implements the Host Protected Area feature set shall implement the following minimum set of commands:

- READ NATIVE MAX ADDRESS

- SET MAX ADDRESS

A device that implements the Host Protected Area feature set and supports the 48-bit Address feature set shall implement the following additional set of commands:

- READ NATIVE MAX ADDRESS EXT

- SET MAX ADDRESS EXT

Devices supporting this feature set shall set bit 10 of word 82 to one in the data returned by the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command.

In addition, a device supporting the Host Protected Area feature set may optionally include the security extensions.  The Host Protected Area security commands use a single command code and are differentiated from one another by the value placed in the Features register.

- SET MAX SET PASSWORD

- SET MAX LOCK

- SET MAX FREEZE LOCK

- SET MAX UNLOCK

Devices supporting these extensions shall set bit 10 of word 82 and bit 8 of word 83 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data to one.

The READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command allows the host to determine the maximum native address space of the device even when a protected area has been allocated.

The SET MAX ADDRESS or SET MAX ADDRESS EXT command allows the host to redefine the maximum address of the user accessible address space.  That is, when the SET MAX ADDRESS or SET MAX ADDRESS EXT command is issued with a maximum address less than the native maximum address, the device reduces the user accessible address space to the maximum specified by the command, providing a protected area above that maximum address.  The SET MAX ADDRESS or SET MAX ADDRESS EXT command shall be immediately preceded by a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command.  After the SET MAX ADDRESS or SET MAX ADDRESS EXT command has been issued, the device shall report only the reduced user address space in response to an IDENTIFY DEVICE command in words 60,  61, 100, 101, 102, and 103.  Any read or write command to an address above the maximum address specified by the SET MAX ADDRESS or SET MAX ADDRESS EXT command shall cause command completion with the IDNF bit set to one and ERR set to one, or command aborted.  A volatility bit in the Sector Count register allows the host to specify if the maximum address set is preserved across power-on or hardware reset cycles.  On power-on or hardware reset the device maximum address returns to the last non-volatile address setting regardless of subsequent volatile SET MAX ADDRESS or SET MAX ADDRESS EXT commands.  If the SET MAX ADDRESS or SET MAX ADDRESS EXT command is issued with a value that exceeds the native maximum address command aborted shall be returned.

Typical use of these commands would be:

On reset

- BIOS receives control after a system reset;

- BIOS issues a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command to find the max capacity of the device;

- BIOS issues a SET MAX ADDRESS or SET MAX ADDRESS EXT command to the values returned by READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT;

- BIOS reads configuration data from the highest area on the disk;

- BIOS issues a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command followed by a SET MAX ADDRESS or SET MAX ADDRESS EXT command to reset the device to the size of the file system.

On save to disk

- BIOS receives control prior to shut down;

- BIOS issues a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command to find the max capacity of the device;

- BIOS issues a volatile SET MAX ADDRESS or SET MAX ADDRESS EXT command to the values returned by READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT;

- Memory is copied to the reserved area;

- Shut down completes;

- On power-on or hardware reset the device max address returns to the last non-volatile setting.

These commands are intended for use only by system BIOS or other low-level boot time process.  Using these commands outside BIOS controlled boot or shutdown may result in damage to file systems on the device.  Devices should return command aborted if a subsequent non-volatile SET MAX ADDRESS or SET MAX ADDRESS EXT command is received after a power-on or hardware reset.

The SET MAX SET PASSWORD command allows the host to define the password to be used during the current power-on cycle.  The password does not persist over a power cycle but does persist over a hardware or software reset.  This password is not related to the password used for the Security Mode Feature set.  When the password is set the device is in the Set_Max_Unlocked mode.

The SET MAX LOCK command allows the host to disable the SET MAX commands (except SET MAX UNLOCK) until the next power cycle or the issuance and acceptance of the SET MAX UNLOCK command.  When this command is accepted the device is in the Set_Max_Locked mode.

The SET MAX UNLOCK command changes the device from the Set_Max_Locked mode to the Set_Max_Unlocked mode.

The SET MAX FREEZE LOCK command allows the host to disable the SET MAX commands (including SET MAX UNLOCK) until the next power cycle.  When this command is accepted the device is in the Set_Max_Frozen mode.

#### BIOS determination of SET MAX security exension status

When the device is locked bit 8 of word 86 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data shall be set to one.

#### BIOS locking SET MAX

To allow for multiple BIOSs to gain access to the protected area the host BIOS should only lock the protected area immediately prior to booting the operating system.

Figure 6 is the SET MAX state diagram.

Figure 6 - SET MAX security state diagram 6

SM0: Set_Max_Security_Inactive: This state shall be entered when the device is powered-on.

When in this state, SET MAX security is disabled.

Transition SM0a:SM0: When a SET MAX ADDRESS command is received, the command shall be executed and the device shall make a transition to the SM0: Set_MAX_Security_Inactive state.

Transition SM0b:SM0: When a SET MAX LOCK, SET MAX UNLOCK, or SET MAX FREEZE LOCK command is received, the device shall abort the command and make a transition to the SM0: Set_MAX_Security_Inactive state.

Transition SM0:SM1: When a SET MAX SET PASSWORD command is received, the device shall make a transition to the SM1: Set_Max_Unlocked state.

SM1: Set_Max_Unlocked: This state is entered when a SET MAX SET PASSWORD or a SET MAX UNLOCK command is received.

When in this state, a SET MAX security password has been established and the SET MAX security is unlocked.  Bit 8 of word 86 of the IDENTIFY DEVICE data shall be set to one.

Transition SM1a:SM1: When a SET MAX ADDRESS command is received, the command shall be executed and the device shall make a transition to the SM1: Set_MAX_Unlocked state.

Transition SM1b:SM1: When a SET MAX SET PASSWORD is received, the password stored by the device shall be changed to the new value and the device shall make a transition to the SM1: Set_MAX_Unlocked state.

Transition SM1c:SM1: When a SET MAX UNLOCK command is received, the command shall not be executed and the device shall make a transition to the SM1: Set_MAX_Unlocked state.

Transition SM1:SM2: When a SET MAX LOCK command is received, the device shall make a transition to the SM2: Set_Max_Locked state.

Transition SM1:SM3: When a SET MAX FREEZE LOCK command is received, the device shall make a transition to the SM3: Set_Max_Frozen state.

SM2: Set_Max_Locked: This state is entered when a SET MAX LOCK command is received.

When in this state, a SET MAX security password has been established and the SET MAX security is locked.  Bit 8 of word 86 of the IDENTIFY DEVICE data shall be set to one.

Transition SM2a:SM2: When a SET MAX ADDRESS or SET MAX SET PASSWORD command is received, the command shall be aborted and the device shall make a transition to the SM2: Set_Max_Locked state.

Transition SM2b:SM2: When a SET MAX LOCK command is received, the command shall be executed and the device shall make a transition to the SM2: Set_Max_Locked state.

Transition SM2:SM1: When a SET MAX UNLOCK command is received, the device shall make a transition to the SM1: Set Max Unlocked state.

Transition SM2:SM3: When a SET MAX FREEZE LOCK command is received, the device may make a transition to the SM3: Set_Max_Frozen state.  Hosts should not issue the SET MAX FREEZE LOCK command when in this state.

SM3: Set_Max_Frozen: This state is entered when a SET MAX FREEZE LOCK command is received.

In this state, the device may not transition to any other state except by a power cycling.  When in this mode bit 8 of word 86 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data shall be set to one.

Transition SM3:SM3: When a SET MAX ADDRESS, SET MAX SET PASSWORD, SET MAX UNLOCK, SET MAX FREEZE LOCK, or SET MAX LOCK command is received, the command shall be aborted and the device shall make a transition to the SM3: Set_Max_Frozen state.

### CompactFlash Association (CFA) feature set

The optional CompactFlash Association (CFA) feature set provides support for solid state memory devices.  A device that implements the CFA feature set shall implement the following minimum set of commands:

- CFA REQUEST EXTENDED ERROR CODE

- CFA WRITE SECTORS WITHOUT ERASE

- CFA ERASE SECTORS

- CFA WRITE MULTIPLE WITHOUT ERASE

- CFA TRANSLATE SECTOR

- SET FEATURES Enable/Disable 8-bit transfer

Devices reporting the value 848Ah in IDENTIFY DEVICE data word 0 or devices having bit 2 of IDENTIFY DEVICE data word 83 set to one shall support the CFA feature Set.  If the CFA feature set is implemented, all five commands shall be implemented.

Support of DMA commands is optional for devices that support the CFA feature set.

The CFA ERASE SECTORS command preconditions the sector for a subsequent CFA WRITE SECTORS WITHOUT ERASE or CFA WRITE MULTIPLE WITHOUT ERASE command to achieve higher performance during the write operation.  The CFA TRANSLATE SECTOR command provides information about a sector such as the number of write cycles performed on that sector and an indication of the sector’s erased precondition.  The CFA REQUEST EXTENDED ERROR CODE command provides more detailed error information.

Command codes B8h through BFh are reserved for assignment by the CompactFlash Association.

### Removable Media Status Notification and Removable Media feature sets

This section describes two feature sets that secure the media in removable media storage devices using the ATA/ATAPI interface protocols.  The Removable Media Status Notification feature set is intended for use in both devices implementing the PACKET Command feature set and those not implementing the PACKET Command feature set.  The Removable Media feature set is intended for use only in devices not implementing the PACKET Command feature set.  Only one of these feature sets shall be enabled at any time.  If the Removable Media Status Notification feature set is in use then the Removable Media feature set is disabled and vice versa.

The reasons for implementing the Removable Media Status Notification feature Set or the Removable Media feature set are:

- to prevent data loss caused by writing to new media while still referencing the previous media’s information.

- to prevent data loss by locking the media until completion of a cached write.

- to prevent removal of the media by unauthorized persons.

#### Removable Media Status Notification feature set

The Removable Media Status Notification feature set is the preferred feature set for securing the media in removable media storage devices.  This feature set uses the SET FEATURES command to enable Removable Media Status Notification.  Removable Media Status Notification gives the host system maximum control of the media.  The host system determines media status by issuing the GET MEDIA STATUS command and controls the device eject mechanism via the MEDIA EJECT command (for devices not implementing the PACKET Command feature set) or the START/STOP UNIT command (for devices implementing the PACKET Command feature set, See Clause 1).  While Removable Media Status Notification is enabled devices not implementing the PACKET Command feature set execute MEDIA LOCK and MEDIA UNLOCK commands without changing the media lock state (no-operation).  While Removable Media Status Notification is enabled the eject button does not eject the media.

Removable Media Status Notification is persistent through medium removal and insertion and is only disabled via the SET FEATURES command, hardware reset, software reset, the DEVICE RESET command, the EXECUTE DEVICE DIAGNOSTIC command, or power-on reset.  Removable Media Status Notification shall be re-enabled after any of the previous reset conditions occur.  All media status is reset when Removable Media Status Notification is disabled because a reset condition occurred.  Any pending media change or media change request is cleared when the Removable Media Status Notification reset condition occurs.

The following commands are defined to implement the Removable Media Status Notification feature set.

- GET MEDIA STATUS

- MEDIA EJECT

- SET FEATURES (Enable media status notification)

- SET FEATURES (Disable media status notification)

NOTE  Devices implementing the PACKET Command feature set control the media eject mechanism via the START/STOP UNIT command packet.

The preferred sequence of events to use the Removable Media Status Notification feature set is as follows:

- Host system checks whether or not the device implements the PACKET Command feature set via the device signature in the Command Block registers.

- Host system issues the IDENTIFY DEVICE command or the IDENTIFY PACKET DEVICE command and checks that the device is a removable media device and that the Removable Media Status Notification feature set is supported.

- Host system uses the SET FEATURES command to enable Media Status Notification that gives control of the media to the host.  At this time the host system checks the LBA High register to determine if:

- the device is capable of locking the media.

- the device is capable of power ejecting the media.

- Media Status Notification was enabled prior to this command.

- Host system periodically checks media status using the GET MEDIA STATUS command to determine if any of the following events occurred:

- no media is present in the device (NM).

- media was changed since the last command (MC).

- a media change request has occurred (MCR).

- media is write protected (WP).

#### Removable Media feature set

The Removable Media feature set is intended only for devices not implementing the PACKET Command feature set.  This feature set operates with Media Status Notification disabled.  The MEDIA LOCK and MEDIA UNLOCK commands are used to secure the media and the MEDIA EJECT command is used to remove the media.  While the media is locked, the eject button does not eject the media.  Media status is determined by checking the media status bits returned by the MEDIA LOCK and MEDIA UNLOCK commands.

Power-on reset, hardware reset, and the EXECUTE DEVICE DIAGNOSTIC command clear the Media Lock (LOCK) state and the Media Change Request (MCR) state.  Software reset clears the Media Lock (LOCK) state, clears the Media Change Request (MCR) state, and preserves the Media Change (MC) state.

The following commands are defined to implement the Removable Media feature set.

- MEDIA EJECT

- MEDIA LOCK

- MEDIA UNLOCK

The preferred sequence of events to use the Removable Media feature set is as follows:

- Host system checks whether or not the device implements the PACKET Command feature set via the device signature in the Command Block registers.

- Host system issues the IDENTIFY DEVICE command and checks that the device is a removable media device and that the Removable Media feature set is supported.

- Host system periodically issues MEDIA LOCK commands to determine if:

- no media is present in the device (NM) - media is locked if present.

- a media change request has occurred (MCR).

### Power-Up In Standby feature set

The optional Power-Up In Standby feature set allows devices to be powered-up into the Standby power management state to minimize inrush current at power-up and to allow the host to sequence the spin-up of devices.  This optional feature set may be enabled or disabled via the SET FEATURES command or may be enabled by use of a jumper or similar means, or both.  When enabled by a jumper, the feature set shall not be disabled via the SET FEATURES command.  The IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data indicates whether this feature set is implemented and/or enabled.

The enabling of this feature set shall be persistent after power-down and power-up.  When this feature set is enabled, the device shall power-up into Standby.

A device may implement a SET FEATURES subcommand that notifies the device to spin-up to the Active state when the device has powered-up into Standby.  If the device implements this SET FEATURES subcommand and power-up into Standby is enabled, the device shall remain in Standby until the SET FEATURES subcommand is received.  If the device implements this SET FEATURES subcommand, the fact that the feature is implemented is reported in the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

If the device:

- implements the Enable/disable Power-up in Standby subcommand,

- power-up into Standby is enabled, and

- an IDENTIFY DEVICE or IDENTIFY PACKET DEVICE is received while the device is in Standby as a result of powering up into Standby,

the device shall respond to the command and remain in Standby without spinning-up.

If the device has IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data that requires access to the media, the device shall set word 0 bit 2 to one to indicate that the response is incomplete.  At a minimum, word 0 and word 2 shall be correctly reported.  Those fields that cannot be provided shall be filled with zeros.  Once the full IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data has been accessed, a full response shall be returned until the next power-down/power-up sequence has taken place.

If the device does not implement the SET FEATURES subcommand to spin-up the device after power-up and power-up into Standby is enabled, the device shall spin-up upon receipt of the first command that requires the device to access the media.

### Automatic Acoustic Management (AAM) feature set

The Automatic Acoustic Management feature set is an optional feature set that allows the host to select an acoustic management level.  The acoustic management level ranges from the setting of 00h to FFh, although many levels are currently reserved (See Table 43).  Device performance and acoustic emanation may increase with increasing acoustic management levels.  The acoustic management levels may contain discrete bands.  For example, a device may implement one acoustic management method from level 80h to A0h, and a higher performance, higher acoustic emanation method from level A1h to FEh.

The Automatic Acoustic Management feature set uses the following functions:

- A SET FEATURES subcommand to enable the Automatic Acoustic Management feature set

- A SET FEATURES subcommand to disable the Automatic Acoustic Management feature set

The IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data indicates if the Automatic Acoustic Management feature set is supported, if the Automatic Acoustic Management feature set is enabled, and the current automatic acoustic management level if the Automatic Acoustic Management feature set is enabled.

### 48-bit Address feature set

The optional 48-bit Address feature set allows devices with capacities up to 281,474,976,710,655 sectors.  This allows device capacity up to 144,115,188,075,855,360 bytes.  In addition, the number of sectors that may be transferred by a single command are increased by increasing the allowable sector count to 16 bits.

The commands in the 48-bit Address feature set are prohibited from use for devices implementing the PACKET Command feature set.

Commands unique to the 48-bit Address feature set are:

- FLUSH CACHE EXT

- READ DMA EXT

- READ DMA QUEUED EXT

- READ MULTIPLE EXT

- READ NATIVE MAX ADDRESS EXT

- READ SECTOR(S) EXT

- READ VERIFY SECTOR(S) EXT

- SET MAX ADDRESS EXT

- WRITE DMA EXT

- WRITE DMA FUA EXT

- WRITE DMA QUEUED EXT

- WRITE DMA QUEUED FUA EXT

- WRITE MULTIPLE EXT

- WRITE MULTIPLE FUA EXT

- WRITE SECTOR(S) EXT

The 48-bit Address feature set operates in LBA only.  Devices implementing the 48-bit Address feature set shall also implement commands that use 28-bit addressing.  28-bit and 48-bit commands may be intermixed.  Support of the 48-bit Address feature set is indicated in the IDENTIFY DEVICE response.

In a device implementing the 48-bit Address feature set, the Features register, the Sector Count register, the LBA Low register, the LBA Mid register, and the LBA High register are each a two byte deep FIFO.  Each time one of these registers is written, the new content written is placed into the “most recently written” location and the previous content of the register is moved to “previous content” location.  For example, when a 48-bit Address feature set READ SECTOR(S) EXT command is written to the device Command register, the address used by the command is as described in Table 5.

Table 5 - 48-bit addresses 5

| Register | “most recently written” | “previous content” |
| --- | --- | --- |
| Features | Reserved | Reserved |
| Sector Count | Sector count (7:0) | Sector count (15:8) |
| LBA Low | LBA (7:0) | LBA (31:24) |
| LBA Mid | LBA (15:8) | LBA (39:32) |
| LBA High | LBA (23:16) | LBA (47:40) |
| Device register | Bits 7 and 5 are obsolete, the LBA bit shall be set to one, the DEV bit shall indicate the selected device, bits (3:0) are reserved | Reserved |

When a READ SECTOR(S) command utilizing 28-bit addressing is written to the device Command register, the address used by the command is as described in Table 6.  Thus commands utilizing 28-bit addressing still function as described in the command descriptions.

Table 6 - 28-bit addresses 6

| Register | “most recently written” | “previous content” |
| --- | --- | --- |
| Features | na | na |
| Sector Count | Sector count (7:0) | na |
| LBA Low | LBA (7:0) | na |
| LBA Mid | LBA (15:8) | na |
| LBA High | LBA (23:16) | na |
| Device register | LBA (27:24) | na |

The host may read the “previous content” of the Features, Sector Count, LBA Low, LBA Mid, and LBA High registers by first setting the High Order Bit (HOB, bit 7) of the Device Control register to one and then reading the desired register.  If HOB (bit 7) in the Device Control register is cleared to zero the host reads the “most recently written” content when the register is read.  A write to any Command Block register shall cause the device to clear the HOB bit to zero in the Device Control register.  The “most recently written” content always gets written by a register write regardless of the state of HOB (bit 7) in the Device Control register.

Registers are written and read as described in Volume 3, Clause 5.

The device shall indicate support of the 48-bit Address feature set in the IDENTIFY DEVICE response.  In addition, IDENTIFY DEVICE data words (103:100) contain the maximum user LBA + 1 that is accessible by 48-bit addressable commands .

If the value contained in IDENTIFY DEVICE data words (103:100) is equal to or less than 268,435,455, then the content of words (61:60) shall be as described in 4.2.1.  If the value in contained IDENTIFY DEVICE data words (103:100) is greater than 268,435,455, then the maximum value in words (61:60) shall be 268,435,455.  That is, if the device contains greater than the capacity addressable with 28-bit commands, words (61:60) shall describe the maximum capacity that can be addressed by 28-bit commands.

When the 48-bit Address feature set is implemented, the native maximum address is the highest address accepted by the device in the factory default condition using a 48-bit Address feature set command.  The native maximum address is the value returned by a READ NATIVE MAX ADDRESS EXT command.  If the native maximum address of a device is equal to or less than 268,435,455, a READ NATIVE MAX ADDRESS shall return the native maximum address.  If the native maximum address is greater than 268,435,455, a READ NATIVE MAX ADDRESS command shall cause the device to return a maximum value of 268,435,454.

When the 48-bit Address feature set is implemented, the SET MAX ADDRESS command shall execute as described in 7.50.2.  However, in addition to modifying the content of words (61:60), the new content of (61:60) shall also be placed in words (103:100).  When a SET MAX ADDRESS EXT command is issued and the address requested is greater than 268,435,455, words (103:100) shall be modified to reflect the requested value but words 60, and 61 shall not be modified.  When a SET MAX ADDRESS EXT command is issued and the address requested is equal to or less than 268,435,455, words (103:100) shall be modified to reflect the requested value and words 60, and 61 shall be modified as described in 7.50.2.1.

If a Host Protected Area has been created using the SET MAX ADDRESS command, all SET MAX ADDRESS EXT commands shall result in command aborted until the Host Protected Area is eliminated by use of the SET MAX ADDRESS command with the address value returned by the READ NATIVE MAX ADDRESS command.  If a Host Protected Area has been created using the SET MAX ADDRESS EXT command, all SET MAX ADDRESS commands shall result in command aborted until the Host Protected Area is eliminated by use of the SET MAX ADDRESS EXT command with the address value returned by the READ NATIVE MAX ADDRESS EXT command.

The WRITE DMA FUA EXT, WRITE DMA QUEUED FUA EXT, and WRITE MULTIPLE FUA EXT commands are unique in that regardless whether or not caching is enabled in the device, the user data shall be written to the media before ending status for the command is reported.

### Device Configuration Overlay feature set

The optional Device Configuration Overlay feature set allows a utility program to modify some of the optional commands, modes, and feature sets that a device reports as supported in the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command data as well as the capacity reported.

Commands unique to the Device Configuration Overlay feature set use a single command code and are differentiated from one another by the value placed in the Features register.  These commands are:

- DEVICE CONFIGURATION FREEZE LOCK

- DEVICE CONFIGURATION IDENTIFY

- DEVICE CONFIGURATION RESTORE

- DEVICE CONFIGURATION SET

The Device Configuration Overlay feature set may affect words (61:60), 63, (88:82), and(103:100) of the IDENTIFY DEVICE and IDENTIFY PACKET DEVICE command responses.  Certain bits in these words that indicate that a command, mode, capacity, or feature set is supported and enabled may be cleared by a DEVICE CONFIGURATION SET command.  For a particular command, mode, capacity, or feature set, when a bit is cleared indicating that the device does not support the feature, the device shall not provide the feature.  Also, the maximum capacity of the device may be reduced.  Since a Host Protected Area may be lost if the capacity of the device is reduced, when a Host Protected Area is set the DEVICE CONFIGURATION SET command shall cause the device to return command aborted.  The address value returned by a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command is modified by the DEVICE CONFIGURATION SET command modifying the maximum capacity of the device.  If a DEVICE CONFIGURATION FREEZE LOCK command has been issued since the device powered-up, the DEVICE CONFIGURATION SET command shall cause the device to return command aborted.  The settings made by a DEVICE CONFIGURATION SET command are maintained over power-down and power-up.

A DEVICE CONFIGURATION IDENTIFY command specifies the selectable commands, modes, capacity, and feature sets that the device is capable of supporting.  After the execution of a DEVICE CONFIGURATION SET command this information is no longer available from an IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command.

A DEVICE CONFIGURATION RESTORE command disables an overlay that has been set by a DEVICE CONFIGURATION SET command and returns the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command data to that indicated by the DEVICE CONFIGURATION IDENTIFY command.  Since a Host Protected Area may be lost if the capacity of the device is reduced, when a Host Protected Area is set the DEVICE CONFIGURATION RESTORE command shall cause the device to return command aborted.  If a DEVICE CONFIGURATION FREEZE LOCK command has been issued since the device powered-up, the DEVICE CONFIGURATION RESTORE command shall cause the device to return command aborted.

A DEVICE CONFIGURATION FREEZE LOCK command prevents accidental modification of the state of the Device Configuration Overlay feature set.  A device always powers-up with configuration freeze lock not set.  After a successful DEVICE CONFIGURATION FREEZE LOCK command is executed, all DEVICE CONFIGURATION SET, DEVICE CONFIGURATION IDENTIFY, and DEVICE CONFIGURATION RESTORE commands are aborted by the device until the device is powered-down and powered-up again.  The freeze locked state is not affected by hardware or software reset.

Figure 7 and the text following the figure describe the operation of the Device Configuration Overlay feature set.

Figure 7 - Device Configuration Overlay state diagram7

DCO0: Factory_config State: This state is entered when the device powers-up with the factory configuration set or a valid DEVICE CONFIGURATION RESTORE command is received.

When in this state, the device shall support all commands, modes, features sets, and the capacity indicated by the response to a DEVICE CONFIGURATION IDENTIFY command.

Transition DCO0:DCO1: When a DEVICE CONFIGURATION FREEZE LOCK command is received, the device shall return successful command completion and make a transition to the DCO1: DCO_locked state.

Transition DCO0:DCO2: When a valid DEVICE CONFIGURATION SET command is received, the device shall return successful command completion and make a transition to the DCO2: Reduced_config state.  See Transition DCO0:DCO0 for the definition of conditions that make a DEVICE CONFIGURATION SET command invalid.  This transition is made even if the configuration described by the DEVICE SET CONFIGURATION SET command is the same as the factory configuration.

Transition DCO0:DCO0: When a DEVICE CONFIGURATION RESTORE command is received, the device shall return command aborted and make a transition to the DCO0: Factory_config state.  When an invalid DEVICE CONFIGURATION SET command is received, the device shall return command aborted and make a transition to the DCO0: Factory_config state.  A DEVICE CONFIGURATION SET command is invalid if the DEVICE CONFIGURATION SET command requests:

- a Host Protected Area has been established using the SET MAX ADDRESS command.

- the elimination of support of a Multiword or Ultra DMA mode if that mode is currently selected or a higher numbered mode is currently selected.

- the elimination of support of the Host Protected Area feature set if a Host Protected Area has been established using a SET MAX ADDRESS command.

- the elimination of support of the Power-up in Standby feature set if the feature set has been enables by a jumper.

- the elimination of support of the Security feature set if the feature set has been enabled.

- the elimination of support of the SMART feature set if bits (2:1) of word 7  are not cleared to zero or if the SMART feature set has been enabled by use of the SMART ENABLE OPERATIONS command.

DCO1: DCO_locked State: This state is entered when a DEVICE CONFIGURATION RESTORE command is received.

When in this state, all DEVICE CONFIGURATION FREEZE LOCK, DEVICE CONFIGURATION IDENTIFY, DEVICE CONFIGURATION SET, or DEVICE CONFIGURATION RESTORE commands shall return command abort and shall remain in the locked state.

Transition DCO1:DCO1: When a DEVICE CONFIGURATION FREEZE LOCK, DEVICE CONFIGURATION IDENTIFY, DEVICE CONFIGURATION SET, or DEVICE CONFIGURATION RESTORE command is received, the device shall return command aborted and make a transition to the DCO1: DCO_locked state.

DCO2: Reduced_config State: This state is entered when the device powers-up with a reduced configuration set or a valid DEVICE CONFIGURATION SET command is received.

When in this state, the device shall support all commands, modes, features sets, and the capacity specified by the DEVICE CONFIGURATION SET command that caused this state to be entered.

Transition DCO2:DCO1: When a DEVICE CONFIGURATION FREEZE LOCK command is received, the device shall return successful command completion and make a transition to the DCO1: DCO_locked state.

Transition DCO2:DCO0: When a valid DEVICE CONFIGURATION RESTORE command is received, the device shall return successful command completion and make a transition to the DCO0: Factory_config state.  See Transition DCO2:DCO2 for the definition of conditions that make a DEVICE CONFIGURATION RESTORE command invalid.

Transition DCO2:DCO2: When a DEVICE CONFIGURATION SET command is received, the device shall return command aborted and make a transition to the DCO2: Reduced_config state.  When an invalid DEVICE CONFIGURATION RESTORE command is received, the device shall return command aborted and make a transition to the DCO2: Reduced_config state.  A DEVICE CONFIGURATION RESTORE command is invalid if a Host Protected Area has been established using the SET MAX ADDRESS command.

### Media Card Pass Through Command feature set

The Media Card Pass Through commands are implemented by a Media Pass Through device.  A device implementing the Media Card Pass Through Command feature set is a bridge to one or more types of media card storage devices.  The bridge device responds to the same command set as described in Error: Reference source not found and to the commands included in this feature set.

Use of the Media Card Pass Through Command feature set is prohibited for PACKET devices.

The Media Card Pass Through Command feature set uses the command codes D1h, D2h, D3h, and D4h and bits in word 84 and word 87 of the IDENTIFY DEVICE response.  The command codes D2h through D4h are reserved for the Media Card Pass Through Command feature set if this feature set is enabled by the CHECK MEDIA CARD TYPE command (D1h).  This feature set embeds small-format flash memory card commands inside the ATA commands.  The adapter’s firmware passes the embedded memory card’s command to the memory card as is from the ATA command.  The Media Card Pass Through Command feature set reduces the number of commands required for this feature set regardless of the number or type of memory card commands.  It also reduces the adapter’s firmware overhead in processing them.  As new memory cards types are defined in the market, they can all be supported within this one feature.

The commands unique to the Media Card Pass Through Command feature set are:

- CHECK MEDIA CARD TYPE

- Command codes D2h through D4h

The CHECK MEDIA CARD TYPE command returns the supporting status of the device to this feature set.  It also enables and disables the device from running the Media Card Pass Through Command feature set.  When the Media Card Pass Through Command feature set is disabled, the command codes D2h through D4h shall not be interpreted as Media Card Pass Through Command feature set commands and the device shall return command aborted.  Power-on, hardware, or software reset shall disable the Media Card Pass Through Command feature set.

The definitions of the commands D2h-D4h are media card type dependent.  Table 7 lists the Media card types and their associated reference document:

Table 7 - Media Card type references 7

| Media Card Type | Reference Document |
| --- | --- |
| SD Card | SD Card ATA Command Extension (SDA 3C) |
| Smart Media | Smart Media ATA Command Extension (SSFDC Forum) |

### Streaming feature set

The Streaming feature set is an optional feature set that allows a host to request delivery of data from a contiguous logical block address range within an allotted time.  This places a priority on time to access the data rather than the integrity of the data.  Streaming feature set commands only support 48-bit addressing.

A device that implements the Streaming feature set shall implement the following minimum set of commands:

- CONFIGURE STREAM

- READ STREAM EXT

- WRITE STREAM EXT

- READ STREAM DMA EXT

- WRITE STREAM DMA EXT

- READ LOG EXT

- WRITE LOG EXT

Support of the Streaming feature set is  indicated in IDENTIFY DEVICE data word 84 bit 4.

NOTE  PIO versions of these commands limit the transfer rate (16.6 MB/s), provide no CRC protection, and limit status reporting as compared to a DMA implementation.

#### Streaming commands

The streaming commands are defined to be time critical data transfers rather than the standard data integrity critical commands.  Each command shall be completed within the time specified in the Configure Stream command or in the streaming command itself in order to ensure the stream requirements of the AV type application.  The device may execute background tasks as long as the READ STREAM and WRITE STREAM command execution time limits are still met.

Using the Configure Stream command, the host may define the various stream properties including the default Command Completion Time Limit (CCTL) to assist the device in setting up its caching for best performance.  If the host does not use a Configure Stream command, the device shall use the CCTL specified in each streaming command, and the time limit is effective for one time only.  If the CCTL is not set by a CONFIGURE STREAM command, the operation of a streaming command with a zero CCTL is device vendor specific.  If Stream ID is not set by a CONFIGURE STREAM command, the device shall operate according to the Stream ID set by the streaming command.  The operation is device vendor specific.

The streaming commands may access any user LBA on a device.  These commands may be interspersed with non-streaming commands, but there may be an impact on performance due to the unknown time required to complete the non-streaming commands.

The streaming commands should be issued using a specified minimum number of sectors transferred per command, as specified in word 95 of the IDENTIFY DEVICE response.  The transfer length of a request should be a multiple of the minimum number of sectors per transfer.

The host provided numeric stream identifier, Stream ID, may be used by the device to configure its resources to support the streaming requirements of the AV content.  One Stream ID may be configured for each read and write operation with different command completion time limits by each CONFIGURE STREAM command.

##### Urgent bit

The Urgent bit in the Read Stream and Write Stream commands specifies that the command should be completed in the minimum possible time by the device and shall be completed within the specified Command Completion Time Limit.

##### Flush to Disk bit

The Flush to Disk bit in the WRITE STREAM command specifies that all data for the specified stream shall be flushed to the media before posting command completion.  If a host requests flushes at times other than the end of each Allocation Unit, streaming performance may be degraded.  The SET FEATURES command to enable/disable caching shall not affect caching for streaming commands.

##### Not Sequential bit

The Not Sequential bit specifies that the next read stream command with the same Stream ID may not be sequential in LBA space.  This information helps the device with pre-fetching decisions.

##### Read Continuous bit

If the Read Continuous bit is set to one for the command, the device shall transfer the requested amount of data to the host within the Command Completion Time Limit even if an error occurs.  The data sent to the host by the device in an error condition is vendor specific.

##### Write Continuous bit

If the Write Continuous bit is set to one for the command, and an error is encountered, the device shall complete the request without posting an error.  If an error cannot be resolved within the Command Completion Time Limit, the erroneous section on the media may be unchanged or may contain undefined data.  A future read of this area may not report an error, even though the data is erroneous.

##### Handle Streaming Error bit

The Handle Streaming Error bit specifies to the device that this command starts at the LBA of a recently reported error section, so the device may attempt to continue its corresponding error recovery sequence where it left off earlier.  This mechanism allows the host to schedule error recovery and defect management for content critical data.

#### Streaming Logs

The Streaming Data Transfer feature set requires two error logs and one performance log.  These logs are accessed via the READ LOG EXT command.  The information included in the error logs is volatile and is not maintained across power cycles, hard resets, or sleep.  These error logs are 512 bytes in length and retain the last 31 errors that occurred during any Streaming Data transfer.

The Streaming Performance log provides specific drive performance characteristics to the host that allows for calculating of streaming performance values.  The contents of the Streaming Performance Parameters Log may be affected by the host issuing a SET FEATURES subcommand 42h, C2h, or 43h (Automatic Acoustic Management, and Typical Host Interface Sector Time).  The host should base its calculations on the larger of its Typical Host Interface Sector Time and the device reported Sector Time values, and on the sum of the device reported Access Time values and any additional latency that only the host is aware of (host command overhead, etc).

### General Purpose Logging feature set

The General Purpose Logging feature set provides a mechanism for accessing logs in a device.  These logs are associated with specific feature sets such as SMART.  Support of the individual logs is determined by support of the associated feature set.  If the device supports a particular feature set, support for any associated log(s) is mandatory.

Support for the General Purpose Logging feature set shall not be disabled.  If the feature set associated with a requested log is disabled, the device shall return command abort.

If the General Purpose Logging feature set is implemented, the following commands shall be supported:

-- READ LOG EXT

-- WRITE LOG EXT

### Overlapped feature set

Serial implementations of ATA have different requirements (See Clause [Editors Note: Need to make a new reference]).

The optional Overlap feature set allows devices that require extended command time to perform a bus release so that the other device on the bus may be used.  To perform a bus release the device shall clear both DRQ and BSY to zero.  When selecting the other device during overlapped operations, the host shall disable assertion of INTRQ via the nIEN bit on the currently selected device before writing the Device register to select the other device and then may re-enable interrupts.

The only commands that may be overlapped are:

- NOP (with a subcommand code other than 00h)

- PACKET

- READ DMA QUEUED

- READ DMA QUEUED EXT

- SERVICE

- WRITE DMA QUEUED

- WRITE DMA QUEUED EXT

- WRITE DMA QUEUED FUA EXT

For the PACKET command, overlap is specified by the OVL bit in the Features register when the PACKET command is issued.

If the device supports PACKET command overlap, the OVL bit is set to one in the Features register and the Release interrupt has been enabled via the SET FEATURES command, then the device shall perform a bus release when the command packet has been received.  This allows the host to select the other device to execute commands.  When the device is ready to continue the command, the device sets SERV to one, and asserts INTRQ if selected and nIEN is cleared to zero.  The host then issues the SERVICE command to continue the execution of the command

If the device supports PACKET command overlap, the OVL bit is set to one in the Features register and the Release interrupt has been disabled via the SET FEATURES command, then the device may or may not perform a bus release.  If the device is ready to complete execution of the command, the device may complete the command immediately as described in the non-overlap case.  If the device is not ready to complete execution of the command, the device may perform a bus release and complete the command as described in the previous paragraph.

For the READ DMA QUEUED and WRITE DMA QUEUED commands, the device may or may not perform a bus release.  If the device is ready to complete execution of the command, the device may complete the command immediately.  If the device is not ready to complete execution of the command, the device may perform a bus release and complete the command via a service request.

If a device has an outstanding command that has been released, the device can only indicate that service is required when the device is selected.  This implies that the host has to poll each device to determine if a device is requesting service.  The polling can be performed at the host either by hardware or by a software routine.  The latter implies a considerable host processor overhead.  Hardware polling is initiated by the NOP Auto Poll command.

The NOP Poll command is a host adapter function and is ignored by the device.  The host software can test for the support of this feature by issuing the NOP Auto Poll subcommand and examining the Status register.  If the host adapter does not support this feature, the response received by the host will be from the device with the ERR bit set to one.  If the host adapter does support the command, the response will be from the host adapter with the ERR bit cleared to zero.  The only action taken by a device supporting the Overlapped feature set will be to return the error indication in the Status register and to not abort any outstanding commands.

When this command is received, the user data shall be written to the device media before ending status for the command is reported regardless of the state of any write cache or queue.  A queue shall not be aborted.

### Queued feature set

The Queued feature set allows the host to issue concurrent commands to the same device.  The Queued feature set is optional if the Overlap feature set is supported.  Only commands included in the Overlapped feature set may be queued.  The queue contains all commands for which command acceptance has occurred but command completion has not occurred.  If a queue exists when a non-queued command is received, the non-queued command shall be command aborted and the commands in the queue shall be discarded.  The ending status shall be command aborted and the results are indeterminate.

The maximum queue depth supported by a device shall be indicated in word 75 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

A queued command shall have a Tag provided by the host in the Sector Count register to uniquely identify the command.  When the device restores register parameters during the execution of the SERVICE command, this Tag shall be restored so that the host may identify the command for which status is being presented.  A Tag value may be any value between 0 and 31, regardless of the queue depth supported.  If a queued command is issued with a Tag value that is identical to the Tag value for a command already in the queue, the entire queue shall be aborted including the new command.  The ending status shall be command aborted and the results are indeterminate.  If any error occurs, the command queue shall be aborted.

When the device is ready to continue the processing of a bus released command and BSY and DRQ are both cleared to zero, the device requests service by setting SERV to one, setting a pending interrupt, and setting Interrupt Pending if selected and if nIEN is cleared to zero.  SERV shall remain set until all commands ready for service have been serviced.  A read of the Status register or a write of the Command register shall clear the Interrupt Pending.

When the device is ready to continue the processing of a bus released command and BSY or DRQ is set to one (i.e., the device is processing another command on the bus), the device requests service by setting SERV to one.  SERV shall remain set until all commands ready for service have been serviced.  At command completion of the current command processing (i.e., when both BSY and DRQ are cleared to zero), the device shall process Interrupt Pending and INTRQ per the protocol for the command being completed.  No additional INTRQ assertion shall occur due to other commands ready for service until after the device’s SERV bit has been cleared to zero.

When the device receives a new command while queued commands are ready for service, the device shall execute the new command and process Interrupt Pending and INTRQ per the protocol for the new command.  If the queued commands ready for service still exist at command completion of this command, SERV remains set to one but no additional INTRQ assertion shall occur due to commands ready for service.

When queuing commands, the host shall disable INTRQ assertion via the nIEN bit before writing a new command to the Command register and may re-enable INTRQ assertion after writing the command.  When reading status at command completion of a command, the host shall check the SERV bit since the SERV bit may be set because the device is ready for service associated with another command.  The host receives no additional INTRQ assertion to indicate that a queued command is ready for service.

### Long Physical Sector Feature Set  for Non-Packet Devices

The purpose of the long physical sector feature set is to allow increased media format efficiency.  During write operations devices calculate and error correction code, ECC, and write the ECC on the media following the data.  ECC encoding is more efficient when used over a larger amount of data.

The long physical sector feature set allows a device to be formatted so that there are multiple logical sectors per physical sector on the media.  Each physical sector has an ECC field.  This allows, for example, a device to have 2048 word physical sectors each containing 8 logical sectors, or one ECC field per 8 256 word logical sectors, See Figure 8 example 3.

A performance penalty may be incurred when writing to devices that implement long physical sector feature set.  A physical sector is read or written in a single operation.  If a host system does not write all of the logical sectors in a physical sector during a single command the device may need to read the logical sectors that are not to be changed into memory and then write the entire physical sector, see Appendix C.

LBA 0

LBA 1

LBA 2

LBA 3

LBA 4

LBA 5

LBA 6

LBA 8

512 B

512 B

512 B

512 B

512 B

512 B

512 B

512 B

LBA 0

LBA 1

LBA 2

LBA 3

LBA 4

LBA 5

524 B

524 B

524 B

524 B

524 B

524 B

. . .

. . .

LBA 0

LBA 1

LBA 2

LBA 3

LBA 4

LBA 5

LBA 6

LBA 8

512 B

512 B

512 B

512 B

512 B

512 B

512 B

512 B

. . .

Physical Sector 0

. . .

Physical Sector 1

LBA 0

LBA 1

LBA 2

LBA 3

LBA 4

LBA 5

524 B

524 B

524 B

524 B

524 B

524 B

Physical Sector 0

Physical Sector 1

LBA 6

524 B

LBA 6

524 B

. . .

. . .

1) Conventional Logical Sector Format: 512 Bytes Per LBA Address

2) Long Logical Sector Format Example: 524 Bytes Per LBA Address

3) Long Physical Sector Format Example: 512 Bytes Per LBA Address, 2048 Bytes Per Physical Sector

4) Long Logical and Long Physical Sector Format Example: 524 Bytes Per LBA Address, 2096 Bytes Per Physical Sector

Figure 8 - Long Logical and long Physical Sector Example

8

### Long Logical Sector Feature Set  for Non-Packet Devices

The purpose of the long logical sector feature set is to allow additional data words per sector for server applications.  Sectors with 520 or 528 bytes are typical.  Devices with long logical sectors set IDENTIFY DEVICE data word 106 bit 13 to 1.  The Long Logical Sector length is described by IDENTIFY DEVICE data words 117-118.

Devices that implement the Long Logical Sector Feature set are not backward compatible with applications that use 256 word logical sectors, e.g.  desktop and laptop system.

Table 8 - Long Logical Sector Function 88

| Command | Words Per Sector Transferred |
| --- | --- |
| CFA ERASE SECTORS | - |
| CFA REQUEST EXTENDED ERROR CODE | - |
| CFA TRANSLATE SECTOR | IDENTIFY DEVICE data words 117-118 |
| CFA WRITE MULTIPLE WITHOUT ERASE | IDENTIFY DEVICE data words 117-118 |
| CFA WRITE SECTORS WITHOUT ERASE | - |
| CHECK MEDIA CARD TYPE | - |
| CHECK POWER MODE | - |
| CONFIGURE STREAM | - |
| DEVICE CONFIGURATION | - |
| DEVICE RESET | - |
| DOWNLOAD MICROCODE | 256 |
| EXECUTE DEVICE DIAGNOSTIC | - |
| FLUSH CACHE | - |
| FLUSH CACHE EXT | - |
| GET MEDIA STATUS | - |
| IDENTIFY DEVICE | 256 |
| IDENTIFY PACKET DEVICE | - |
| IDLE | - |
| IDLE IMMEDIATE | - |
| MEDIA EJECT | - |
| MEDIA LOCK | - |
| MEDIA UNLOCK | - |
| NOP | - |
| PACKET | - |
| READ BUFFER | 256 |
| READ DMA | IDENTIFY DEVICE data words 117-118 |
| READ DMA EXT | IDENTIFY DEVICE data words 117-118 |
| READ DMA QUEUED | IDENTIFY DEVICE data words 117-118 |
| READ DMA QUEUED EXT | IDENTIFY DEVICE data words 117-118 |
| READ LOG EXT | 256 |
| READ MULTIPLE | IDENTIFY DEVICE data words 117-118 |
| READ MULTIPLE EXT | IDENTIFY DEVICE data words 117-118 |
| READ NATIVE MAX ADDRESS | - |
| READ NATIVE MAX ADDRESS EXT | - |
| READ SECTOR(S) | IDENTIFY DEVICE data words 117-118 |
| READ SECTOR(S) EXT | IDENTIFY DEVICE data words 117-118 |
| READ STREAM DMA EXT | IDENTIFY DEVICE data words 117-118 |
| READ STREAM EXT | IDENTIFY DEVICE data words 117-118 |
| READ VERIFY SECTOR(S) | IDENTIFY DEVICE data words 117-118 |
| READ VERIFY SECTOR(S) EXT | - |
| SECURITY DISABLE PASSWORD | 256 |

(continued)

Table 8 - Long Logical Sector Function (continued)88

| Command | Words Per Sector Transferred |
| --- | --- |
| SECURITY ERASE PREPARE | - |
| SECURITY ERASE UNIT | 256 |
| SECURITY FREEZE LOCK | - |
| SECURITY SET PASSWORD | 256 |
| SECURITY UNLOCK | 256 |
| SEEK | - |
| SERVICE | - |
| SET FEATURES | - |
| SET MAX SET PASSWORD | 256 |
| SET MAX LOCK | - |
| SET MAX FREEZE LOCK | - |
| SET MAX UNLOCK | 256 |
| SET MAX ADDRESS | - |
| SET MAX ADDRESS EXT | - |
| SET MULTIPLE MODE | - |
| SLEEP | - |
| SMART DISABLE OPERATIONS | - |
| SMART ENABLE/DISABLE AUTOSAVE | - |
| SMART ENABLE OPERATIONS | - |
| SMART EXECUTE OFF-LINE IMMEDIATE | - |
| SMART READ DATA | 256 |
| SMART READ LOG | 256 |
| SMART RETURN STATUS | - |
| SMART WRITE LOG | 256 |
| STANDBY | - |
| STANDBY IMMEDIATE | - |
| WRITE BUFFER | 256 |
| WRITE DMA | IDENTIFY DEVICE data words 117-118 |
| WRITE DMA EXT | IDENTIFY DEVICE data word 117 |
| WRITE DMA QUEUED | IDENTIFY DEVICE data word 117 |
| WRITE DMA QUEUED EXT | IDENTIFY DEVICE data word 117 |
| WRITE LOG EXT | 256 |
| WRITE MULTIPLE | IDENTIFY DEVICE data word 117 |
| WRITE MULTIPLE EXT | IDENTIFY DEVICE data word 117 |
| WRITE SECTOR(S) | IDENTIFY DEVICE data word 117 |
| WRITE SECTOR(S) EXT | IDENTIFY DEVICE data word 117 |
| WRITE STREAM DMA EXT | IDENTIFY DEVICE data word 117 |
| WRITE STREAM EXT | IDENTIFY DEVICE data word 117 |

(concluded)

Table 8 describes the command behavior of drives that have been manufactured with long logical sectors.  Data transfer commands transfer either the long logical sector length or 256 words depending on the command.  For example, Read and Write Extended commands transfer data in long logical sectors while READ LOG EXT and WRITE LOG EXT commands transfer 256 words per sector, regardless of the logical sector length.  Figure 8 example 2 shows a diagram of a device formatted with long logical sectors.

### Devices Implementing the Long Physical Sector Feature Set and the Long Logical Feature Sector Set

The long physical sector feature set and the long logical sector feature set are not exclusive.  Figure 8 example 4 illustrates a device implementing both the Long Physical Sector and Long Logical Sector feature sets.

### Write-Read-Verify Feature Set

The optional Write Read Verify feature set allows a host to control Read After Write behavior in a device.

This feature set is available for all devices.

The device may report whether the feature set is supported and/or enabled via the IDENTIFY DEVICE command.

To enable or disable the feature of Write/Read/Verify, the host may execute a SET FEATURES command with one of two subcommand codes.

This feature set shall not be controllable via the DEVICE CONFIGURATION OVERLAY (DCO) command.

It is possible that the device may experience a performance degradation when the WriteReadVerify feature is enabled.

These commands are affected by this feature:

WRITE DMA

WRITE DMA EXT

WRITE DMA FUA EXT

WRITE DMA QUEUED

WRITE DMA QUEUED EXT

WRITE DMA QUEUED FUA EXT

WRITE MULTIPLE

WRITE MULITIPLE EXT

WRITE MULTIPLE FUA EXT

WRITE SECTOR(S)

WRITE SECTOR(S) EXT

See clause Error: Reference source not found for a description of device behavior when this feature set is supported and enabled.

## ATA Protocols

### Overview

ATA Protocols are fully described in the transport documents ATA8-SATA and ATA8-PATA.  The protocols listed here shall be implemented by all transports that use ATA8-ACS commands.

### Power-On

[Editors Note: Provide Description]

### Hardware Reset

[Editors Note: Provide Description]

### Software Reset

[Editors Note: Provide Description]

### Non-data Command Protocol

[Editors Note: Provide Description]

### PIO data-in Command Protocol

[Editors Note: Provide Description]

### PIO data-out Command Protocol

[Editors Note: Provide Description]

### DMA Command Protocol

[Editors Note: Provide Description]

### Packet Command Protocol

[Editors Note: Provide Description]

### DMA Queued Command Protocol

[Editors Note: Provide Description]

### Execute Device Diagnostic Command Protocol

[Editors Note: Provide Description]

### Device Reset Command Protocol

[Editors Note: Provide Description]

### Ultra DMA data-in Command Protocol

[Editors Note: Provide Description]

### Ultra DMA Data-Out Command Protocol

[Editors Note: Provide Description]

## Status and Error bits

### Overview

There are several bits that are provided by both the Parallel and Serial transports.  Blah, blah, blah

### Status Bits

      - Busy (BSY)

Busy shall be cleared to zero.  This bit is transport dependent.  Refer to the appropriate transport standard for a definition of this bit.

      - Check Condition (CHK)

Check Condition shall be set to one if an Error register sense key or code bit is set.  T

      - Error (ERR)

The Error bit in the status field shall be set to one if any bit in the Error field (see 6.3) is set to one

      - Device Fault (DF)

Device Fault shall be set to one if a device fault has occurred.  [Editors Note: Describe more about DF behavior]

      - Device Ready (DRDY)

Device Ready shall be set to one.  This bit is transport dependent.  Refer to the appropriate transport standard for a definition of this bit.

      - Data Request (DRQ)

Device Request shall be cleared to zero.  This bit is transport dependent.  Refer to the appropriate transport standard for a definition of this bit.

      - Deferred Write Error (DWE)

DWE shall be set to one if an error was detected in a deferred write to the media for a previous WRITE STREAM DMA EXT or WRITE STREAM EXT command.  This error is from a previously issued command.  If DWE is set to one, the location of the deferred error is only reported in the Write Stream error log.

      - DMA Ready (DMRD)

[Editors Note: Need Definition]

      - Service (SERV)

shall be cleared to zero when no other queued command is ready for service.  SERV shall be set to one when another queued command is ready for service.  SERV shall be set to one when the device has prepared this command for service.  If overlap is not supported, this bit is command specific.

      - Stream Error (SE)

Stream Error shall be set to one if an error has occurred during the execution of the command and the Read Continuous (See clause 7.38.3 for a definition of Read Continuous) bit is set to one.  In this case the LBA returned in the Sector Number registers shall be the address of the first sector in error, and the Sector Count registers shall contain the number of consecutive sectors that may contain errors.  If the Read Continuous bit is set to one when the command is issued and an ICRC, UNC, IDNF, ABRT, or CCTO (See clause 6.3 for a definition of these bits) error occurs, the SE bit shall be set to one, the ERR bit shall be cleared to zero, and the bits that would normally be set in the Error register shall be set in the error log.

      - Transport Dependent (TD)

ATA/ATAPI-7 defines the status bits BSY,DRDY, and DRQ.  This bits are fully documented in the transport standards.  Although all of the commands in this standard use BSY=0, DRDY=1 and DRQ=0 to specify that the device is ready to accept a command and to specify that a command is complet, they are processed differently in the various transport standard.

### Error Bits

      - Abort (ABRT)

Abort shall be set to one if the command is not supported.  Abort may be set to one if the device is not able to complete the action requested by the command.  Abort shall also be set to one if an address outside of the range of user-accessible addresses is requested if IDNF is not set to one.

      - Command Completion Time Out (CCTO)

CCTO shall be set to one if a Command Completion Time Limit Out error has occurred.

      - End of Media? (EOM)

The operation of this bit is specific to the SCSI command set implemented by the packet device.

      - ID Not Found (IDNF)

ID Not Found shall be set to one if a user-accessible address could not be found.  ID Not Found shall be set to one if an address outside of the range of user-accessible addresses is requested when command aborted is not returned.

      - Illegal Legenth Indicator (ILI)

The operation of this bit is specific to the SCSI command set implemented by the packet device.

      - Interface CRC (ICRC)

Interface CRC shall be set to one if an interface CRC error has occurred during an Ultra DMA data transfer.  The content of this bit is not applicable for Multiword DMA transfers.

      - Media Change (MC)

Media Change shall be set to one if the media in a removable media device changed since the issuance of the last command.  The device shall clear the device internal media change detected state.

      - Media Change Request (MCR)

Media Change Request shall be set to one if a media change request has been detected by a removable media device.  This bit is only cleared by a GET MEDIA STATUS or a media access command.

      - Media Error (MED)

Media Error shall be set to one if a media error is detected.

      - No Media (NM)

No Media shall be set to one if no media is present in the device.  This bit shall be set to one for each execution of GET MEDIA STATUS until media is inserted into the device.

      - Sense Key

The operation of this four bit field is specific to the SCSI command set implemented by the packet device.

      - Uncorrectable Error (UNC)

Uncorrectable Error shall be set to one if data is uncorrectable.

      - Write Protect (WP)

Write Protect shall be set to one for each execution of GET MEDIA STATUS while the media is write protected.

### Interrupt Reason Bits

      - Command/Data (C/D)

Shall be set to one if the transfer is a command packet.  Shall be set to zero if the transfer is data.

      - Input/Output (I/O)

Shall be cleared to zero if the transfer is to the device (O).  Shall be set to one if the transfer is to the host (I).

      - Release (REL)

Shall be set to one if a command has been accepted but not completed and the device is ready to accept another command.

      - Tag

If the device supports command queuing and overlap is enabled, this field contains the command Tag for the command.  A Tag value may be any value between 0 and 31 regardless of the queue depth supported.  If the device does not support command queuing or overlap is disabled, this field is N/A.

## Command Descriptions

### Overview

[Editors Note: Need to state that this word ordering is mapped differently to different transports]

      - Introduction

In ATA/ATAPI-7, commands were described by indicating how Features, Sector Count, LBA Low, LBA Mid, LBA High, Device, and Command registers are initialized.  The introduction of ATA8-ST has changed the way that ATA8-ACS needs to describe commands.  In ATA8-ST, commands are initiated by setting up fields in a FIS and then sending the FIS to a device that implements ATA8-ST.  Therefore, it is inconsistent to describe commands through register initialization sequences.  ATA8-ST also makes the status bits Busy (BSY), Device Ready (DRDY), and Device Request (DRQ) transport specific.

The basic structure of an ATA8 command has Feature, Count, LBA, and Command fields.  These fields vary in length based on the command being 28-bit addressing or 48-bit addressing.  Since the sector address is now contained in a single field, the Device register described in ATA/ATAPI-7 is mainly a transport specific register.  There is currently only one command that uses the device register for more than just bits 27:24 of an address.  The Check Media Card Type command  returns the media card type in the low 4 bits of the register.

ATA8-ACS describes the ATA command set in a transport independent fashion.  Each command is defined by the sections described in clauses x through y.

      - Command Name - Command Code [/featurecode], Command Protocol

The heading for each command starts with the name of the command.  The name is follwed by “-“ and then the command code and protocol used to execute the command.  A sample heading reads:

READ SECTOR(S) - 20h, PIO data-in

The name of the command is “READ SECTOR(S)”.  The command code is 20h.  The protocol used to transfer the data is PIO data-out.

The protocols are defined in detail in each transport.  There is an overview of each protocol in clause 5

      - Description

Each command starts with a description.  This description contains information regarding the feature set, if the command is optional or mandatory, and any prerequisites that may be required before the command is executed.

      - Inputs

        - Inputs for 28-bit Read/Write Commands

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | In ATA/ATAPI-7 this was the Feature register. Each transport standard shows how the Feature field is mapped for proper functionality. The transport documents also show how 28-bit commands are mapped differently from 48-bit commands. |  |  |
| 01h | Count | Each transport standard shows how the Feature field is mapped for proper functionality. The transport documents also show how 28-bit commands are mapped differently from 48-bit commands. |  |  |
| 02h | LBA | MSB | Address of first sector to be erased. Bits 47:28 shall be cleared to zero. Each transport shows how these 48-bits are mapped to the appropriate fields or registers. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | The command number goes here. |  |  |

The Feature, Count, and Command fields are 8 bits; the LBA field is 28 bits.  In each 28-bit command description, the high order 8 bits of the Feature and Count fields shall be reserved.  The command register shall always be represented as an 8-bit value.

        - Inputs for 48-bit Read/Write Commands

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | In ATA/ATAPI-7 this was the Feature register. Each transport standard shows how the Feature field is mapped for proper functionality. The transport documents also shows how 28-bit commands are mapped differently from 48-bit commands. |  |  |
| 01h | Count | Each transport standard shows how the Feature field is mapped for proper functionality. The transport documents also show how 28-bit commands are mapped differently from 48-bit commands. |  |  |
| 02h | LBA | MSB | Each transport shows how these 48-bits are mapped to the appropriate fields or registers. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | The command number goes here. |  |  |

The Feature, and Count, fields shall be 16 bit values; the LBA field shall be a 48-bit value.  The command register shall always be represented as an 8-bit value.

        - Inputs for Non Data Commands

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | In ATA/ATAPI-7 this was the Feature register. Each transport standard shows how the Feature field is mapped for proper functionality. The transport documents also shows how 28-bit commands are mapped differently from 48-bit commands. |
| 01h | Count | Each transport standard shows how the Feature field is mapped for proper functionality. The transport documents also show how 28-bit commands are mapped differently from 48-bit commands. |
| 02h-04h | LBA | Each transport shows how these 48-bits are mapped to the appropriate fields or registers. |
| 05h | Command | The command number goes here. |

      - Normal outputs

Many of the commands have a Normal Outputs description that lookes exactly like the one shown above.  A command with normal outputs has no error, this field is reserved in every command.  Count and LBA may be reserved.  In some commands these fields have return parameters on successful command completion.  The status register shows the Device Fault bit and the Error bit.  The See clause link is active.  This allows the reader to click on the link and see the description of the bit.  Bits 7,6, and 3 are marked Transport Dependent in all the Normal Outputs.  These bits correspond to Busy, Device Ready, and Device Request and are transport specific in their operation.  All commands set Device Ready to one and clear Busy and Device Request to zero.

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2 | N/A |  |
|  |  |  | 1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

      - Error Outputs

Error Outputs shows the Error, Count, LBA and Status fields.  An Error Output occurs when a bit in the Status field indicates that an error has occurred.  Examples of status bits that indicate and error has occurred include Error, Device Fault, Stream Error, etc.  If the Error bit is set to one, the Error field shall indicate the type of Error that occurred.

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |
|  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | Obsolete |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

### CFA ERASE SECTORS - C0h, non-data

      - Feature Set

This command is mandatory for devices implementing the CFA feature set.  This command code is Vendor Specific for devices not implementing the CFA feature Set.

      - Description

This command pre-erases and conditions from 1 to 256 sectors as specified in the Sector Count register.  This command should be issued in advance of a CFA WRITE SECTORS WITHOUT ERASE or a CFA WRITE MULTIPLE WITHOUT ERASE command to increase the execution speed of the write operation.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | number of sectors to be erased. A value of 00h specifies that 256 sectors are to be erased. |  |  |
| 02h | LBA | MSB | Address of first sector to be erased. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 24h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 77

### CFA REQUEST EXTENDED ERROR CODE - 03h, non-data

      - Feature Set

This command is mandatory for devices implementing the CFA featureset.

      - Description

This command provides an extended error code which identifies the cause of an error condition in more detail than is available with Status and Error register values.  The CFA REQUEST EXTENDED ERROR CODE command shall return an extended error code if the previous command completed with an error or a no error detected extended error code if the previous command completed without error.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | 03h |

      - Normal outputs

See Table 64

Table 9 - Extended error codes9

| Extended error code | Description |
| --- | --- |
| 00h | No error detected / no additional information |
| 01h | Self-test passed |
| 03h | Write / Erase failed |
| 05h | Self-test or diagnostic failed |
| 09h | Miscellaneous error |
| 0Bh | Vendor specific |
| 0Ch | Corrupted media format |
| 0D-0Fh | Vendor specific |
| 10h | ID Not Found / ID Error |
| 11h | Uncorrectable ECC error |
| 14h | ID Not Found |
| 18h | Corrected ECC error |
| 1Dh, 1Eh | Vendor specific |
| 1Fh | Data transfer error / command aborted |
| 20h | Invalid command |
| 21h | Invalid address |
| 22-23h | Vendor specific |
| 27h | Write protect violation |
| 2Fh | Address overflow (address too large) |
| 30-34h | Self-test or diagnostic failed |
| 35h, 36h | Supply or generated voltage out of tolerance |
| 37h, 3Eh | Self-test or diagnostic failed |
| 38h | Corrupted media format |
| 39h | Vendor specific |
| 3Ah | Spare sectors exhausted |
| 3Bh 3Ch, 3Fh | Corrupted media format |
| 3Dh | Vendor specific |
| All other values | Reserved |

      - Error outputs

See Table 78

### CFA TRANSLATE SECTOR - 87h, PIO data-in

      - Feature Set

This command is mandatory for devices implementing the CFA feature set.  This command code is Vendor Specific for devices not implementing the CFA feature Set.

      - Description

This command provides information related to a specific sector.  The data indicates the erased or not erased status of the sector, and the number of erase and write cycles performed on that sector.  Devices may return zero in fields that do not apply or that are not supported by the device.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | Number of sectors to be erased. A value of 00h specifies that 256 sectors are to be erased. |  |  |
| 02h | LBA | MSB | Sector Address. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 87h |  |  |

      - Normal outputs

See Table Table 66 for the data structure layout.  A 512 byte information table is transferred to the host, see Table 10.

Table 10 - CFA TRANSLATE SECTOR Information10

| Byte | Description |
| --- | --- |
| 00h-03h | Obsolete |
| 04h | LBA bits (23:16) |
| 05h | LBA bits (15:8) |
| 06h | LBA bits (7:0) |
| 07h-12h | Reserved |
| 13h | Sector erased flag (FFh = erased; 00h = not erased) |
| 14h-17h | Reserved |
| 18h | Sector write cycles count bits (23:16) |
| 19h | Sector write cycles count bits (15:8) |
| 1Ah | Sector write cycles count bits (7:0) |
| 1Bh-FFh | Reserved |

      - Error outputs

See Table 78

### CFA WRITE MULTIPLE WITHOUT ERASE - CDh, PIO data-out

      - Feature Set

This command is mandatory for devices implementing the CFA feature set.

      - Description

This command is similar to the WRITE MULTIPLE command.  Interrupts are not generated on every sector, but on the transfer of a block that contains the number of sectors defined by the SET MULTIPLE MODE.

Command execution is identical to the WRITE MULTIPLE operation except that the sectors are written without an implied erase operation.  The sectors should be pre-erased by a preceding CFA ERASE SECTORS command.

if bit 8 of IDENTIFY DEVICE data word 59 is cleared to zero, a successful SET MULTIPLE MODE command shall precede a CFA WRITE MULTIPLE WITHOUT ERASE command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | N/A |  |  |
| 01h | Count | Number of sectors to be transferred. A value of 00h specifies that 256 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Starting sector address to be written. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | CDh |  |  |

      - Normal outputs

See Table 66.

      - Error outputs

The device shall return command aborted if the command is not supported.  An unrecoverable error encountered during execution of this command results in the termination of the command.  The command block registers contain the address of the sector where the first unrecovered error occurred.  The amount of data transferred is indeterminate.  See Table 77.

### CFA WRITE SECTORS WITHOUT ERASE - 38h, PIO data-out

      - Feature Set

This command is mandatory for device implementing the CFA feature set.

      - Description

This command is similar to the WRITE SECTORS command.  Command execution is identical to the WRITE SECTORS operation except that the sectors are written without an implied erase operation.  The sectors should be pre-erased by a preceding CFA ERASE SECTORS command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | N/A |  |  |
| 01h | Count | Number of sectors to be transferred. A value of 00h specifies that 256 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Starting sector address to be written. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 38h |  |  |

      - Normal outputs

See Table 66.

      - Error outputs

The device shall return command aborted if the command is not supported.  An unrecoverable error encountered during execution of this command results in the termination of the command.  The command block registers contain the address of the sector where the first unrecovered error occurred.  The amount of data transferred is indeterminate.  See Table 77.

### CHECK MEDIA CARD TYPE - D1h, Non-data

      - Feature Set

This command is mandatory for devices implementing the Media Card Pass Through Command feature set.

      - Description

The CHECK MEDIA CARD TYPE command allows the host to determine if the device supports the Media Card Pass Through Command feature set.  If theEnable bit in the Features register is set to one, IDENTIFY DEVICE data bit 3 word 87 shall be set to one upon successful command completion.

If the adapter supports the Media Card Pass Through Command feature set and the Enable bit of the Features register is set to one, the adapter shall process any further Media Card Pass Through Command feature set commands.  If the Enable bit is cleared to zero, the adapter shall not interpret the command codes D2 through D4 as the Media Card Pass Through Command feature set commands.  If the adapter does not support the Media Card Pass Through Command feature set, or the host has disabled the Media Card Pass Through Command feature set mode by clearing the Enable bit to zero, the host shall not send any further Media Card Pass Through Command feature set commands to the adapter.

      - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:1 | Reserved |  |
|  |  |  | 0 | Enable - Shall be set to one to enable the Media Card Pass Through Command feature set. Enable shall be cleared to zero to disable the Media Card Pass Through Command feature set. |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA | N/A |  |  |  |
| 05h | Command | D1h |  |  |  |
| Note - Power-on, hardware, or software reset disables the Media Card Pass Through Command feature set |  |  |  |  |  |

      - Normal Outputs

See Table 67

      - Error Outputs

See Table 79

### CHECK POWER MODE - E5h, Non-data

      - Feature Set

This command is mandatory for devices implementing the General feature set.  This command is mandatory for devices implementing the PACKET Command feature set when power management is not implemented in the PACKET command set.

      - Description

The CHECK POWER MODE command allows the host to determine the current power mode of the device.  The CHECK POWER MODE command shall not cause the device to change power or affect the operation of the Standby timer.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | E5h |

      - Normal outputs

See Table 68

      - Error outputs

The device shall return command aborted if the device does not support the Power Management feature set.  See Table 79

### CONFIGURE STREAM - 51h, Non-data

      - Feature Set

This command is Mandatory for devices that implement the Streaming feature set.

      - Description

The CONFIGURE STREAM command specifies the operating parameters of an individual stream.  A CONFIGURE STREAM command may be issued for each stream that is to be added or removed from the current operating configuration.  If A/R = 1 and the specified Stream ID is already valid at the device, the new parameters shall replace the old parameters, unless Command Abort is returned (See Abort conditions for Error register).  In this case the old parameters for the specified Stream ID shall remain in effect.

      - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15 | A/R - Specifies a request to add a new stream if set to one. If cleared to zero, a request to remove a previous configured stream is specified. |  |
|  |  |  | 14 | R/W - Specifies a read stream if cleared to zero and a write stream if set to one. |  |
|  |  |  | 13:11 | Reserved |  |
|  |  |  | 10:8 | Stream ID - The Stream ID shall be a value between 0 and 7. |  |
|  |  |  | 7:0 | Default CCTL - Default Command Completion Time Limit (CCTL). This value is calculated as follows: (Default CCTL) = ((content of the Features register bits 7:0)  (IDENTIFY DEVICE data words (99:98))) microseconds. This time shall be used by the device when a streaming command with the same stream ID and a CCTL of zero is issued. The time is measured from the write of the command register to command completion. |  |
| 01h | Count | Allocation Unit (AU) size in sectors |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Command | 51h |  |  |  |

      - Normal Outputs

See Table 69

      - Error Outputs

See Table 80.  Abort shall be set to one if any of the following are true:

- The drive cannot support the requested stream configuration

- A/R = 0 and the Features Register contains an unconfigured Stream ID

- The Default CCTL cannot be supported by the device

- The device does not support the Streaming Feature Set.

### DEVICE CONFIGURATION

Individual Device Configuration Overlay feature set commands are identified by the value placed in the Features register.  Table 11 shows these Features register values.

Table 11 - Device Configuration Overlay Features register values 11

| Value | Command |
| --- | --- |
| C0h | DEVICE CONFIGURATION RESTORE |
| C1h | DEVICE CONFIGURATION FREEZE LOCK |
| C2h | DEVICE CONFIGURATION IDENTIFY |
| C3h | DEVICE CONFIGURATION SET |
| 00h-BFh, C4h-FFh | Reserved |

#### DEVICE CONFIGURATION RESTORE - B1h/C0h, Non-data

        - Feature Set

This command is mandatory for devices implementing the Device Configuration Overlay feature set.

        - Description

The DEVICE CONFIGURATION RESTORE command disables any setting previously made by a DEVICE CONFIGURATION SET command and returns the content of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command data to the original settings as indicated by the data returned from the execution of a DEVICE CONFIGURATION IDENTIFY command.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | C0h |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | B1h |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if a Host Protected Area has been set by a SET MAX ADDRESS or SET MAX ADDRESS EXT command, or if DEVICE CONFIGURATION FREEZE LOCK is set.  See Table 79

#### DEVICE CONFIGURATION FREEZE LOCK - B1h/C1h, Non-data

        - Feature Set

This command is mandatory for devices implementing the Device Configuration Overlay feature set.

        - Description

The DEVICE CONFIGURATION FREEZE LOCK command prevents accidental modification of the Device Configuration Overlay settings.  After successful execution of a DEVICE CONFIGURATION FREEZE LOCK command, all DEVICE CONFIGURATION SET, DEVICE CONFIGURATION FREEZE LOCK, DEVICE CONFIGURATION IDENTIFY, and DEVICE CONFIGURATION RESTORE commands shall be  aborted by the device.  The DEVICE CONFIGURATION FREEZE LOCK condition shall be cleared by a power-down.  The DEVICE CONFIGURATION FREEZE LOCK condition shall not be cleared by hardware or software reset.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | C1h |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | B1h |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if the device has executed a previous DEVICE CONFIGURATION FREEZE LOCK command since power-up.  See Table 79.

#### DEVICE CONFIGURATION IDENTIFY - B1h/C2h, PIO Data-in

        - Feature Set

This command is mandatory for devices implementing the Device Configuration Overlay feature set.

        - Description

The DEVICE CONFIGURATION IDENTIFY command returns a 512 byte data structure via PIO data-in transfer.  The content of this data structure indicates the selectable commands, modes, and feature sets that the device is capable of supporting.  If a DEVICE CONFIGURATION SET command has been issued reducing the capabilities, the response to an IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command will reflect the reduced set of capabilities, while the DEVICE CONFIGURATION IDENTIFY command will reflect the entire set of selectable capabilities.

The term ‘is allowed’ indicates that the device may report that a feature is supported and/or enabled.

If the device is not ‘allowed’ to report support, then the device shall not support and shall report that the selected feature is both ‘not supported’ and if appropriate ‘not enabled.’

The format of the Device Configuration Overlay data structure is shown in Table 12.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | C2h |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | B1h |

        - Normal outputs

See Table 65

        - Error outputs

See Clause 7.10.2.5

        - Input Data

Table 12 - Device Configuration Identify data structure  12

| Word | Content |  |
| --- | --- | --- |
| 0 | Data structure revision |  |
| 1 | Multiword DMA modes supported |  |
|  | 15:3 | Reserved |
|  | 2 | 1 = Reporting support for Multiword DMA mode 2 and below is allowed |
|  | 1 | 1 = Reporting support for Multiword DMA mode 1 and below is allowed |
|  | 0 | 1 = Reporting support for Multiword DMA mode 0 is allowed |
| 2 | Ultra DMA modes supported |  |
|  | 15:7 | Reserved |
|  | 6 | 1 = Reporting support for Ultra DMA mode 6 and below is allowed |
|  | 5 | 1 = Reporting support for Ultra DMA mode 5 and below is allowed |
|  | 4 | 1 = Reporting support for Ultra DMA mode 4 and below is allowed |
|  | 3 | 1 = Reporting support for Ultra DMA mode 3 and below is allowed |
|  | 2 | 1 = Reporting support for Ultra DMA mode 2 and below is allowed |
|  | 1 | 1 = Reporting support for Ultra DMA mode 1 and below is allowed |
|  | 0 | 1 = Reporting support for Ultra DMA mode 0 is allowed |
| 3-6 | Maximum LBA |  |
| 7 | Command set/feature set supported |  |
|  | 15:14 | Reserved |
|  | 13 | 1 = Reporting support for SMART Conveyance self-test is allowed |
|  | 12 | 1 = Reporting support for SMART Selective self-test is allowed |
|  | 11 | 1 = Reporting support for Forced Unit Access is allowed |
|  | 10 | Reserved |
|  | 9 | 1 = Reporting support for Streaming feature set is allowed |
|  | 8 | 1 = Reporting support for 48-bit Addressing feature set is allowed |
|  | 7 | 1 = Reporting support for Host Protected Area feature set is allowed |
|  | 6 | 1 = Reporting support for Automatic acoustic management is allowed |
|  | 5 | 1 = Reporting support for READ/WRITE DMA QUEUED commands is allowed |
|  | 4 | 1 = Reporting support for Power-up in Standby feature set is allowed |
|  | 3 | 1 = Reporting support for Security feature set is allowed |
|  | 2 | 1 = Reporting support for SMART error log is allowed |
|  | 1 | 1 = Reporting support for SMART self-test is allowed |
|  | 0 | 1 = Reporting support for SMART feature set is allowed |
| 8-9 | Reserved for serial ATA |  |
| 10-207 | Reserved |  |
| 208-254 | Vendor Specific |  |
| 255 | Integrity word |  |
|  | 15:8 | Checksum |
|  | 7:0 | Signature |

###### Word 0: Data structure revision

Word 0 shall contain the value 0002h.

###### Word 1: Multiword DMA modes supported

Word 1 bits (2:0) contain the same information as contained in word 63 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command data (See 7.17.7.23).  Bits (15:3) of word 1 are reserved.

###### Word 2: Ultra DMA modes supported

Word 2 bits (6:0) contain the same information as contained in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command data (See 7.17.7.44).  Bits (15:7) of word 2 are reserved.

###### Words (6:3): Maximum LBA

Words (6:3) define the maximum LBA.  This is the highest address accepted by the device in the factory default condition.  If no DEVICE CONFIGURATION SET command has been executed modifying the factory default condition, this is the same value as that returned by a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command.

###### Word 7: Command/features set supported

Word 7 bit 0 if set to one indicates that the device is  allowed to report support for  the SMART feature set.

Word 7 bit 1 if set to one indicates that the device allowed to report support for SMART self-test including the self-test log.

Word 7 bit 2 if set to one indicates that the device is allowed to report support for SMART error logging.

Word 7 bit 3 if set to one indicates that the device is allowed to report support for the Security feature set.

Word 7 bit 4 if set to one indicates that the device is allowed to report support for the Power-up in Standby feature set.

Word 7 bit 5 if set to one indicates that the device is allowed to report support for the READ DMA QUEUED and WRITE DMA QUEUED commands.

Word 7 bit 6 if set to one indicates that the device is allowed to report support for the Automatic Acoustic Management feature set.

Word 7 bit 7 if set to one indicates that the device is allowed to report support for the Host Protected Area feature set.

Word 7 bit 8 if set to one indicates that the device is allowed to report support for the 48-bit Addressing feature set.

Word 7 bit 9 if set to one indicates that the device is allowed to report support for Streaming feature set.

Word 7 bit 10 -Reserved

Word 7 bit 11if set to one indicates that the device is allowed to report support for Force Unit Access commands.

Word 7 bit 12 if set to one indicates that the device is allowed to report support for SMART Selective self-test.

Word 7 bit 13  if set to one indicates that the device is allowed to report support for SMART Conveyance self-test.

###### Word 8-9: Reserved for serial ATA

These words are reserved for future serial ATA use.

###### Words (254:10): Reserved

###### Word 255: Integrity word

Bits (7:0) of this word shall contain the value A5h.  Bits (15:8) of this word shall contain the data structure checksum.  The data structure checksum shall be the two’s complement of the sum of all byte in words (154:0) and the byte consisting of bits (7:0) of word 255.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all bytes is zero when the checksum is correct.

This command is mandatory for devices implementing the Device Configuration Overlay feature set.

#### DEVICE CONFIGURATION SET - B1h/C3h, PIO Data Out

        - Feature Set

This command is mandatory for devices implementing the Device Configuration Overlay feature set.

        - Description

The DEVICE CONFIGURATION SET command allows a device manufacturer or a personal computer system manufacturer to reduce the set of optional commands, modes, or feature sets supported by a device as indicated by a DEVICE CONFIGURATION IDENTIFY command.  The DEVICE CONFIGURATION SET command transfers an overlay that modifies some of the bits set in words 63, 82, 83, 84, and 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command data.  When the bits in these words are cleared, the device shall no longer support the indicated command, mode, or feature set.  If a bit is set in the overlay transmitted by the device that is not set in the overlay received from a DEVICE CONFIGURATION IDENTIFY command, no action is taken for that bit.  Modifying the maximum LBA of the device also modifies the address value returned by a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command.

The format of the overlay transmitted by the device is described in 7.10.4.6.  The restrictions on changing these bits are also described in clause 7.10.4.6.  If any of the bit modification restrictions described are violated, the device shall return command aborted.

The term ‘is allowed’ indicates that the device may report that a feature is supported and/or enabled.

If the device is not ‘allowed’ to report support, then the device shall not support and shall report that the selected feature is both ‘not supported’ and if appropriate ‘not enabled.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | C3h |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | B1h |

        - Normal outputs

See Table 65

        - Error outputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |  |
|  |  |  | 15:8 |  | Reserved |  |  |
|  |  |  | 7:3 |  | N/A |  |  |
|  |  |  | 2 |  | Abort - See clause 6.3.1. Abort shall be set to one if a DEVICE CONFIGURATION SET command has already modified the original settings as reported by a DEVICE CONFIGURATION IDENTIFY command, if DEVICE CONFIGURATION FREEZE LOCK is set, if any of the bit modification restrictions described in 7.10.4.1 are violated, or if a Host Protected Area has been established by the execution of a SET MAX ADDRESS or SET MAX ADDRESS EXT command, or if an attempt was made to modify a mode or feature that cannot be modified with the device in its current state. |  |  |
|  |  |  | 1:0 |  | N/A |  |  |
| 01h | Count | Vendor Specific |  |  |  |  |  |
| 02h-04h | LBA |  |  |  |  |  |  |
|  |  |  |  | Bit |  | Description |  |
|  |  |  |  | 47:24 |  | Reserved |  |
|  |  |  |  | 23:16 |  | Word location - If the command was aborted because an attempt was made to modify a bit that cannot be modified with the device in its current state, this register shall contain the offset of the first word encountered that cannot be changed. If an illegal maximum LBA is encountered, the offset of word 3 shall be entered. If a checksum error occurred, the value FFh shall be entered. A value of 00h indicates that the Data Structure Revision was invalid. |  |
|  |  |  |  | 15:0 |  | Bit Location - If the command was aborted because an attempt was made to modify a mode or feature that cannot be modified with the device in its current state, this register shall contain bits set in the bit positions that correspond to the bits in the device configuration overlay data structure words 1, 2, or 7 for each mode or feature that cannot be changed. If not, the value shall be 00h. [Editors Note: This is really unclear] |  |
| 05h | Status |  |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |  |
|  |  |  | 15:8 |  | Reserved |  |  |
|  |  |  | 7:6 |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 |  | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 |  | N/A |  |  |
|  |  |  | 3 |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 |  | N/A |  |  |
|  |  |  | 0 |  | Error - See clause 6.2.3 |  |  |

        - Output Data Structure

Table 13 - Device Configuration Overlay data structure 13

| Word | Content |  |
| --- | --- | --- |
| 0 | Data structure revision |  |
| 1 | Multiword DMA modes supported |  |
|  | 15:3 | Reserved |
|  | 2 | 1 = Reporting support for Multiword DMA mode 2 and below is allowed |
|  | 1 | 1 = Reporting support for Multiword DMA mode 1 and below is allowed |
|  | 0 | 1 = Reporting support for Multiword DMA mode 0 is allowed |
| 2 | Ultra DMA modes supported |  |
|  | 15:7 | Reserved |
|  | 6 | 1 = Reporting support for Ultra DMA mode 6 and below is allowed |
|  | 5 | 1 = Reporting support for Ultra DMA mode 5 and below is allowed |
|  | 4 | 1 = Reporting support for Ultra DMA mode 4 and below is allowed |
|  | 3 | 1 = Reporting support for Ultra DMA mode 3 and below is allowed |
|  | 2 | 1 = Reporting support for Ultra DMA mode 2 and below is allowed |
|  | 1 | 1 = Reporting support for Ultra DMA mode 1 and below is allowed |
|  | 0 | 1 = Reporting support for Ultra DMA mode 0 is allowed |
| 3-6 | Maximum LBA |  |
| 7 | Command set/feature set supported |  |
|  | 15:14 | Reserved |
|  | 13 | 1 = Reporting support for SMART Conveyance self-test is allowed |
|  | 12 | 1 = Reporting support for SMART Selective self-test is allowed |
|  | 11 | 1 = Reporting support for Forced Unit Access is allowed |
|  | 10 | -Reserved for technical report |
|  | 9 | 1 = Reporting support for Streaming feature set is allowed |
|  | 8 | 1 = Reporting support for 48-bit Addressing feature set is allowed |
|  | 7 | 1 = Reporting support for Host Protected Area feature set is allowed |
|  | 6 | 1 = Reporting support for Automatic acoustic management is allowed |
|  | 5 | 1 = Reporting support for READ/WRITE DMA QUEUED commands is allowed |
|  | 4 | 1 = Reporting support for Power-up in Standby feature set is allowed |
|  | 3 | 1 = Reporting support for Security feature set is allowed |
|  | 2 | 1 = Reporting support for SMART error log is allowed |
|  | 1 | 1 = Reporting support for SMART self-test is allowed |
|  | 0 | 1 = Reporting support for SMART feature set is allowed |
| 8-9 | Reserved for serial ATA |  |
| 10-254 | Reserved |  |
| 255 | Integrity word |  |
|  | 15:8 | Checksum |
|  | 7:0 | Signature |

###### Word 0: Data structure revision

Word 0 shall contain the value 0002h.

###### Word 1: Multiword DMA modes supported

Word 1 bits (15:3) are reserved.

Word 1 bit 2 is cleared to disable support for Multiword DMA mode 2 and has the effect of clearing bit 2 in word 63 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Multiword DMA mode 2 is currently selected.

Word 1 bit 1 is cleared to disable support for Multiword DMA mode 1 and has the effect of clearing bit 1 to zero in word 63 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Multiword DMA mode 2 is supported or Multiword DMA mode 1 or 2 is selected.

Word 1 bit 0 shall not be cleared to zero.

###### Word 2: Ultra DMA modes supported

Word 2 bits (15:7) are reserved.

Word 2 bit 6 is cleared to zero to disable support for Ultra DMA mode 6 and has the effect of clearing bit 6 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 6 is currently selected.

Word 2 bit 5 is cleared to zero to disable support for Ultra DMA mode 5 and has the effect of clearing bit 5 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 5 is currently selected.

Word 2 bit 4 is cleared to zero to disable support for Ultra DMA mode 4 and has the effect of clearing bit 4 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 5 is supported or if Ultra DMA mode 5 or 4 is selected.

Word 2 bit 3 is cleared to zero to disable support for Ultra DMA mode 3 and has the effect of clearing bit 3 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 5 or 4 is supported or if Ultra DMA mode 5, 4, or 3 is selected.

Word 2 bit 2 is cleared to zero to disable support for Ultra DMA mode 2 and has the effect of clearing bit 2 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 5, 4, or 3 is supported or if Ultra DMA mode 5, 4, 3, or 2 is selected.

Word 2 bit 1 is cleared to zero to disable support for Ultra DMA mode 1 and has the effect of clearing bit 1 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 5, 4, 3, or 2 is supported or if Ultra DMA mode 5, 4, 3, 2, or 1 is selected.

Word 2 bit 0 is cleared to zero to disable support for Ultra DMA mode 0 and has the effect of clearing bit 0 to zero in word 88 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  This bit shall not be cleared to zero if Ultra DMA mode 5, 4, 3, 2, or 1 is supported or if Ultra DMA mode 5, 4, 3, 2, 1, or 0 is selected.

###### Words (6:3): Maximum LBA

Words (6:3) define the maximum LBA.  This shall be the highest address accepted by the device after execution of the command.  When this value is changed, the content of IDENTIFY DEVICE data words (61:60) and (103:100) shall be changed as described in the SET MAX ADDRESS and SET MAX ADDRESS EXT command descriptions to reflect the maximum address set with this command.  This value shall not be changed and command aborted shall be returned if a Host Protected Area has been established by the execution of a SET MAX ADDRESS or SET MAX ADDRESS EXT command with an address value less than that returned by a READ NATIVE MAX ADDRESS or READ NATIVE MAX ADDRESS EXT command.  Any data contained in the Host Protected Area is not affected.

###### Word 7: Command/features set supported

Word 7 bits (15:14) are reserved.

Word 7 bit 13 is cleared to zero to disable support for the SMART Conveyance self-test.  Subsequent attempts to start this test via the SMART EXECUTE OFF-LINE IMMEDIATE command shall cause that command to abort.  In addition, the SMART READ DATA command shall clear bit 5 to zero in the “Off-line data collection capabilities“ field.  If this bit is supported by DEVICE CONFIGURATION SET, then this feature shall not be disabled by bit 1 of word 7.

Word 7 bit 12 is cleared to zero to disable support for the SMART Selective self-test.  Subsequent attempts to start this test test via the SMART EXECUTE OFF-LINE IMMEDIATE command shall cause that command to abort.  In addition, the SMART READ DATA command shall clear bit 6 to zero in the “Off-line data collection capabilities “ field.  If this bit is supported by DEVICE CONFIGURATION SET, then this feature shall not be disabled by bit 1 of word 7.

Word 7 bit 11 is cleared to zero to disable support for the Force Unit Access commands and has the effect of clearing bits 6 and 7 to zero in word 84 and word 87 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 10 is -Reserved

Word 7 bit 9 is cleared to zero to disable support for the Streaming feature set and has the effect of clearing bits 4, 9 and 10 to zero in word 84 and word 87 and clearing the value in words (99:95) and word 104  of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 8 is cleared to zero to disable support for the 48-bit Addressing feature set and has the effect of clearing bit 10 to zero in word 83 and word 86 and clearing the value in words (103:100) of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 7 is cleared to zero to disable support for the Host Protected Area feature set and has the effect of clearing bit 10 to zero in word 82 and word 85 and clearing bit 8 to zero in word 83 and word 86 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  If a Host Protected Area has been established by use of the SET MAX ADDRESS or SET MAX ADDRESS EXT command, these bits shall not be cleared to zero and the device shall return command aborted.

Word 7 bit 6 is cleared to zero to disable for the Automatic Acoustic Management feature set and has the effect of clearing bit 9 to zero in word 83 and word 94 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 5 is cleared to zero to disable support for the READ DMA QUEUED and WRITE DMA QUEUED commands and has the effect of clearing bit 1 to zero in word 83 and word 86 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 4 is cleared to zero to disable support for the Power-up in Standby feature set and has the effect of clearing bits (6:5) to zero in word 83 and word 86 and clearing the value in word 94 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  If Power-up in Standby has been enabled by a jumper, these bits shall not be cleared.

Word 7 bit 3 is cleared to zero to disable support for the Security feature set and has the effect of clearing bit 1 to zero in word 82 and word 85 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  These bits shall not be cleared if the Security feature set has been enabled.

Word 7 bit 2 is cleared to zero to disable support for the SMART error logging and has the effect of clearing bit 0 to zero in word 84 and word 87 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 1 is cleared to zero to disable support for the SMART self-test and has the effect of clearing bit 1 to zero in word 84 and word 87 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.

Word 7 bit 1 disables support for the offline, short , extended self-tests (off-line and captive modes).

If bit 12 or bit 13 of word 7 are not supported, Word 7 bit 1 may also disable support for conveyance self-test and selective self-test.  [Editors Note: This is unclear?]

Word 7 bit 0 is cleared to zero to disable support for the SMART feature set and has the effect of clearing bit 0 to zero in word 82 and word 85 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE response.  If bits (2:1) of word 7 are not cleared to zero or if the SMART feature set has been enabled by use of the SMART ENABLE OPERATIONS command, these bits shall not be cleared and the device shall return command aborted.

###### Words 8-9: Reserved for serial ATA

These words are reserved for future serial ATA use.

###### Words (254:10): Reserved

###### Word 255: Integrity word

Bits (7:0) of this word shall contain the value A5h.  Bits (15:8) of this word shall contain the data structure checksum.  The data structure checksum shall be the two’s complement of the sum of all byte in words (254:0) and the byte consisting of bits (7:0) of word 255.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all bytes is zero when the checksum is correct.

This command is mandatory for devices implementing the Device Configuration Overlay feature set.

### DEVICE RESET - 08h, Device reset

      - Feature Set

This command is mandatory for devices implementing the PACKET Command feature set.

      - Description

The DEVICE RESET command enables the host to reset a device.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | 08h |

      - Normal outputs

See Table 70

      - Error outputs

If supported, this command shall not end in an error condition.  If this command is not supported and the device has the BSY bit or the DRQ bit set to one when the command is written, the results of this command are indeterminate.  If this command is not supported and the device has the BSY bit and the DRQ bit cleared to zero when the command is written, the device shall respond with command aborted.  [Editors Note: Not sure what to do with this, Jim hatfield to wordsmith]

### DOWNLOAD MICROCODE - 92h, PIO Data-out

      - Feature Set

This command is optional for devices implementing the General feature set

      - Description

This command enables the host to alter the device’s microcode.  The data transferred using the DOWNLOAD MICROCODE command is vendor specific.

All transfers shall be an integer multiple of the sector size.  The size of the data transfer is determined by the contents of the LBA and Count fields.  The LBA field shall be used to extend the Count field to create a 16-bit sector count value.  The low order 8 bits of the LBA field shall be the most significant eight bits and the Count field shall be the least significant eight bits.  A value of zero in both field shall specify no data is to be transferred.  This allows transfer sizes from 0 bytes to 33,553,920 bytes, in 512 byte increments.

The Features field shall be used to determine the effect of the DOWNLOAD MICROCODE command as described in 7.12.3.

      - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |
|  |  |  | Sub command | Description |  |
|  |  |  | 00h | Reserved |  |
|  |  |  | 01h | Download is for immediate, temporary use. |  |
|  |  |  | 02h-06h | Reserved |  |
|  |  |  | 07h | Save downloaded code for immediate and future use. |  |
|  |  |  | 08-FFh | Reserved |  |
| 01h | Count | Sector Count (Low Order) |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:8 | Reserved |  |
|  |  |  | 7:0 | Sector Count (High Order) |  |
| 05h | Command | 92h |  |  |  |

      - Normal outputs

See Table 65

      - Error outputs

The device shall return command aborted if the device did not accept the microcode data.  The device shall return command aborted if subcommand code is not a supported value.  See Table 79.

### EXECUTE DEVICE DIAGNOSTIC - 90h, Device diagnostic

      - Feature Set

This command is mandatory for all devices.

      - Description

This command shall cause the devices to perform the internal diagnostic tests.  Both devices, if present, shall execute this command regardless of which device is selected.

If the host issues an EXECUTE DEVICE DIAGNOSTIC command while a device is in or going to a power management mode except Sleep, then the device shall execute the EXECUTE DEVICE DIAGNOSTIC sequence.

This command shall be accepted regardless of the state of DRDY.  [Editors Note: This probably needs to move to the transport standards]

      - Inputs

Only the command code (90h).  All other registers shall be ignored.

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | 90h |

DEV shall be ignored.  [Editors Note: This needs to be in the transport standards]

      - Normal outputs

See Table 70.

The diagnostic code written into the Error field is an 8-bit code.  Table 14 defines these values. ]

Table 14 - Diagnostic codes14

| Code (See note 1) | Description |
| --- | --- |
| When this code is in the Device 0 Error register |  |
| 01h | Device 0 passed, Device 1 passed or not present |
| 00h, 02h-7Fh | Device 0 failed, Device 1 passed or not present |
| 81h | Device 0 passed, Device 1 failed |
| 80h, 82h-FFh | Device 0 failed, Device 1 failed |
| When this code is in the Device 1 Error register |  |
| 01h | Device 1 passed (See note 2) |
| 00h, 02h-7Fh | Device 1 failed (See note 2) |
| NOTE  1 Codes other than 01h and 81h may indicate additional information about the failure(s). 2 If Device 1 is not present, the host may see the information from Device 0 even though Device 1 is selected. |  |

      - Error outputs

Table 14 shows the error information that is returned as a diagnostic code in the Error register.

### FLUSH CACHE - E7h, Non-data

      - Feature Set

This command is mandatory for devices not implementing the PACKET Command feature set.  This command is optional for devices implementing the PACKET Command feature set.

      - Description

This command is used by the host to request the device to flush the write cache.  If there is data in the write cache, that data shall be written to the media.  The BSY bit shall remain set to one until all data has been successfully written or an error occurs.

NOTE  This command may take longer than 30 s to complete.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | E7h |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during execution of writing data results in the termination of the command and the Command Block registers contain the sector address of the sector where the first unrecoverable error occurred.  Subsequent FLUSH CACHE commands continue the process of flushing the cache starting with the first sector after the sector in error.  LBA bits 47:28 shall be cleared to zero.  See Table 81

### FLUSH CACHE EXT - EAh, Non-data

      - Feature Set

This command is mandatory for devices implementing the 48-bit Address feature set

      - Description

This command is used by the host to request the device to flush the write cache.  If there is data in the write cache, that data shall be written to the madia.The BSY bit shall remain set to one until all data has been successfully written or an error occurs.

NOTE  This command may take longer than 30 s to complete.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | Reserved |
| 01h | Count | Reserved |
| 02h-04h | LBA | Reserved |
| 05h | Command | EAh |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered while writing data results in the termination of the command and the Command Block registers contain the sector address of the sector where the first unrecoverable error occurred.  Subsequent FLUSH CACHE EXT commands continue the process of flushing the cache starting with the first sector after the sector in error. See Table 81.

### GET MEDIA STATUS - DAh, Non-data

      - Feature Set

This command is mandatory for devices implementing the Removable Media Status Notification feature set.  This command is optional for devices implementing the Removable Media feature set.  [Editors Note: Should we obsolete this feature set? or sets?]

      - Description

This command returns media status bits WP, MC, MCR, and NM, as defined above.  When Media Status Notification is disabled this command returns zeros in the WP, MC, MCR, and NM bits.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | Reserved |
| 01h | Count | Reserved |
| 02h-04h | LBA | Reserved |
| 05h | Command | DAhh |

      - Normal outputs

Normal outputs are returned if Media Status Notification is disabled or if no bits are set to one in the Error register.  See Table 65 for return status format

      - Error outputs

See Table 82

### IDENTIFY DEVICE - ECh, PIO Data-in

      - Feature Set

This command is mandatory for all devices.

      - Description

The IDENTIFY DEVICE command enables the host to receive parameter information from the device.  See Table 15 for a description of the return data.  [Editors Note: Need to figure out how to sort out the transport specific data

Some devices may have to read the media in order to complete this command.

When the command is issued, the device sets the BSY bit to one, prepares to transfer the 256 words of device identification data to the host, sets the DRQ bit to one, clears the BSY bit to zero, and asserts INTRQ if nIEN is cleared to zero.  The host may then transfer the data by reading the Data register.  Input Data [Editors Note; Add a column that indicates which transport uses the field] defines the arrangement and meaning of the parameter words in the buffer.  All reserved bits or words shall be zero.  [Editors Note: Need to rework this to stay away from BSY/DRQ]

Some parameters are defined as a 16-bit value.  A word that is defined as a 16-bit value places the most significant bit of the value on signal line DD15 and the least significant bit on signal line DD0 (See 3.2.6).  For serial implementation see 3.2.7.

Some parameters are defined as 32-bit values (e.g., words (61:60)).  Such fields are transferred using two successive word transfers.  The device shall first transfer the least significant bits, bits (15:0) of the value, on signal lines DD(15:0) respectively.  After the least significant bits have been transferred, the most significant bits, bits (31:16) of the value, shall be transferred on DD(15:0) respectively (See 3.2.6).

Some parameters are defined as a string of ACSII characters.  Such fields are transferred as defined in 3.2.6.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | ECh |

      - Normal outputs for devices that do not implement the Packet Command feature set

See Table 65

      - Normal Outputs for PACKET Command feature set devices

In response to this command, devices that implement the PACKET Command feature set shall post command aborted and place the PACKET Command feature set signature in the appropriate fields, see 7.11.4.

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:3 | N/A |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |
|  |  |  | 1:0 | N/A |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | N/A |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5:4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

      - Error outputs

Devices not implementing the PACKET Command feature set shall not report an error.  [Editors Note: Does this mean that a device can not abort IDENTIFY DEVICE]

      - Input Data [Editors Note; Add a column that indicates which transport uses the field]

Table 15 - IDENTIFY DEVICE data15

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 0 | M |  | General configuration bit-significant information: |  |
|  |  | F | 15 | 0 = ATA device |
|  |  | X | 14:8 | Retired |
|  |  | F | 7 | 1 = removable media device |
|  |  | X | 6 | Obsolete |
|  |  | X | 5:3 | Retired |
|  |  | V | 2 | Response incomplete |
|  |  | X | 1 | Retired |
|  |  | F | 0 | Reserved |
| 1 |  | X | Obsolete |  |
| 2 | O | V | Specific configuration |  |
| 3 |  | X | Obsolete |  |
| 4-5 |  | X | Retired |  |
| 6 |  | X | Obsolete |  |
| 7-8 | O | V | Reserved for assignment by the CompactFlash Association |  |
| 9 |  | X | Retired |  |
| 10-19 | M | F | Serial number (20 ASCII characters) |  |
| 20-21 |  | X | Retired |  |
| 22 |  | X | Obsolete |  |
| 23-26 | M | F | Firmware revision (8 ASCII characters) |  |
| 27-46 | M | F | Model number (40 ASCII characters) |  |
| 47 | M | F | 15:8 | 80h |
|  |  | F | 7:0 | 00h = Reserved |
|  |  | F |  | 01h-FFh = Maximum number of sectors that shall be transferred per interrupt on READ/WRITE MULTIPLE commands |
| 48 |  | F | Reserved |  |
| 49 | M |  | Capabilities |  |
|  |  | F | 15:14 | Reserved for the IDENTIFY PACKET DEVICE command. |
|  |  | F | 13 | 1 = Standby timer values as specified in this standard are supported |
|  |  |  |  | 0 = Standby timer values shall be managed by the device |
|  |  | F | 12 | Reserved for the IDENTIFY PACKET DEVICE command. |
|  |  | F | 11 | 1 = IORDY supported |
|  |  |  |  | 0 = IORDY may be supported |
|  |  | F | 10 | 1 = IORDY may be disabled |
|  |  | F | 9 | 1 = LBA supported |
|  |  | F | 8 | 1 = DMA supported. |
|  |  | X | 7:0 | Retired |
| 50 | M |  | Capabilities |  |
|  |  | F | 15 | Shall be cleared to zero. |
|  |  | F | 14 | Shall be set to one. |
|  |  | F | 13:2 | Reserved. |
|  |  | X | 1 | Obsolete |
|  |  | F | 0 | Shall be set to one to indicate a device specific Standby timer value minimum. |
| 51-52 |  | X | Obsolete |  |
| 53 | M | F | 15:3 | Reserved |
|  |  | F | 2 | 1 = the fields reported in word 88 are valid |
|  |  |  |  | 0 = the fields reported in word 88 are not valid |
|  |  | F | 1 | 1 = the fields reported in words (70:64) are valid |
|  |  |  |  | 0 = the fields reported in words (70:64) are not valid |
|  |  | X | 0 | Obsolete |

(continued)

Table 15 - IDENTIFY DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 54-58 |  | X | Obsolete |  |
| 59 | M | F | 15:9 | Reserved |
|  |  | V | 8 | 1 = Multiple sector setting is valid |
|  |  | V | 7:0 | xxh = Current setting for number of sectors that shall be transferred per interrupt on R/W Multiple command |
| 60-61 | M | F | Total number of user addressable sectors |  |
| 62 |  | X |  | Obsolete |
| 63 | M | F | 15:11 | Reserved |
|  |  | V | 10 | 1 = Multiword DMA mode 2 is selected |
|  |  |  |  | 0 = Multiword DMA mode 2 is not selected |
|  |  | V | 9 | 1 = Multiword DMA mode 1 is selected |
|  |  |  |  | 0 = Multiword DMA mode 1 is not selected |
|  |  | V | 8 | 1 = Multiword DMA mode 0 is selected |
|  |  |  |  | 0 = Multiword DMA mode 0 is not selected |
|  |  | F | 7:3 | Reserved |
|  |  | F | 2 | 1 = Multiword DMA mode 2 and below are supported |
|  |  | F | 1 | 1 = Multiword DMA mode 1 and below are supported |
|  |  | F | 0 | 1 = Multiword DMA mode 0 is supported |
| 64 | M | F | 15:8 | Reserved |
|  |  | F | 7:0 | PIO modes supported |
| 65 | M |  | Minimum Multiword DMA transfer cycle time per word |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 66 | M |  | Manufacturer’s recommended Multiword DMA transfer cycle time |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 67 | M |  | Minimum PIO transfer cycle time without flow control |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 68 | M |  | Minimum PIO transfer cycle time with IORDY flow control |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 69-70 |  | F | Reserved (for future command overlap and queuing) [Editors Note: should this be just reserved] |  |
| 71-74 |  | F | Reserved for the IDENTIFY PACKET DEVICE command. |  |
| 75 | O |  | Queue depth |  |
|  |  | F | 15:5 | Reserved |
|  |  | F | 4:0 | Maximum queue depth - 1 |
| 76-79 |  | F | Reserved for Serial ATA |  |
| 80 | M |  | Major version number 0000h or FFFFh = device does not report version |  |
|  |  | F | 15 | Reserved |
|  |  | F | 14 | Reserved for ATA/ATAPI-14 |
|  |  | F | 13 | Reserved for ATA/ATAPI-13 |
|  |  | F | 12 | Reserved for ATA/ATAPI-12 |
|  |  | F | 11 | Reserved for ATA/ATAPI-11 |
|  |  | F | 10 | Reserved for ATA/ATAPI-10 |
|  |  | F | 9 | Reserved for ATA/ATAPI-9 |
|  |  | F | 8 | 1 = supports ATA/ATAPI8-ACS [Editors Note: Is this for command set only? Are we going to need some new status bits?] |
|  |  | F | 7 | 1 = supports ATA/ATAPI-7 |
|  |  | F | 6 | 1 = supports ATA/ATAPI-6 |
|  |  | F | 5 | 1 = supports ATA/ATAPI-5 |
|  |  | F | 4 | 1 = supports ATA/ATAPI-4 |
|  |  | F | 3 | Obsolete |
|  |  | X | 2 | Obsolete |
|  |  | X | 1 | Obsolete |
|  |  | F | 0 | Reserved |

(continued)

Table 15 - IDENTIFY DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 81 | M | F | Minor version number 0000h or FFFFh = device does not report version 0001h-FFFEh = See 7.17.7.41 |  |
| 82 | M |  | Command set supported. |  |
|  |  | X | 15 | Obsolete |
|  |  | F | 14 | 1 = NOP command supported |
|  |  | F | 13 | 1 = READ BUFFER command supported |
|  |  | F | 12 | 1 = WRITE BUFFER command supported |
|  |  | X | 11 | Obsolete |
|  |  | F | 10 | 1 = Host Protected Area feature set supported |
|  |  | F | 9 | 1 = DEVICE RESET command supported |
|  |  | F | 8 | 1 = SERVICE interrupt supported |
|  |  | F | 7 | 1 = release interrupt supported |
|  |  | F | 6 | 1 = look-ahead supported |
|  |  | F | 5 | 1 = write cache supported |
|  |  | F | 4 | Shall be cleared to zero to indicate that the PACKET Command feature set is not supported. |
|  |  | F | 3 | 1 = mandatory Power Management feature set supported |
|  |  | F | 2 | 1 = Removable Media feature set supported |
|  |  | F | 1 | 1 = Security Mode feature set supported |
|  |  | F | 0 | 1 = SMART feature set supported |
| 83 | M |  | Command sets supported. |  |
|  |  | F | 15 | Shall be cleared to zero |
|  |  | F | 14 | Shall be set to one |
|  |  | F | 13 | 1 = FLUSH CACHE EXT command supported |
|  |  | F | 12 | 1 = mandatory FLUSH CACHE command supported |
|  |  | F | 11 | 1 = Device Configuration Overlay feature set supported |
|  |  | F | 10 | 1 = 48-bit Address feature set supported |
|  |  | F | 9 | 1 = Automatic Acoustic Management feature set supported |
|  |  | F | 8 | 1 = SET MAX security extension supported |
|  |  | F | 7 | See Address Offset Reserved Area Boot, INCITS TR27:2001 |
|  |  | F | 6 | 1 = SET FEATURES subcommand required to spinup after power-up |
|  |  | F | 5 | 1 = Power-Up In Standby feature set supported |
|  |  | F | 4 | 1 = Removable Media Status Notification feature set supported |
|  |  | F | 3 | 1 = Advanced Power Management feature set supported |
|  |  | F | 2 | 1 = CFA feature set supported |
|  |  | F | 1 | 1 = READ/WRITE DMA QUEUED supported |
|  |  | F | 0 | 1 = DOWNLOAD MICROCODE command supported |
| 84 | M |  | Command set/feature supported extension. |  |
|  |  | F | 15 | Shall be cleared to zero |
|  |  | F | 14 | Shall be set to one |
|  |  | F | 13 | 1 = IDLE IMMEDIATE with UNLOAD FEATURE supported |
|  |  | F | 12 | Reserved for technical report INCITS TR-37-2004 (TLC) |
|  |  | F | 11 | Reserved for technical report INCITS TR-37-2004 (TLC) |
|  |  | F | 10 | 1 = URG bit supported for WRITE STREAM DMA EXT and WRITE STREAM EXT |
|  |  | F | 9 | 1 = URG bit supported for READ STREAM DMA EXT and READ STREAM EXT |
|  |  | F | 8 | 1 = 64-bit World wide name supported |
|  |  | F | 7 | 1 = WRITE DMA QUEUED FUA EXT command supported |
|  |  | F | 6 | 1 = WRITE DMA FUA EXT and WRITE MULTIPLE FUA EXT commands supported |
|  |  | F | 5 | 1 = General Purpose Logging feature set supported |
|  |  | F | 4 | 1 = Streaming feature set supported |
|  |  | F | 3 | 1 = Media Card Pass Through Command feature set supported |
|  |  | F | 2 | 1 = Media serial number supported |
|  |  | F | 1 | 1 = SMART self-test supported |
|  |  | F | 0 | 1 = SMART error logging supported |

(continued)

Table 15 - IDENTIFY DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 85 | M |  | Command set/feature enabled. |  |
|  |  | X | 15 | Obsolete |
|  |  | F | 14 | 1 = NOP command enabled |
|  |  | F | 13 | 1 = READ BUFFER command enabled |
|  |  | F | 12 | 1 = WRITE BUFFER command enabled |
|  |  | X | 11 | Obsolete |
|  |  | V | 10 | 1 = Host Protected Area feature set enabled |
|  |  | F | 9 | 1 = DEVICE RESET command enabled |
|  |  | V | 8 | 1 = SERVICE interrupt enabled |
|  |  | V | 7 | 1 = release interrupt enabled |
|  |  | V | 6 | 1 = look-ahead enabled |
|  |  | V | 5 | 1 = write cache enabled |
|  |  | F | 4 | Shall be cleared to zero to indicate that the PACKET Command feature set is not supported. |
|  |  | F | 3 | 1 = Power Management feature set enabled |
|  |  | F | 2 | 1 = Removable Media feature set enabled |
|  |  | V | 1 | 1 = Security Mode feature set enabled |
|  |  | V | 0 | 1 = SMART feature set enabled |
| 86 | M |  | Command set/feature enabled. |  |
|  |  |  | 15 | Enable detection of words 120:119 (Need Proposal) |
|  |  | F | 14 | Reserved |
|  |  | F | 13 | 1 = FLUSH CACHE EXT command supported |
|  |  | F | 12 | 1 = FLUSH CACHE command supported |
|  |  | F | 11 | 1 = Device Configuration Overlay supported |
|  |  | F | 10 | 1 = 48-bit Address features set supported |
|  |  | V | 9 | 1 = Automatic Acoustic Management feature set enabled |
|  |  | F | 8 | 1 = SET MAX security extension enabled by SET MAX SET PASSWORD |
|  |  | F | 7 | Reserved for Address Offset Reserved Area Boot, INCITS TR27:2001 |
|  |  | F | 6 | 1 = SET FEATURES subcommand required to spin-up after power-up |
|  |  | V | 5 | 1 = Power-Up In Standby feature set enabled |
|  |  | V | 4 | 1 = Removable Media Status Notification feature set enabled |
|  |  | V | 3 | 1 = Advanced Power Management feature set enabled |
|  |  | F | 2 | 1 = CFA feature set enabled |
|  |  | F | 1 | 1 = READ/WRITE DMA QUEUED command supported |
|  |  | F | 0 | 1 = DOWNLOAD MICROCODE command supported |
| 87 | M |  | Command set/feature enabled. |  |
|  |  | F | 15 | Shall be cleared to zero |
|  |  | F | 14 | Shall be set to one |
|  |  | F | 13 | 1 = IDLE IMMEDIATE with UNLOAD FEATURE supported |
|  |  | V | 12 | Reserved for technical report- INCITS TR-37-2004 (TLC) |
|  |  | V | 11 | Reserved for technical report- INCITS TR-37-2004 (TLC) |
|  |  | F | 10 | 1 = URG bit supported for WRITE STREAM DMA EXT and WRITE STREAM EXT |
|  |  | F | 9 | 1 = URG bit supported for READ STREAM DMA EXT and READ STREAM EXT |
|  |  | F | 8 | 1 = 64 bit World wide name supported |
|  |  | F | 7 | 1 = WRITE DMA QUEUED FUA EXT command supported |
|  |  | F | 6 | 1 = WRITE DMA FUA EXT and WRITE MULTIPLE FUA EXT commands supported |
|  |  | F | 5 | 1 = General Purpose Logging feature set supported |
|  |  | V | 4 | 1 = Valid CONFIGURE STREAM command has been executed |
|  |  | V | 3 | 1 = Media Card Pass Through Command feature set enabled |
|  |  | V | 2 | 1 = Media serial number is valid |
|  |  | F | 1 | 1 = SMART self-test supported |
|  |  | F | 0 | 1 = SMART error logging supported |

(continued)

Table 15 - IDENTIFY DEVICE data (continued)

| Word | O/M | F/V | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 88 | O | F | 15 | Reserved |  |  |
|  |  | V | 14 | 1 = Ultra DMA mode 6 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 6 is not selected |  |  |
|  |  | V | 13 | 1 = Ultra DMA mode 5 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 5 is not selected |  |  |
|  |  | V | 12 | 1 = Ultra DMA mode 4 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 4 is not selected |  |  |
|  |  | V | 11 | 1 = Ultra DMA mode 3 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 3 is not selected |  |  |
|  |  | V | 10 | 1 = Ultra DMA mode 2 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 2 is not selected |  |  |
|  |  | V | 9 | 1 = Ultra DMA mode 1 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 1 is not selected |  |  |
|  |  | V | 8 | 1 = Ultra DMA mode 0 is selected |  |  |
|  |  |  |  | 0 = Ultra DMA mode 0 is not selected |  |  |
|  |  | F | 7 | Reserved |  |  |
|  |  | F | 6 | 1 = Ultra DMA mode 6 and below are supported |  |  |
|  |  | F | 5 | 1 = Ultra DMA mode 5 and below are supported |  |  |
|  |  | F | 4 | 1 = Ultra DMA mode 4 and below are supported |  |  |
|  |  | F | 3 | 1 = Ultra DMA mode 3 and below are supported |  |  |
|  |  | F | 2 | 1 = Ultra DMA mode 2 and below are supported |  |  |
|  |  | F | 1 | 1 = Ultra DMA mode 1 and below are supported |  |  |
|  |  | F | 0 | 1 = Ultra DMA mode 0 is supported |  |  |
| 89 | O | F | Time required for security erase unit completion |  |  |  |
| 90 | O | F | Time required for Enhanced security erase completion |  |  |  |
| 91 | O | V | Current advanced power management value |  |  |  |
| 92 | O | V | Master Password Revision Code |  |  |  |
| 93 | * |  | Hardware reset result. The contents of bits (12:0) of this word shall change only during the execution of a hardware reset. |  |  |  |
|  |  | F | 15 | Shall be cleared to zero. |  |  |
|  |  | F | 14 | Shall be set to one. |  |  |
|  |  | V | 13 | 1 = device detected CBLID- above ViH |  |  |
|  |  |  |  | 0 = device detected CBLID- below ViL |  |  |
|  |  |  | 12:8 | Device 1 hardware reset result. Device 0 shall clear these bits to zero. Device 1 shall set these bits as follows: |  |  |
|  |  | F |  | 12 | Reserved. |  |
|  |  | V |  | 11 | 0 = Device 1 did not assert PDIAG-. |  |
|  |  |  |  |  | 1 = Device 1 asserted PDIAG-. |  |
|  |  | V |  | 10:9 | These bits indicate how Device 1 determined the device number: |  |
|  |  |  |  |  | 00 = Reserved. |  |
|  |  |  |  |  | 01 = a jumper was used. |  |
|  |  |  |  |  | 10 = the CSEL signal was used. |  |
|  |  |  |  |  | 11 = some other method was used or the method is unknown. |  |
|  |  |  |  | 8 | Shall be set to one. |  |
|  |  |  | 7:0 | Device 0 hardware reset result. Device 1 shall clear these bits to zero. Device 0 shall set these bits as follows: |  |  |
|  |  | F |  | 7 |  | Reserved. |
|  |  | F |  | 6 |  | 0 = Device 0 does not respond when Device 1 is selected. |
|  |  |  |  |  |  | 1 = Device 0 responds when Device 1 is selected. |
|  |  | V |  | 5 |  | 0 = Device 0 did not detect the assertion of DASP-. |
|  |  |  |  |  |  | 1 = Device 0 detected the assertion of DASP-. |
|  |  | V |  | 4 |  | 0 = Device 0 did not detect the assertion of PDIAG-. |
|  |  |  |  |  |  | 1 = Device 0 detected the assertion of PDIAG-. |
|  |  | V |  | 3 |  | 0 = Device 0 failed diagnostics. |
|  |  |  |  |  |  | 1 = Device 0 passed diagnostics. |
|  |  | V |  | 2:1 |  | These bits indicate how Device 0 determined the device number: |
|  |  |  |  |  |  | 00 = Reserved. |
|  |  |  |  |  |  | 01 = a jumper was used. |
|  |  |  |  |  |  | 10 = the CSEL signal was used. |
|  |  |  |  |  |  | 11 = some other method was used or the method is unknown. |
|  |  | F |  | 0 |  | Shall be set to one. |

(continued)

Table 15 - IDENTIFY DEVICE information (continued)

| Word | O/M | F/V | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 94 | O | V | 15:8 |  | Vendor’s recommended acoustic management value. |  |
|  |  | V | 7:0 |  | Current automatic acoustic management value. |  |
| 95 |  | F | Stream Minimum Request Size |  |  |  |
| 96 |  | V | Streaming Transfer Time - DMA |  |  |  |
| 97 |  | V | Streaming Access Latency - DMA and PIO |  |  |  |
| 98-99 |  | F | Streaming Performance Granularity |  |  |  |
| 100-103 | O | V | Maximum user LBA for 48-bit Address feature set. |  |  |  |
| 104 | O | V | Streaming Transfer Time - PIO |  |  |  |
| 105 |  | F | Reserved |  |  |  |
| 106 | O |  | Physical sector size / Logical Sector Size |  |  |  |
|  |  | F | 15 |  |  | Shall be cleared to zero |
|  |  | F | 14 |  |  | Shall be set to one |
|  |  | F | 13 |  |  | 1 = Device has multiple logical sectors per physical sector. |
|  |  |  | 12 |  |  | 1= Device Logical Sector Longer than 256 Words |
|  |  | F | 11:4 |  |  | Reserved |
|  |  | F | 3:0 |  |  | 2X logical sectors per physical sector |
| 107 | O | F | Inter-seek delay for ISO-7779 acoustic testing in microseconds |  |  |  |
| 108 | O | F | 15:12 11:0 |  |  | NAA (3:0) IEEE OUI (23:12) |
| 109 | O | F | 15:4 3:0 |  |  | IEEE OUI (11:0) Unique ID (35:32) |
| 110 | O | F | 15:0 |  |  | Unique ID (31:16) |
| 111 | O | F | 15:0 |  |  | Unique ID (15:0) |
| 112-115 | O | F | Reserved for world wide name extension to 128 bits |  |  |  |
| 116 | O | V | Reserved for TLC technical report- INCITS TR-37-2004 |  |  |  |
| 117-118 | O | F | Words per Logical Sector |  |  |  |
| 119 | M | F | Supported Settings (Continued from words 84:82) [Editors Note: Description not updated] |  |  |  |
|  |  |  | 15 | Shall be cleared to zero |  |  |
|  |  |  | 14 | Shall be set to one |  |  |
|  |  |  | 13:2 | Reserved |  |  |
|  |  |  | 1 | 1 = Write Read Verify feature set is supported |  |  |
|  |  |  | 0 | reserved for DRQ technical report |  |  |
| 120 | M |  | Enabled Settings (Continued from words 87:85) [Editors Note: Description not updated] |  |  |  |
|  |  |  | 15 | Shall be cleared to zero |  |  |
|  |  |  | 14 | Shall be set to one |  |  |
|  |  |  | 13:2 | Reserved |  |  |
|  |  |  | 1 | 1 = Write Read Verify feature set is enabled |  |  |
|  |  |  | 0 | Reserved for DRQ technical report |  |  |
| 121-126 |  | F | Reserved |  |  |  |
| 127 | O |  | Removable Media Status Notification feature set support |  |  |  |
|  |  | F | 15:2 |  | Reserved |  |
|  |  | F | 1:0 |  | 00 = Removable Media Status Notification feature set not supported |  |
|  |  |  |  |  | 01 = Removable Media Status Notification feature supported |  |
|  |  |  |  |  | 10 = Reserved |  |
|  |  |  |  |  | 11 = Reserved |  |
| 128 | O |  | Security status |  |  |  |
|  |  | F | 15:9 |  | Reserved |  |
|  |  | V | 8 |  | Security level 0 = High, 1 = Maximum |  |
|  |  | F | 7:6 |  | Reserved |  |
|  |  | F | 5 |  | 1 = Enhanced security erase supported |  |
|  |  | V | 4 |  | 1 = Security count expired |  |
|  |  | V | 3 |  | 1 = Security frozen |  |
|  |  | V | 2 |  | 1 = Security locked |  |
|  |  | V | 1 |  | 1 = Security enabled |  |
|  |  | F | 0 |  | 1 = Security supported |  |

(continued)

Table 15 - IDENTIFY DEVICE data (continued)

| Word | O/M | F/V | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| 129-159 |  | X | Vendor specific |  |  |
| 160 | O |  | CFA power mode 1 |  |  |
|  |  | F | 15 | Word 160 supported |  |
|  |  | F | 14 | Reserved |  |
|  |  | F | 13 | CFA power mode 1 is required for one or more commands implemented by the device |  |
|  |  | V | 12 | CFA power mode 1 disabled |  |
|  |  | F | 11:0 | Maximum current in ma |  |
| 161-175 |  | X | Reserved for assignment by the CompactFlash Association |  |  |
| 176-205 | O | V | Current media serial number |  |  |
| 206 |  |  | [Editors Note: 206 is reserved for SCT] |  |  |
| 207-254 |  | F | Reserved [Editors Note: Use 209 for Alignment] |  |  |
| 255 | M | X | Integrity word |  |  |
|  |  |  | 15:8 |  | Checksum |
|  |  |  | 7:0 |  | Signature |
| Key: O/M = Mandatory/optional requirement. M = Support of the word is mandatory. O = Support of the word is optional. * = See 7.17.7.65. F/V = Fixed/variable content F = the content of the word is fixed and does not change. For removable media devices, these values may change when media is removed or changed. V = the contents of the word is variable and may change depending on the state of the device or the commands executed by the device. X = the content of the word may be fixed or variable. |  |  |  |  |  |

(concluded)

        - Word 0: General configuration

Devices that conform to this standard shall clear bit 15 to zero.

If bit 7 is set to one, the device is a removable media device.

Bit 6 is obsolete.

If bit 2 is set to one it indicates that the content of the IDENTIFY DEVICE data is incomplete.  This will occur if the device supports the Power-up in Standby feature set and required data is contained on the device media.  In this case the content of at least word 0 and word 2 shall be valid.

Devices supporting the CFA feature set shall place the value 848Ah in word 0.  In this case, the above definitions for the bits in word 0 are not valid.

        - Word 1: Obsolete

        - Word 2: Specific configuration.

Word 2 shall be set as follows:

| Value | Description |
| --- | --- |
| 37C8h | Device requires SET FEATURES subcommand to spin-up after power-up and IDENTIFY DEVICE data is incomplete (See 4.12). |
| 738Ch | Device requires SET FEATURES subcommand to spin-up after power-up and IDENTIFY DEVICE data is complete (See 4.12). |
| 8C73h | Device does not require SET FEATURES subcommand to spin-up after power-up and IDENTIFY DEVICE data is incomplete (See 4.12). |
| C837h | Device does not require SET FEATURES subcommand to spin-up after power-up and IDENTIFY DEVICE data is complete (See 4.12). |
| All other values | Reserved. |

        - Word 3: Obsolete

        - Word (5:4): Retired.

        - Word 6: Obsolete

        - Words (8:7): Reserved for assignment by the CompactFlash Association

        - Word 9: Retired.

        - Words (19:10): Serial number

This field contains the serial number of the device.  The contents of this field is an ASCII character string of twenty bytes.  The device shall pad the character string with spaces (20h), if necessary, to ensure that the string is the proper length.  The combination of Serial number (words (19:10)) and Model number (words (46:27)) shall be unique for a given manufacturer (See 3.2.6).

        - Word (21:20): Retired.

        - Word 22: Obsolete.

        - Word (26:23): Firmware revision

This field contains the firmware revision number of the device.  The contents of this field is an ASCII character string of eight bytes.  The device shall pad the character string with spaces (20h), if necessary, to ensure that the string is the proper length (See 3.2.6).

        - Words (46:27): Model number

This field contains the model number of the device.  The contents of this field is an ASCII character string of forty bytes.  The device shall pad the character string with spaces (20h), if necessary, to ensure that the string is the proper length.  The combination of Serial number (words (19:10)) and Model number (words (46:27)) shall be unique for a given manufacturer (See 3.2.6).

        - Word 47: READ/WRITE MULTIPLE support.

Bits (7:0) of this word define the maximum number of sectors per block that the device supports for READ/WRITE MULTIPLE commands.  If the serial interface is implemented, this field shall be set to 16 or less.

        - Word 48: Reserved.

        - Word (50:49): Capabilities

Bits (15:14) of word 49 are reserved for use in the IDENTIFY PACKET DEVICE command data.

Bit 13 of word 49 is used to determine whether a device uses the Standby timer values as defined in this standard.  Table 18 specifies the Standby timer values used by the device if bit 13 is set to one.  If bit 13 is cleared to zero, the timer values shall be vendor specific.

Bit 12 of word 49 is reserved for use in the IDENTIFY PACKET DEVICE command data.

Bit 11 of word 49 indicates whether a device supports IORDY.  If this bit is set to one, then the device supports IORDY operation.  All devices except CFA and PCMCIA devices shall support PIO mode 3 or higher, shall support IORDY, and shall set this bit to one.  If the serial interface is implemented, this bit shall be set to one.

Bit 10 of word 49 is used to indicate a device’s ability to enable or disable the use of IORDY.  If this bit is set to one, then the device supports the disabling of IORDY.  Disabling and enabling of IORDY is accomplished using the SET FEATURES command.  If the serial interface is implemented, this bit shall be set to one.

Bit 9 of word 49 shall be set to one to indicate that an LBA transition is supported.

Bits 8 of word 49 Shall be set to one to indicate that DMA is supported.  For devices not implementing the CompactFlash feature set this bit shall be set to one.

Bits (7:0) of word 49 are retired.

Bit 15 of word 50 shall be cleared to zero to indicate that the contents of word 50 are valid.

Bit 14 of word 50 shall be set to one to indicate that the contents of word 50 are valid.

Bits (13:2) of word 50 are reserved.

Bit 1 of word 50 is obsolete.

Bit 0 of word 50 set to one indicates that the device has a minimum Standby timer value that is device specific.

        - Words (52:51): Obsolete

        - Word 53: Field validity

Bit 0 of word 53 is obsolete.

If bit 1 of word 53 is set to one, the values reported in words (70:64) are valid.  If this bit is cleared to zero, the values reported in words (70:64) are not valid.  All devices except CFA and PCMCIA devices shall support PIO mode 3 or above and shall set bit 1 of word 53 to one and support the fields contained in words (70:64).  If the serial interface is implemented, this bit shall be set to one.

If the device supports Ultra DMA and the values reported in word 88 are valid, then bit 2 of word 53 shall be set to one.  If the device does not support Ultra DMA and the values reported in word 88 are not valid, then this bit is cleared to zero.  If the serial interface is implemented, this bit shall be set to one.

        - Word (58:54): Obsolete

        - Word 59: Multiple sector setting

If bit 8 is set to one, bits (7:0) reflect the number of sectors currently set to transfer on a READ/WRITE MULTIPLE command.  This field may default to the preferred value for the device (See 7.52).

        - Word (61:60): Total number of user addressable sectors

This field contains a value that is one greater than the maximum user accessable logical block address (See 4.2).  The maximum value that shall be placed in this field is 0FFFFFFFh.

        - Word 62: Obsolete

        - Word 63: Multiword DMA transfer

Word 63 identifies the Multiword DMA transfer modes supported by the device and indicates the mode that is currently selected.  Only one DMA mode shall be selected at any given time.  If an Ultra DMA mode is enabled, then no Multiword DMA mode shall be enabled.  If a Multiword DMA mode is enabled then no Ultra DMA mode shall be enabled.

        - Reserved

Bits (15:11) of word 63 are reserved.

        - Multiword DMA mode 2 selected

If bit 10 of word 63 is set to one, then Multiword DMA mode 2 is selected.  If this bit is cleared to zero, then Multiword DMA mode 2 is not selected.  If bit 9 is set to one or if bit 8 is set to one, then this bit shall be cleared to zero.

        - Multiword DMA mode 1 selected

If bit 9 of word 63 is set to one, then Multiword DMA mode 1 is selected.  If this bit is cleared to zero then Multiword DMA mode 1 is not selected.  If bit 10 is set to one or if bit 8 is set to one, then this bit shall be cleared to zero.

        - Multiword DMA mode 0 selected

If bit 8 of word 63 is set to one, then Multiword DMA mode 0 is selected.  If this bit is cleared to zero then Multiword DMA mode 0 is not selected.  If bit 10 is set to one or if bit 9 is set to one, then this bit shall be cleared to zero.

        - Reserved

Bits (7:3) of word 63 are reserved.

        - Multiword DMA mode 2 supported

If bit 2 of word 63 is set to one, then Multiword DMA modes 2 and below are supported.  If this bit is cleared to zero, then Multiword DMA mode 2 is not supported.  If Multiword DMA mode 2 is supported, then Multiword DMA modes 1 and 0 shall also be supported.  If this bit is set to one, bits (1:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Multiword DMA mode 1 supported

If bit 1 of word 63 is set to one, then Multiword DMA modes 1 and below are supported.  If this bit is cleared to zero, then Multiword DMA mode 1 is not supported.  If Multiword DMA mode 1 is supported, then Multiword DMA mode 0 shall also be supported.  If this bit is set to one, bit 0 shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Multiword DMA mode 0 supported

If bit 0 of word 63 is set to one, then Multiword DMA mode 0 is supported.  If the serial interface is implemented, this bit shall be set to one.

        - Word 64: PIO transfer modes supported

Bits (7:0) of word 64 of the IDENTIFY DEVICE data is defined as the  PIO data and register transfer supported field.  If this field is supported, bit 1 of word 53 shall be set to one.  This field is bit significant.  Any number of bits may be set to one in this field by the device to indicate the PIO modes the device is capable of supporting.

Of these bits, bits (7:2) are Reserved for future PIO modes.  Bit 0, if set to one, indicates that the device supports PIO mode 3.  All devices except CFA and PCMCIA devices shall support PIO mode 3 and shall set bit 0 to one.  Bit 1, if set to one, indicates that the device supports PIO mode 4.  If the serial interface is implemented, bits (1:0) shall be set to one.

        - Word 65: Minimum Multiword DMA transfer cycle time per word

Word 65 of the parameter information of the IDENTIFY DEVICE command data is defined as the minimum Multiword DMA transfer cycle time per word.  This field defines, in nanoseconds, the minimum cycle time that the device supports when performing Multiword DMA transfers on a per word basis.  If the serial interface is implemented, this value shall be set to indicate 120 ns.

If this field is supported, bit 1 of word 53 shall be set to one.  Any device that supports Multiword DMA mode 1 or above shall support this field, and the value in word 65 shall not be less than the minimum cycle time for the fastest DMA mode supported by the device.

If bit 1 of word 53 is set to one because a device supports a field in words (70:64) other than this field and the device does not support this field, the device shall return a value of zero in this field.

        - Word 66: Device recommended Multiword DMA cycle time

Word 66 of the parameter information of the IDENTIFY DEVICE command data is defined as the device recommended Multiword DMA transfer cycle time.  This field defines, in nanoseconds, the minimum cycle time per word during a single sector host transfer while performing a multiple sector READ DMA or WRITE DMA command for any location on the media under nominal conditions.  If a host runs at a faster cycle rate by operating at a cycle time of less than this value, the device may negate DMARQ for flow control.  The rate at which DMARQ is negated could result in reduced throughput despite the faster cycle rate.  Transfer at this rate does not ensure that flow control will not be used, but implies that higher performance may result.  If the serial interface is implemented, this value shall be set to indicate 120 ns.

If this field is supported, bit 1 of word 53 shall be set to one.  Any device that supports Multiword DMA mode 1 or above shall support this field, and the value in word 66 shall not be less than the value in word 65.

If bit 1 of word 53 is set to one because a device supports a field in words (70:64) other than this field and the device does not support this field, the device shall return a value of zero in this field.

        - Word 67: Minimum PIO transfer cycle time without IORDY flow control

Word 67 of the parameter information of the IDENTIFY DEVICE command data is defined as the minimum PIO transfer without IORDY flow control cycle time.  This field defines, in nanoseconds, the minimum cycle time that, if used by the host, the device guarantees data integrity during the transfer without utilization of IORDY flow control.  If the serial interface is implemented, this value shall be set to indicate 120 ns.

If this field is supported, Bit 1 of word 53 shall be set to one.

Any device that supports PIO mode 3 or above shall support this field, and the value in word 67 shall not be less than the value reported in word 68.

If bit 1 of word 53 is set to one because a device supports a field in words (70:64) other than this field and the device does not support this field, the device shall return a value of zero in this field.

        - Word 68: Minimum PIO transfer cycle time with IORDY flow control

Word 68 of the parameter information of the IDENTIFY DEVICE command data is defined as the minimum PIO transfer with IORDY flow control cycle time.  This field defines, in nanoseconds, the minimum cycle time that the device supports while performing data transfers while utilizing IORDY flow control.  If the serial interface is implemented, this value shall be set to indicate 120 ns.

If this field is supported, Bit 1 of word 53 shall be set to one.

All devices except CFA and PCMCIA devices shall support PIO mode 3 and shall support this field, and the value in word 68 shall be the fastest defined PIO mode supported by the device.  The maximum value reported in this field shall be 180 to indicate support for PIO mode 3 or above.

If bit 1 of word 53 is set to one because a device supports a field in words (70:64) other than this field and the device does not support this field, the device shall return a value of zero in this field.

        - Words (74:69): Reserved

        - Word 75: Queue depth

Bits (4:0) of word 75 indicate the maximum queue depth supported by the device.  The queue depth includes all commands for which command acceptance has occurred and command completion has not occurred.  The value in this field equals (maximum queue depth - 1), e.g., a value of zero indicates a queue depth of one, a value of 31 indicates a queue depth of 32.  If bit 1 of word 83 is cleared to zero indicating that the device does not support READ/WRITE DMA QUEUED commands, the value in this field shall be zero.  A device may support READ/WRITE DMA QUEUED commands to provide overlap only (i.e., queuing not supported), in this case, bit 1 of word 83 shall be set to one and the queue depth shall be set to zero.  Support of this word is mandatory if the Queuing feature set is supported.

        - Words (79:76): Reserved for Serial ATA

        - Word 80: Major version number

If not 0000h or FFFFh, the device claims compliance with the major version(s) as indicated by bits (6:3) being set to one.  Values other than 0000h and FFFFh are bit significant.  Since ATA standards maintain downward compatibility, a device may set more than one bit.

        - Word 81: Minor version number

If an implementor claims that the revision of the standard they used to guide their implementation does not need to be reported or if the implementation was based upon a standard prior to the ATA-3 standard, word 81 shall be 0000h or FFFFh.

Table 16 defines the value that may optionally be reported in word 81 to indicate the revision of the standard that guided the implementation.

Table 16 - Minor version number16

| Value | Minor revision |
| --- | --- |
| 0001h | Obsolete |
| 0002h | Obsolete |
| 0003h | Obsolete |
| 0004h | Obsolete |
| 0005h | Obsolete |
| 0006h | Obsolete |
| 0007h | Obsolete |
| 0008h | Obsolete |
| 0009h | Obsolete |
| 000Ah | Obsolete |
| 000Bh | Obsolete |
| 000Ch | Obsolete |
| 000Dh | ATA/ATAPI-4 X3T13 1153D revision 6 |
| 000Eh | ATA/ATAPI-4 T13 1153D revision 13 |
| 000Fh | ATA/ATAPI-4 X3T13 1153D revision 7 |
| 0010h | ATA/ATAPI-4 T13 1153D revision 18 |
| 0011h | ATA/ATAPI-4 T13 1153D revision 15 |
| 0012h | ATA/ATAPI-4 published, ANSI INCITS 317-1998 |
| 0013h | ATA/ATAPI-5 T13 1321D revision 3 |
| 0014h | ATA/ATAPI-4 T13 1153D revision 14 |
| 0015h | ATA/ATAPI-5 T13 1321D revision 1 |
| 0016h | ATA/ATAPI-5 published, ANSI INCITS 340-2000 |
| 0017h | ATA/ATAPI-4 T13 1153D revision 17 |
| 0018h | ATA/ATAPI-6 T13 1410D revision 0 |
| 0019h | ATA/ATAPI-6 T13 1410D revision 3a |
| 001Ah | ATA/ATAPI-7 T13 1532D revision 1 |
| 001Bh | ATA/ATAPI-6 T13 1410D revision 2 |
| 001Ch | ATA/ATAPI-6 T13 1410D revision 1 |
| 001Dh | ATA/ATAPI-7 published ANSI INCITS 397-2005. |
| 001Eh | ATA/ATAPI-7 T13 1532D revision 0 |
| 001Fh | Reserved |
| 0020h | Reserved |
| 0021h | ATA/ATAPI-7 T13 1532D revision 4a |
| 0022h | ATA/ATAPI-6 published, ANSI INCITS 361-2002 |
| 0023h-FFFFh | Reserved |

        - Words (84:82): Features/command sets supported

Words (84:82) shall indicate features/command sets supported.  If a defined bit is cleared to zero, the indicated features/command set is not supported.  If bit 14 of word 83 is set to one and bit 15 of word 83 is cleared to zero, the contents of words (83:82) contain valid support information.  If not, support information is not valid in these words.  If bit 14 of word 84 is set to one and bit 15 of word 84 is cleared to zero, the contents of word 84 contains valid support information.  If not, support information is not valid in this word.

If bit 0 of word 82 is set to one, the SMART feature set is supported.

If bit 1 of word 82 is set to one, the Security Mode feature set is supported.

If bit 2 of word 82 is set to one, the Removable Media feature set is supported.

Bit 3 of word 82 shall be set to one indicating the mandatory Power Management feature set is supported.

Bit 4 of word 82 shall be cleared to zero to indicate that the PACKET Command feature set is not supported.

If bit 5 of word 82 is set to one, write cache is supported.

If bit 6 of word 82 is set to one, look-ahead is supported.

If bit 7 of word 82 is set to one, release interrupt is supported.

If bit 8 of word 82 is set to one, SERVICE interrupt is supported.

If bit 9 of word 82 is set to one, the DEVICE RESET command is supported.

If bit 10 of word 82 is set to one, the Host Protected Area feature set is supported.

Bit 11 of word 82 is obsolete.

If bit 12 of word 82 is set to one, the device supports the WRITE BUFFER command.

If bit 13 of word 82 is set to one, the device supports the READ BUFFER command.

If bit 14 of word 82 is set to one, the device supports the NOP command.

Bit 15 of word 82 is obsolete.

If bit 0 of word 83 is set to one, the device supports the DOWNLOAD MICROCODE command.

If bit 1 of word 83 is set to one, the device supports the READ DMA QUEUED and WRITE DMA QUEUED commands.

If bit 2 of word 83 is set to one, the device supports the CFA feature set.

If bit 3 of word 83 is set to one, the device supports the Advanced Power Management feature set.

If bit 4 of word 83 is set to one, the device supports the Removable Media Status feature set.

If bit 5 of word 83 is set to one, the device supports the Power-Up In Standby feature set.

If bit 6 of word 83 is set to one, the device requires the SET FEATURES subcommand to spin-up after power-up if the Power-Up In Standby feature set is enabled (See 7.49.11).

Bit 7 is defined in Address Offset Reserved Area Boot, INCITS TR27:2001.

If bit 8 of word 83 is set to one, the device supports the SET MAX security extension.

If bit 9 of word 83 is set to one, the device supports the Automatic Acoustic Management feature set.

If bit 10 of word 83 is set to one, the 48-bit Address feature set is supported.

If bit 11 of word 83 is set to one, the device supports the Device Configuration Overlay feature set.

Bit 12 of word 83 shall be set to one indicating the device supports the mandatory FLUSH CACHE command.

If bit 13 of word 83 is set to one, the device supports the FLUSH CACHE EXT command.

If bit 0 of word 84 is set to one, the device supports SMART error logging.

If bit 1 of word 84 is set to one, the device supports SMART self-test.

If bit 2 of word 84 is set to one, the device supports the media serial number field words (205:176).

If bit 3 of word 84 is set to one, the device supports the Media Card Pass Through Command feature set.

If bit 4 of word 84 is set to one, the device supports the Streaming feature set.

If bit 5 of word 84 is set to one, the device supports the General Purpose Logging feature set.

If bit 6 of word 84 is set to one, the device supports the WRITE DMA FUA EXT and WRITE MULTIPLE FUA EXT commands.

If bit 7 of word 84 is set to one, the device supports the WRITE DMA QUEUED FUA EXT command.

If bit 8 of word 84 is set to one, the device supports a world wide name.

If bit 9 of word 84 is set to one, the device supports the URG bit for READ STREAM DMA EXT and READ STREAM EXT commands.

If bit 10 of word 84 is set to one, the device supports the URG bit for WRITE STREAM DMA EXT and WRITE STREAM EXTcommands.

Bit 11 of word 84 is reserved for technical report-

Bit 12 of word 84  is reserved for technical report-

If bit 13 of word 84 is set to one, the device supports IDLE IMMEDIATE with UNLOAD FEATURE.

        - Words (87:85): Features/command sets enabled

Words (87:85) shall indicate features/command sets enabled.  If a defined bit is cleared to zero, the indicated features/command set is not enabled.  If a supported features/command set is supported and cannot be disabled, it is defined as supported and the bit shall be set to one.  If bit 14 of word 87 is set to one and bit 15 of word 87 is cleared to zero, the contents of words (87:85) contain valid information.  If not, information is not valid in these words.

If bit 0 of word 85 is set to one, the SMART feature set has been enabled via the SMART ENABLE OPERATIONS command.  If bit 0 of word 85 is cleared to zero, the SMART feature set has been disabled via the SMART DISABLE OPERATIONS command.

If bit 1 of word 85 is set to one, the Security Mode feature set has been enabled via the SECURITY SET PASSWORD command.  If bit 1 of word 85 is cleared to zero, the Security Mode feature set has been disabled via the SECURITY DISABLE PASSWORD command.

If bit 2 of word 85 is set to one, the Removable Media feature set is supported.

Bit 3 of word 85 shall be set to one indicating the mandatory Power Management feature set is supported.

Bit 4 of word 85 shall be cleared to zero to indicate that the PACKET Command feature set is not supported.

If bit 5 of word 85 is set to one, write cache has been enabled via the SET FEATURES command (See 7.49.4).  If bit 5 of word 85 is cleared to zero, write cache has been disabled via the SET FEATURES command.

If bit 6 of word 85 is set to one, look-ahead has been enabled via the SET FEATURES command (See 7.49.15).  If bit 6 of word 85 is cleared to zero, look-ahead has been disabled via the SET FEATURES command.

If bit 7 of word 85 is set to one, release interrupt has been enabled via the SET FEATURES command (See 7.49.16).  If bit 7 of word 85 is cleared to zero, release interrupt has been disabled via the SET FEATURES command.

If bit 8 of word 85 is set to one, SERVICE interrupt has been enabled via the SET FEATURES command (See 7.49.17).  If bit 8 of word 85 is cleared to zero, SERVICE interrupt has been disabled via the SET FEATURES command.

If bit 9 of word 85 is set to one, the DEVICE RESET command is supported.

If bit 10 of word 85 is set to one, the Host Protected Area feature set is supported.

Bit 11 of word 85 is obsolete.

If bit 12 of word 85 is set to one, the device supports the WRITE BUFFER command.

If bit 13 of word 85 is set to one, the device supports the READ BUFFER command.

If bit 14 of word 85 is set to one, the device supports the NOP command.

Bit 15 of word 85 is obsolete.

If bit 0 of word 86 is set to one, the device supports the DOWNLOAD MICROCODE command.

If bit 1 of word 86 is set to one, the device supports the READ DMA QUEUED and WRITE DMA QUEUED commands.

If bit 2 of word 86 is set to one, the device supports the CFA feature set.

If bit 3 of word 86 is set to one, the Advanced Power Management feature set has been enabled via the SET FEATURES command.  If bit 3 of word 86 is cleared to zero, the Advanced Power Management feature set has been disabled via the SET FEATURES command.

If bit 4 of word 86 is set to one, the Removable Media Status feature set has been enabled via the SET FEATURES command.  If bit 4 of word 86 is cleared to zero, the Removable Media Status feature set has been disabled via the SET FEATURES command.

If bit 5 of word 86 is set to one, the Power-Up In Standby feature set has been enabled via the SET FEATURES command (See 7.49.7).  If bit 5 of word 86 is cleared to zero, the Power-Up In Standby feature set has been disabled via the SET FEATURES command

If bit 6 of word 86 is set to one, the device requires the SET FEATURES subcommand to spin-up after power-up (See 7.49.11).

Bit 7 of word 86 is defined in Address Offset Reserved Area Boot, INCITS TR27:2001.

If bit 8 of word 86 is set to one, the device has had the SET MAX security extension enabled via a SET MAX SET PASSWORD command.

If bit 9 of word 86 is set to one, the device has had the Automatic Acoustic Management feature set enabled via a SET FEATURES command and the value in word 94 is valid.

If bit 10 of word 86 is set to one, the 48-bit Address feature set is supported.

If bit 11 of word 86 is set to one, the device supports the Device Configuration Overlay feature set.

Bit 12 of word 86 shall be  set to oneindicating the device supports the mandatory FLUSH CACHE command.

If bit 13 of word 86 is set to one, the device supports the FLUSH CACHE EXT command.

If bit 0 of word 87 is set to one, the device supports SMART error logging.

If bit 1 of word 87 is set to one, the device supports SMART self-test.

If bit 2 of word 87 is set to one, the media serial number field in words (205:176) is valid.  This bit shall be cleared to zero if the media does not contain a valid serial number or if no media is present.

If bit 3 of word 87 is set to one, the Media Card Pass Through feature set has been enabled.

If bit 4 of word 87 is set to one, a valid CONFIGURE STREAM command has been executed.

If bit 5 of word 87 is set to one, the device supports the General Purpose Logging feature set.

If bit 6 of word 87 is set to one, the device supports the WRITE DMA FUA EXT and WRITE MULTIPLE FUA EXT commands.

If bit 7 of word 87 is set to one, the device supports the WRITE DMA QUEUED FUA EXT command.

If bit 8 of word 87 is set to one, the device supports a world wide name.

If bit 9 of word 87 is set to one, the device supports the URG bit for READ STREAM DMA EXT and READ STREAM EXT commands.

If bit 10 of word 87 is set to one, the device supports the URG bit for WRITE STREAM DMA EXT and WRITE STREAM EXT commands.

Bit 11 of word 87 is reserved for technical report.

Bit 12 of word 87 is reserved for technical report.

If bit 13 of word 87 is set to one, the device supports IDLE IMMEDIATE with UNLOAD FEATURE.

        - Word 88: Ultra DMA modes

Word 88 identifies the Ultra DMA transfer modes supported by the device and indicates the mode that is currently selected.  Only one DMA mode shall be selected at any given time.  If an Ultra DMA mode is selected, then no Multiword DMA mode shall be selected.  If a Multiword DMA mode is selected, then no Ultra DMA mode shall be selected.  Support of this word is mandatory if Ultra DMA is supported.

        - Reserved

Bit (15) of word 88 is reserved.

        - Ultra DMA mode 6 selected

If bit 14 of word 88 is set to one, then Ultra DMA mode 6 is selected.  If this bit is cleared to zero, then Ultra DMA mode 6 is not selected.  If bit 13 or bit 12 or bit 11 or bit 10 or bit 9 or bit 8 is set to one, then this bit shall be cleared to zero.

        - Ultra DMA mode 5 selected

If bit 13 of word 88 is set to one, then Ultra DMA mode 5 is selected.  If this bit is cleared to zero, then Ultra DMA mode 5 is not selected.  If bit 12 or bit 11 or bit 10 or bit 9 or bit 8 is set to one, then this bit shall be cleared to zero.

        - Ultra DMA mode 4 selected

If bit 12 of word 88 is set to one, then Ultra DMA mode 4 is selected.  If this bit is cleared to zero, then Ultra DMA mode 4 is not selected.  If bit 13 or 11 or bit 10 or bit 9 or bit 8 is set to one, then this bit shall be cleared to zero.

        - Ultra DMA mode 3 selected

If bit 11 of word 88 is set to one, then Ultra DMA mode 3 is selected.  If this bit is cleared to zero, then Ultra DMA mode 3 is not selected.  If bit 13 or 12 or bit 10 or bit 9 or bit 8 is set to one, then this bit shall be cleared to zero.

        - Ultra DMA mode 2 selected

If bit 10 of word 88 is set to one, then Ultra DMA mode 2 is selected.  If this bit is cleared to zero, then Ultra DMA mode 2 is not selected.  If bit 13 or 12 or bit 11 or bit 9 or bit 8 is set to one, then this bit shall be cleared to zero.

        - Ultra DMA mode 1 selected

If bit 9 of word 88 is set to one, then Ultra DMA mode 1 is selected.  If this bit is cleared to zero then Ultra DMA mode 1 is not selected.  If bit 13 or 12 or bit 11 or bit 10 or bit 8 is set to one, then this bit shall be cleared to zero.

        - Ultra DMA mode 0 selected

If bit 8 of word 88 is set to one, then Ultra DMA mode 0 is selected.  If this bit is cleared to zero then Ultra DMA mode 0 is not selected.  If bit 13 or 12 or bit 11 or bit 10 or bit 9 is set to one, then this bit shall be cleared to zero.

        - Reserved

Bit (7) of word 88 are reserved.

        - Ultra DMA mode 6 supported

If bit 6 of word 88 is set to one, then Ultra DMA modes 6 and below are supported.  If this bit is cleared to zero, then Ultra DMA mode 6 is not supported.  If Ultra DMA mode 6 is supported, then Ultra DMA modes 5, 4, 3, 2, 1 and 0 shall also be supported.  If this bit is set to one, then bits (5:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Ultra DMA mode 5 supported

If bit 5 of word 88 is set to one, then Ultra DMA modes 5 and below are supported.  If this bit is cleared to zero, then Ultra DMA mode 5 is not supported.  If Ultra DMA mode 5 is supported, then Ultra DMA modes 4, 3, 2, 1 and 0 shall also be supported.  If this bit is set to one, then bits (4:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Ultra DMA mode 4 supported

If bit 4 of word 88 is set to one, then Ultra DMA modes 4 and below are supported.  If this bit is cleared to zero, then Ultra DMA mode 4 is not supported.  If Ultra DMA mode 4 is supported, then Ultra DMA modes 3, 2, 1 and 0 shall also be supported.  If this bit is set to one, then bits (3:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Ultra DMA mode 3 supported

If bit 3 of word 88 is set to one, then Ultra DMA modes 3 and below are supported.  If this bit is cleared to zero, then Ultra DMA mode 3 is not supported.  If Ultra DMA mode 3 is supported, then Ultra DMA modes 2, 1 and 0 shall also be supported.  If this bit is set to one, then bits (2:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Ultra DMA mode 2 supported

If bit 2 of word 88 is set to one, then Ultra DMA modes 2 and below are supported.  If this bit is cleared to zero, then Ultra DMA mode 2 is not supported.  If Ultra DMA mode 2 is supported, then Ultra DMA modes 1 and 0 shall also be supported.  If this bit is set to one, bits (1:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Ultra DMA mode 1 supported

If bit 1 of word 88 is set to one, then Ultra DMA modes 1 and below are supported.  If this bit is cleared to zero, then Ultra DMA mode 1 is not supported.  If Ultra DMA mode 1 is supported, then Ultra DMA mode 0 shall also be supported.  If this bit is set to one, bit 0 shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

        - Ultra DMA mode 0 supported

If bit 0 of word 88 is set to one, then Ultra DMA mode 0 is supported.  If this bit is cleared to zero, then Ultra DMA is not supported.  If the serial interface is implemented, this bit shall be set to one.

        - Word 89: Time required for Security erase unit completion

Word 89 specifies the time required for the SECURITY ERASE UNIT command to complete.  Support of this word is mandatory if the Security feature set is supported.

| Value | Time |
| --- | --- |
| 0 | Value not specified |
| 1-254 | (Value2) minutes |
| 255 | >508 minutes |

        - Word 90: Time required for Enhanced security erase unit completion

Word 90 specifies the time required for the ENHANCED SECURITY ERASE UNIT command to complete.  Support of this word is mandatory if support of the Enhanced Security feature set is supported.

| Value | Time |
| --- | --- |
| 0 | Value not specified |
| 1-254 | (Value2) minutes |
| 255 | >508 minutes |

        - Word 91: Advanced power management level value

Bits (7:0) of word 91 contain the current Advanced Power Management level setting.  Support of this word is mandatory if advanced power management is supported.

        - Word 92: Master Password Revision Code

Word 92 contains the value of the Master Password Revision Code set when the Master Password was last changed.  Valid values are 0001h through FFFEh.  A value of 0000h or FFFFh indicates that the Master Password Revision is not supported.  Support of this word is mandatory if the Security feature set is supported.

        - Word 93: Hardware configuration test results

If bit 14 of word 93 is set to one and bit 15 of word 93 is cleared to zero, the content of word 93 contains valid information.  During hardware reset execution, Device 0 shall clear bits (12:8) of this word to zero and shall set bits (7:0) of the word as indicated to show the result of the hardware reset execution.  During hardware reset execution, Device 1 shall clear bits (7:0) of this word to zero and shall set bits (12:8) as indicated to show the result of the hardware reset execution.  Support of bits (15:13) are mandatory.  Support of bits (12:0) is optional.

Bit 13 shall be set or cleared by the selected device to indicate whether the device detected CBLID- above VIH or below VIL at any time during execution of each IDENTIFY DEVICE routine after receiving the command from the host but before returning data to the host.  This test may be repeated as desired by the device during command execution (See Volume 2, annex A).

If the serial interface is implemented, word 93 shall be set to the value 0000h.

        - Word 94: Current automatic acoustic management value

Bits (15:8) contain the device vendor’s recommended acoustic management level (See Table 45 for an enumeration of all of the possible acoustic management levels).  If the host desires the drive to perform with highest performance, it should set the automatic acoustic management level to Feh.  If the OEM host desires the vendor’s recommended acoustic management level as defined by the device’s vendor, the host should set the automatic acoustic management level to the value returned to the host in these 8 bits of the IDENTIFY DEVICE data.  The use of this setting may not provide the lowest acoustics, or the best tradeoff of acoustics and performance, in all configurations.  Support of this word is mandatory if the Acoustic Management feature set is supported.

Bits (7:0) contain the current automatic acoustic management level.  If the Automatic Acoustic Management feature set is supported by the device, but the level has not been set by the host, this byte shall contain the drive’s default setting.  If the Automatic Acoustic Management feature set is not supported by the device, the value of this byte shall be zero.

        - Word 95: Stream Minimum Request Size

Number of sectors that provides optimum performance in a streaming environment.  This number shall be a power of two, with a minimum of eight sectors (4096 bytes).  The starting LBA value for each streaming command should be evenly divisible by this request size.

        - Word 96: Streaming Transfer Time -- DMA

Word 96 defines the Streaming Transfer Time for DMA mode.  The worst-case sustainable transfer time per sector for the device is calculated as follows:

Streaming Transfer Time = (word 96)  (words (99:98) / 65536)

The content of IDENTIFY DEVICE data word 96 may be affected by the host issuing a SET FEATURES subcommand 43h (Typical Host Interface Sector Time for DMA mode).  Because of this effect, an IDENTIFY DEVICE command shall be issued after a SET FEATURES command that may affect these words.  If the Streaming Feature Set is not supported by the device, the content of word 96 shall be zero.

        - Word 97: Streaming Access Latency - DMA and PIO

Word 97 defines the Streaming Access Latency for DMA and PIO mode.  The worst-case access latency of the device for a streaming command is calculated as follows:

Access Latency = (word 97)  (words (99:98) / 256)

The content of IDENTIFY DEVICE data word 97 may be affected by the host issuing a SET FEATURES subcommand 42h or C2h (Automatic Acoustic Management).  Because of this effect, an IDENTIFY DEVICE command shall be issued after a SET FEATURES command that may affect these words.  If the Streaming Feature Set is not supported by the device, the content of word 97 shall be zero.

        - Words (99:98): Streaming Performance Granularity

These words define the fixed unit of time that is used in IDENTIFY DEVICE data words (97:96) and (104), and SET FEATURES subcommand 43h, and in the Streaming Performance Parameters log, which is accessed by use of the READ LOG EXT command, and in the Command Completion Time Limit that is passed in streaming commands.  The unit of time for this parameter shall be in microseconds, e.g., a value of 10000 indicates 10 milliseconds.  If yy was returned by the drive for this parameter, then

- the Command Completion Time Limit in the Features register for a streaming command shall be yy microseconds.

- the Streaming Transfer Time shall be ( (word 96)  (yy/65536) ) microseconds, ( (word 104)  (yy/65536) ) microseconds, or ( (a Sector Time array entriy in the Streaming Performance Parameters log)  (yy/65536) ) microseconds.

- The Streaming Access Latency shall be ((word 97)  (yy/256)) microseconds, or ((an Access Time array entries in the Streaming Performance Parameters log)  (yy/256)) microseconds.

- taking these units into account, the host may calculate the estimated time for a streaming command of size S sectors as ( ( word 96  S / 65536) + (word 97  / 256 ) )  yy microseconds for DMA mode.

- taking these units into account, the host may calculate the estimated time for a streaming command of size S sectors as ( ( word 104  S / 65536) + (word 97  / 256 ) )  yy microseconds for PIO mode.

The value of the Streaming Performance Granularity is vendor specific and fixed for a device.

        - Words (103:100): Maximum user LBA for 48-bit Address feature set

Words (103:100) contain a value that is one greater than the maximum LBA in user accessable space when the 48-bit Addressing feature set is supported.  The maximum value that shall be placed in this field is 0000FFFFFFFFFFFFh.  Support of these words is mandatory if the 48-bit Address feature set is supported.

        - Word 104: Streaming Transfer Time - PIO

Word 104 defines the Streaming Transfer Time for PIO mode.  The worst-case sustainable transfer time per sector for the device is calculated as follows:

Streaming Transfer Time = (word 104)  (words (99:98) / 65536)

The content of IDENTIFY DEVICE data word 104 may be affected by the host issuing a SET FEATURES subcommand 43h (Typical Host Interface Sector Time for PIO mode).  Because of this effect, an IDENTIFY DEVICE command shall be issued after a SET FEATURES command that may affect these words.  If the Streaming Feature Set is not supported by the device, the content of word 104 shall be zero.

        - Word 106: Physical sector size / Logical Sector Size

If bit 14 of word 106 is set to one and bit 15 of word 106 is cleared to zero, the contents of word 106 contain valid information.  If not, information is not valid in this word.

Bit 13 of word 106 shall be set to one to indicate that the device has more than one logical sector per physical sector.

Bit 12 of word 106 shall be set to 1 to indicate that the device has been formatted with a logical sector size larger than 256 words.  Bit 12 of word 106 shall be cleared to 0 to indicate that words 117-118 are invalid and that the logical sector size is 256 words.

Bits (11:4) of word 106 are reserved.

Bits (3:0) of word 106 indicate the size of the device physical sectors in power of two logical sectors.

Examples:

Bits (3:0): 0 = 20 = 1 logical sector per physical sector

Bits (3:0): 1 = 21 = 2 logical sector per physical sector

Bits (3:0): 2 = 22 = 4 logical sector per physical sector

Bits (3:0): 3 = 23 = 8 logical sector per physical sector

        - Word 107: Inter-seek delay for ISO 7779 standard acoustic testing

Word 107 is defined as the manufacturer’s recommended time delay between seeks during ISO-7779 standard acoustic testing in microseconds (ISO 7779 value tD.  See ISO 7779:1999 (E) Clause C.9 Equipment Category: Disk units and storage subsystems.

        - Words (111:108): World wide name

Words 111-108 shall contain the optional value of the world wide name (WWN) for the device.

Word 108 bits 15-12 shall contain 5h, indicating that the naming authority is IEEE.  All other values are reserved.

Words 108 bits 11-0 and word 109 bits 15-4 shall contain the Organization Unique Identifier (OUI) for the device manufacturer.  The OUI shall be assigned by the IEEE/RAC as specified by ISO/IEC 13213:1994 (See 3.1.80).

The identifier may be obtained from:

Institute of Electrical and Electronic Engineers, Inc.

Registration Authority Committee

445 Hoes Lane

Piscataway, NJ 08855-1331

Word 109 bits 3-0, word 110, and word 111 shall contain a value assigned by the vendor that is unique for the OUI domain.

        - Words (115:112): Reserved for a 128-bit world wide name

        - Word 116: Reserved for TLC  technical report.

This field is described in Time-Limited Commands (TLC) INCITS TR-37-2004

        - Words 117-118: Logical Sector Size

Words 117,118 indicate the size of device logical sectors in words.  The value of words 117,118 shall be equal to or greater than 256.  The value in words 117,118 shall be valid when word 106 bit 12 is set to 1.  All logical sectors on a device shall be 117,118 words long.

        - Words (126:119): Reserved

        - Word 127: Removable Media Status Notification feature set support

If bit 0 of word 127 is set to one and bit 1 of word 127 is cleared to zero, the device supports the Removable Media Status Notification feature set.  Bits (15:2) shall be cleared to zero.  Support of this word is mandatory if the Removable Media Status Notification feature set is supported.

        - Word 128: Security status

Support of this word is mandatory if the Security feature set is supported.

Bit 8 of word 128 indicates the security level.  If security mode is enabled and the security level is high, bit 8 shall be cleared to zero.  If security mode is enabled and the security level is maximum, bit 8 shall be set to one.  When security mode is disabled, bit 8 shall be cleared to zero.

Bit 5 of word 128 indicates the Enhanced security erase unit feature is supported.  If bit 5 is set to one, the Enhanced security erase unit feature set is supported.

Bit 4 of word 128 indicates that the security count has expired.  If bit 4 is set to one, the security count is expired and SECURITY UNLOCK and SECURITY ERASE UNIT are command aborted until a power-on reset or hardware reset.

Bit 3 of word 128 indicates security frozen.  If bit 3 is set to one, the security is frozen.

Bit 2 of word 128 indicates security locked.  If bit 2 is set to one, the security is locked.

Bit 1 of word 128 indicates security enabled.  If bit 1 is set to one, the security is enabled.

Bit 0 of word 128 indicates the Security Mode feature set supported.  If bit 0 is set to one, security is supported.

        - Words (159:129): Vendor specific.

        - Word 160: CFA power mode

Word 160 indicates the presence and status of a CFA feature set device that supports CFA Power Mode 1.  Support of this word is mandatory if CFA Power Mode 1 is supported.

If bit 13 of word 160 is set to one then the device shall be in CFA Power Mode 1 to perform one or more commands implemented by the device.

If bit 12 of word 160 is set to one the device is in CFA Power Mode 0 (See 7.49.9).

Bits (11:0) indicate the maximum average RMS current in Milliamperes required during 3.3V or 5V device operation in CFA Power Mode 1.

        - Words (175:161): Reserved for assignment by the CompactFlash Association

        - Words (205:176): Current media serial number

Words (205:176) contain the current media serial number.  Serial numbers shall consist of 60 bytes.  The first 40 bytes shall indicate the media serial number and the remaining 20 bytes shall indicate the media manufacturer.

For removable ATA devices (e.g., flash media with native ATA interfaces) that do not support removable media, the first 20 words of this field shall be the same as words (46:27) of the IDENTIFY DEVICE data and the next ten words shall be the same as words (19:10) of the IDENTIFY DEVICE response.

        - Words (254:206): Reserved.

        - Word 255: Integrity word

The use of this word is optional.  If bits (7:0) of this word contain the signature A5h, bits (15:8) contain the data structure checksum.  The data structure checksum is the two’s complement of the sum of all bytes in words (254:0) and the byte consisting of bits (7:0) in word 255.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all 512 bytes is zero when the checksum is correct.

### IDENTIFY PACKET DEVICE - A1h, PIO Data-in

      - Feature Set

This command is mandatory for devices implementing the PACKET Command feature set.

      - Description

The IDENTIFY PACKET DEVICE command enables the host to receive parameter information from a device that implements the PACKET Command feature set.  .  See Table 17 for a description of the return data.

Some devices may have to read the media in order to complete this command.

When the command is issued, the device sets the BSY bit to one, prepares to transfer the 256 words of device identification data to the host, sets the DRQ bit to one, clears the BSY bit to zero, and asserts INTRQ if nIEN is cleared to zero.  The host may then transfer the data by reading the Data register.  Input Data [Editors note: add column indicating transport] defines the arrangement and meanings of the parameter words in the buffer.  All reserved bits or words shall be zero.

Some parameters are defined as a group of bits.  A word that is defined as a set of bits is transmitted with indicated bits on the respective data bus bit (e.g., bit 15 appears on DD15).

Some parameters are defined as a 16-bit value.  A word that is defined as a 16-bit value places the most significant bit of the value on bit DD15 and the least significant bit on bit DD0 (See 3.2.6).

Some parameters are defined as 32-bit values (e.g., words (61:60)).  Such fields are transferred using two word transfers.  The device shall first transfer the least significant bits, bits (15:0) of the value, on bits DD(15:0) respectively.  After the least significant bits have been transferred, the most significant bits, bits (31:16) of the value, shall be transferred on DD(15:0) respectively (See 3.2.6).

Some parameters are defined as a string of ASCII characters (See 3.2.6).

This command shall be accepted regardless of the state of DRDY.  [Editors Note: This needs to go in the transport documents]

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | A1h |

      - Normal outputs

See Table 65

      - Error outputs

The device shall return command aborted if the device does not implement this command, otherwise, the device shall not report an error.  See Table 79.

      - Input Data [Editors note: add column indicating transport]

        - Overview

Table 17 - IDENTIFY PACKET DEVICE data 17

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 0 | M |  | General configuration bit-significant information: |  |
|  |  | F | 15:14 | 10 = ATAPI device |
|  |  | F |  | 11 = Reserved |
|  |  | F | 13 | Reserved |
|  |  | F | 12:8 | Field indicates command packet set used by device |
|  |  | F | 7 | 1 = removable media device |
|  |  | F | 6:5 | 00 = Device shall set DRQ to one within 3 ms of receiving PACKET command. |
|  |  |  |  | 01 = Obsolete. |
|  |  |  |  | 10 = Device shall set DRQ to one within 50 s of receiving PACKET command. |
|  |  |  |  | 11 = Reserved |
|  |  | F | 4:3 | Reserved |
|  |  | V | 2 | Incomplete response |
|  |  | F | 1:0 | 00 = 12 byte command packet |
|  |  |  |  | 01 = 16 byte command packet |
|  |  |  |  | 1x = Reserved |
| 1 |  | F | Reserved |  |
| 2 |  | V | Unique configuration |  |
| 3-9 |  | F | Reserved |  |
| 10-19 | M | F | Serial number (20 ASCII characters) |  |
| 20-22 |  | F | Reserved |  |
| 23-26 | M | F | Firmware revision (8 ASCII characters) |  |
| 27-46 | M | F | Model number (40 ASCII characters) |  |
| 47-48 |  | F | Reserved |  |
| 49 | M |  | Capabilities |  |
|  |  | F | 15 | 1 = interleaved DMA supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 14 | 1 = command queuing supported |
|  |  | F | 13 | 1 = overlap operation supported |
|  |  | F | 12 | 1 = ATA software reset required (Obsolete) |
|  |  | F | 11 | 1 = IORDY supported |
|  |  | F | 10 | 1 = IORDY may be disabled |
|  |  | F | 9 | Shall be set to one. |
|  |  | F | 8 | 1 = DMA supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0 |
|  |  | X | 7:0 | Vendor specific |
| 50 | O |  | Capabilities |  |
|  |  | F | 15 | Shall be cleared to zero. |
|  |  | F | 14 | Shall be set to one. |
|  |  | F | 13:2 | Reserved |
|  |  | X | 1 | Obsolete |
|  |  | F | 0 | Shall be set to one to indicate a device specific Standby timer value minimum. |
| 51-52 |  | X | Obsolete |  |
| 53 | M | F | 15:3 | Reserved |
|  |  | F | 2 | 1 = the fields reported in word 88 are valid |
|  |  |  |  | 0 = the fields reported in word 88 are not valid |
|  |  | F | 1 | 1 = the fields reported in words (70:64) are valid |
|  |  |  |  | 0 = the fields reported in words (70:64) are not valid |
|  |  | X | 0 | Obsolete |
| 54-61 |  | F | Reserved |  |

(continued)

Table 17 - IDENTIFY PACKET DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 62 | M | F | 15 | 1 = DMADIR bit in the Packet command is required for DMA transfers 0 = DMADIR bit in Packet command is not required for DMA transfers. |
|  |  | F | 14:11 | Reserved |
|  |  | F | 10 | 1 = DMA is supported |
|  |  | F | 9 | 1 = Multiword DMA mode 2 is supported |
|  |  | F | 8 | 1 = Multiword DMA mode 1 is supported |
|  |  | F | 7 | 1 = Multiword DMA mode 0 is supported |
|  |  | F | 6 | 1 = Ultra DMA mode 6 and below are supported |
|  |  | F | 5 | 1 = Ultra DMA mode 5 and below are supported |
|  |  | F | 4 | 1 = Ultra DMA mode 4 and below are supported |
|  |  | F | 3 | 1 = Ultra DMA mode 3 and below are supported |
|  |  | F | 2 | 1 = Ultra DMA mode 2 and below are supported |
|  |  | F | 1 | 1 = Ultra DMA mode 1 and below are supported |
|  |  | F | 0 | 1 = Ultra DMA mode 0 is supported |
| 63 | M | F | 15:11 | Reserved |
|  |  | V | 10 | 1 = Multiword DMA mode 2 is selected |
|  |  |  |  | 0 = Multiword DMA mode 2 is not selected |
|  |  | V | 9 | 1 = Multiword DMA mode 1 is selected |
|  |  |  |  | 0 = Multiword DMA mode 1 is not selected |
|  |  | V | 8 | 1 = Multiword DMA mode 0 is selected |
|  |  |  |  | 0 = Multiword DMA mode 0 is not selected |
|  |  | F | 7:3 | Reserved |
|  |  | F | 2 | 1 = Multiword DMA mode 2 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 1 | 1 = Multiword DMA mode 1 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 0 | 1 = Multiword DMA mode 0 is supported Multiword DMA mode selected Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
| 64 | M | F | 15:8 | Reserved |
|  |  | F | 7:0 | PIO transfer modes supported |
| 65 | M |  | Minimum Multiword DMA transfer cycle time per word |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 66 | M |  | Manufacturer’s recommended Multiword DMA transfer cycle time |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 67 | M |  | Minimum PIO transfer cycle time without flow control |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 68 | M |  | Minimum PIO transfer cycle time with IORDY flow control |  |
|  |  | F | 15:0 | Cycle time in nanoseconds |
| 69-70 |  | F | Reserved (for future command overlap and queuing) |  |
| 71 | O | F | Typical time in ns from receipt of PACKET command to bus release. |  |
| 72 | O | F | Typical time in ns from receipt of SERVICE command to BSY cleared to zero |  |
| 73-74 |  | F | Reserved |  |
| 75 | O |  | Queue depth |  |
|  |  | F | 15:5 | Reserved |
|  |  | F | 4:0 | Maximum queue depth supported - 1 |
| 76-79 |  | R | Reserved for Serial ATA |  |

(continued)

Table 17 - IDENTIFY PACKET DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 80 | M |  | Major version number 0000h or FFFFh = device does not report version |  |
|  |  | F | 15 | Reserved |
|  |  | F | 14 | Reserved for ATA/ATAPI-14 |
|  |  | F | 13 | Reserved for ATA/ATAPI-13 |
|  |  | F | 12 | Reserved for ATA/ATAPI-12 |
|  |  | F | 11 | Reserved for ATA/ATAPI-11 |
|  |  | F | 10 | Reserved for ATA/ATAPI-10 |
|  |  | F | 9 | Reserved for ATA/ATAPI-9 |
|  |  | F | 8 | Reserved for ATA/ATAPI-8 |
|  |  | F | 7 | 1 = Supports ATA/ATAPI-7 |
|  |  | F | 6 | 1 = supports ATA/ATAPI-6 |
|  |  | F | 5 | 1 = supports ATA/ATAPI-5 |
|  |  | F | 4 | 1 = supports ATA/ATAPI-4 |
|  |  | F | 3 | Obsolete |
|  |  | X | 2 | Obsolete |
|  |  | X | 1 | Obsolete |
|  |  | F | 0 | Reserved |
| 81 | M |  | Minor version number |  |
|  |  |  | 0000h or FFFFh=device does not report version |  |
|  |  |  | 0001h-FFFEh=See 7.17.7.41 |  |
| 82 | M |  | Command set supported. If words (83:82) = 0000h or FFFFh command set notification not supported. |  |
|  |  | X | 15 | Obsolete |
|  |  | F | 14 | 1 = NOP command supported |
|  |  | F | 13 | 1 = READ BUFFER command supported |
|  |  | F | 12 | 1 = WRITE BUFFER command supported |
|  |  | X | 11 | Obsolete |
|  |  | F | 10 | 1 = Host Protected Area feature set supported |
|  |  | F | 9 | 1 = DEVICE RESET command supported |
|  |  | F | 8 | 1 = SERVICE interrupt supported |
|  |  | F | 7 | 1 = release interrupt supported |
|  |  | F | 6 | 1 = look-ahead supported |
|  |  | F | 5 | 1 = write cache supported |
|  |  | F | 4 | Shall be set to one indicating the PACKET Command feature set is supported. |
|  |  | F | 3 | 1 = Power Management feature set supported |
|  |  | F | 2 | 1 = Removable Media feature set supported |
|  |  | F | 1 | 1 = Security Mode feature set supported |
|  |  | F | 0 | 1 = SMART feature set supported |
| 83 | M |  | Command sets supported. If words (83:82) = 0000h or FFFFh command set notification not supported. |  |
|  |  | F | 15 | Shall be cleared to zero |
|  |  | F | 14 | Shall be set to one |
|  |  | F | 13 | Reserved |
|  |  | F | 12 | 1 = FLUSH CACHE command supported |
|  |  | F | 11 | 1 = Device Configuration Overlay feature set supported |
|  |  | F | 10 | Reserved |
|  |  | F | 9 | 1 = AUTOMATIC Acoustic Management feature set supported |
|  |  | F | 8 | 1 = SET MAX security extension supported |
|  |  | F | 7 | See Address Offset Reserved Area Boot, INCITS TR27:2001 |
|  |  | F | 6 | 1 = SET FEATURES subcommand required to spinup after power-up |
|  |  | F | 5 | 1 = Power-Up In Standby feature set supported |
|  |  | F | 4 | 1 = Removable Media Status Notification feature set supported |
|  |  | F | 3:1 | Reserved |
|  |  | F | 0 | 1 = DOWNLOAD MICROCODE command supported [Editors Note: DM is not supported on ATAPI devices] |

(continued)

Table 17 - IDENTIFY PACKET DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 84 | M |  | Command set/feature supported extension. If words 82, 83, and 84 = 0000h or FFFFh command set notification extension is not supported. |  |
|  |  | F | 15 | Shall be cleared to zero |
|  |  | F | 14 | Shall be set to one |
|  |  | F | 13:0 | Reserved |
| 85 | M |  | Command set/feature enabled. If words 85, 86, and 87 = 0000h or FFFFh command set enabled notification is not supported. |  |
|  |  | X | 15 | Obsolete |
|  |  | F | 14 | 1 = NOP command enabled |
|  |  | F | 13 | 1 = READ BUFFER command enabled |
|  |  | F | 12 | 1 = WRITE BUFFER command enabled |
|  |  | X | 11 | Obsolete |
|  |  | V | 10 | 1 = Host Protected Area feature set enabled |
|  |  | F | 9 | 1 = DEVICE RESET command enabled |
|  |  | V | 8 | 1 = SERVICE interrupt enabled |
|  |  | V | 7 | 1 = release interrupt enabled |
|  |  | V | 6 | 1 = look-ahead enabled |
|  |  | V | 5 | 1 = write cache enabled |
|  |  | F | 4 | Shall be set to one indicating the PACKET Command feature set is supported. |
|  |  | F | 3 | 1 = Power Management feature set enabled |
|  |  | V | 2 | 1 = Removable Media feature set enabled |
|  |  | V | 1 | 1 = Security Mode feature set enabled |
|  |  | V | 0 | 1 = SMART feature set enabled |
| 86 | M |  | Command set/feature enabled. If words 85, 86, and 87 = 0000h or FFFFh command set enabled notification is not supported. |  |
|  |  | F | 15:13 | Reserved |
|  |  | V | 12 | 1 = FLUSH CACHE command supported |
|  |  | F | 11 | 1 = Device Configuration Overlay feature set supported |
|  |  | F | 10 | Reserved |
|  |  | V | 9 | 1 = Automatic Acoustic Management feature set enabled |
|  |  | V | 8 | 1 = SET MAX security extension enabled by a SET MAX SET PASSWORD |
|  |  | V | 7 | See Address Offset Reserved Area Boot, INCITS TR27:2001 |
|  |  | F | 6 | 1 = SET FEATURES subcommand required to spinup after power-up |
|  |  | V | 5 | 1 = Power-Up In Standby feature set enabled |
|  |  | V | 4 | 1 = Removable Media Status Notification feature set enabled via the SET FEATURES command. |
|  |  | F | 3:1 | Reserved |
|  |  | F | 0 | 1 = DOWNLOAD MICROCODE command enabled [Editors Note: DM is not implemented on ATAPI devices.] |
| 87 | M |  | Command set/feature default. If words 85, 86, and 87 = 0000h or FFFFh command set default notification is not supported. |  |
|  |  | F | 15 | Shall be cleared to zero |
|  |  | F | 14 | Shall be set to one |
|  |  | F | 13:0 | Reserved |

(continued)

Table 17 - IDENTIFY PACKET DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 88 | M | F | 15 | Reserved |
|  |  |  | 14 | 1 = Ultra DMA mode 6 is selected |
|  |  |  |  | 0 = Ultra DMA mode 6 is not selected |
|  |  | V | 13 | 1 = Ultra DMA mode 5 is selected |
|  |  |  |  | 0 = Ultra DMA mode 5 is not selected |
|  |  | V | 12 | 1 = Ultra DMA mode 4 is selected |
|  |  |  |  | 0 = Ultra DMA mode 4 is not selected |
|  |  | V | 11 | 1 = Ultra DMA mode 3 is selected |
|  |  |  |  | 0 = Ultra DMA mode 3 is not selected |
|  |  | V | 10 | 1 = Ultra DMA mode 2 is selected |
|  |  |  |  | 0 = Ultra DMA mode 2 is not selected |
|  |  | V | 9 | 1 = Ultra DMA mode 1 is selected |
|  |  |  |  | 0 = Ultra DMA mode 1 is not selected |
|  |  | V | 8 | 1 = Ultra DMA mode 0 is selected |
|  |  |  |  | 0 = Ultra DMA mode 0 is not selected |
|  |  | F | 7 | Reserved |
|  |  | F | 6 | 1 = Ultra DMA mode 6 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 5 | 1 = Ultra DMA mode 5 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 4 | 1 = Ultra DMA mode 4 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 3 | 1 = Ultra DMA mode 3 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 2 | 1 = Ultra DMA mode 2 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 1 | 1 = Ultra DMA mode 1 and below are supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
|  |  | F | 0 | 1 = Ultra DMA mode 0 is supported Devices which require the DMADIR bit in the Packet command shall clear this bit to 0. |
| 89-92 |  | F | Reserved |  |

(continued)

Table 17 - IDENTIFY PACKET DEVICE data (continued)

| Word | O/M | F/V | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| 93 | * |  | Hardware reset result. The contents of bits (12:0) of this word shall change only during the execution of a hardware reset. |  |  |
|  |  | F | 15 | Shall be cleared to zero. |  |
|  |  | F | 14 | Shall be set to one. |  |
|  |  | V | 13 | 1 = device detected CBLID- above ViH |  |
|  |  |  |  | 0 = device detected CBLID- below ViL |  |
|  |  |  | 12:8 | Device 1 hardware reset result. Device 0 shall clear these bits to zero. Device 1 shall set these bits as follows: |  |
|  |  | F |  | 12 | Reserved. |
|  |  | V |  | 11 | 0 = Device 1 did not assert PDIAG-. |
|  |  |  |  |  | 1 = Device 1 asserted PDIAG-. |
|  |  | V |  | 10:9 | These bits indicate how Device 1 determined the device number: |
|  |  |  |  |  | 00 = Reserved. |
|  |  |  |  |  | 01 = a jumper was used. |
|  |  |  |  |  | 10 = the CSEL signal was used. |
|  |  |  |  |  | 11 = some other method was used or the method is unknown. |
|  |  | F |  | 8 | Shall be set to one. |
|  |  |  | 7:0 | Device 0 hardware reset result. Device 1 shall clear these bits to zero. Device 0 shall set these bits as follows: |  |
|  |  | F |  | 7 | Reserved. |
|  |  | F |  | 6 | 0 = Device 0 does not respond when Device 1 is selected. |
|  |  |  |  |  | 1 = Device 0 responds when Device 1 is selected. |
|  |  | V |  | 5 | 0 = Device 0 did not detect the assertion of DASP-. |
|  |  |  |  |  | 1 = Device 0 detected the assertion of DASP-. |
|  |  | V |  | 4 | 0 = Device 0 did not detect the assertion of PDIAG-. |
|  |  |  |  |  | 1 = Device 0 detected the assertion of PDIAG-. |
|  |  | V |  | 3 | 0 = Device 0 failed diagnostics. |
|  |  |  |  |  | 1 = Device 0 passed diagnostics. |
|  |  |  |  | 2:1 | These bits indicate how Device 0 determined the device number: |
|  |  | F |  |  | 00 = Reserved. |
|  |  | V |  |  | 01 = a jumper was used. |
|  |  | V |  |  | 10 = the CSEL signal was used. |
|  |  | V |  |  | 11 = some other method was used or the method is unknown. |
|  |  | F |  | 0 | Shall be set to one. |
| 94 | O | V | 15:8 | Vendor’s recommended acoustic management value. |  |
|  |  | V | 7:0 | Current automatic acoustic mamagement value. |  |
| 95-124 |  | F | Reserved |  |  |
| 125 | M | F | ATAPI byte count = 0 behavior |  |  |
| 126 |  | X | Obsolete |  |  |
| 127 | O |  | Removable Media Status Notification feature set support |  |  |
|  |  | F | 15:2 | Reserved |  |
|  |  | F | 1:0 | 00 = Removable Media Status Notification feature set not supported |  |
|  |  |  |  | 01 = Removable Media Status Notification feature set supported |  |
|  |  |  |  | 10 = Reserved |  |
|  |  |  |  | 11 = Reserved |  |

(continued)

Table 17 - IDENTIFY PACKET DEVICE data (continued)

| Word | O/M | F/V | Description |  |
| --- | --- | --- | --- | --- |
| 128 | O |  | Security status |  |
|  |  | F | 15:9 | Reserved |
|  |  | V | 8 | Security level 0 = High, 1 = Maximum |
|  |  | F | 7:6 | Reserved |
|  |  | F | 5 | 1 = Enhanced security erase supported |
|  |  | V | 4 | 1 = Security count expired |
|  |  | V | 3 | 1 = Security frozen |
|  |  | V | 2 | 1 = Security locked |
|  |  | V | 1 | 1 = Security enabled |
|  |  | F | 0 | 1 = Security supported |
| 129-159 |  | X | Vendor specific |  |
| 160-175 |  | F | Reserved for assignment by the CompactFlash Association |  |
| 176-254 |  | F | Reserved |  |
| 255 | O | X | Integrity word |  |
|  |  |  | 15:8 | Checksum |
|  |  |  | 7:0 | Signature |
| Key: O/M = Mandatory/optional requirement. M = Support of the word is mandatory. O = Support of the word is optional. * = See 7.17.7.65. F/V = Fixed/variable content. F = the content of the word is fixed and does not change. For removable media devices, these values may change when media is removed or changed. V = the contents of the word is variable and may change depending on the state of the device or the commands executed by the device. X = the content of the word may be fixed or variable. |  |  |  |  |

(Concluded)

        - Word 0: General configuration

Bits (15:14) of word 0 indicate the type of device.  Bit 15 shall be  set to one and bit 14 shall be cleared to zeroto indicate the device implements the PACKET Command feature set.

Bits (12:8) of word 0 indicate the command packet set implemented by the device.  This value follows the peripheral device type value as defined in SCSI Primary Commands, ANSI INCITS 301:1997.

| Value | Description |
| --- | --- |
| 00h | Direct-access device |
| 01h | Sequential-access device |
| 02h | Printer device |
| 03h | Processor device |
| 04h | Write-once device |
| 05h | CD-ROM device |
| 06h | Scanner device |
| 07h | Optical memory device |
| 08h | Medium changer device |
| 09h | Communications device |
| 0A-0Bh | Reserved for ACS IT8 (Graphic arts pre-press devices) |
| 0Ch | Array controller device |
| 0Dh | Enclosure services device |
| 0Eh | Reduced block command devices |
| 0Fh | Optical card reader/writer device |
| 10-1Eh | Reserved |
| 1Fh | Unknown or no device type |

Bit 7 if set to one indicates that the device has removable media.

Bits (6:5) of word 0 indicate the DRQ response time when a PACKET command is received.  A value of 00b indicates a maximum time of 3 ms from receipt of PACKET to the setting of DRQ to one.  A value of 10b indicates a maximum time of 50 s from the receipt of PACKET to the setting of DRQ to one.  The value 11b is reserved.

If bit 2 is set to one it indicates that the content of the IDENTIFY DEVICE data is incomplete.  This will occur if the device supports the Power-up in Standby feature set and required data is contained on the device media.  In this case the content of at least word 0 and word 2 shall be valid.

Bits (1:0) of word 0 indicate the packet size the device supports.  A value of 00b indicates that a 12-byte packet is supported; a value of 01b indicates a 16 byte packet.  The values 10b and 11b are reserved.

        - Word 1: Reserved

        - Word 2: Specific configuration

Word 2 shall have the same content described for word 2 of the IDENTIFY DEVICE command.

        - Words (9:3): Reserved

        - Words (19:10): Serial number

The use of these words is optional.  If not implemented, the content shall be zeros.  If implemented, the content shall be as described in words (19:10) of the IDENTIFY DEVICE command (See 7.17).

        - Words (22:20): Reserved

        - Words (26:23): Firmware revision

Words (26:23) shall have the content described for words (26:23) of the IDENTIFY DEVICE command.

        - Words (46:27): Model number

Words (46:27) shall have the content described for words (46:27) of the IDENTIFY DEVICE command.

        - Words (48:47): Reserved

        - Word 49: Capabilities

Bit 15 of word 49 is used to indicate that the device supports interleaved DMA data transfer for overlapped DMA commands.  Devices which require the DMADIR bit in the Packet command shall clear this bit to 0.

Bit 14 of word 49 is used to indicate that the device supports command queuing for overlapped commands.  If bit 14 is set to one, bit 13 shall be set to one.

Bit 13 of word 49 is used to indicate that the device supports command overlap operation.

Bit 12 of word 49 is obsolete.

Bit 11 of word 49 is used to determine whether a device supports IORDY.  If this bit is set to one, then the device supports IORDY operation.  If this bit is zero, the device may support IORDY.  This ensures backward compatibility.  If a device supports PIO mode 3 or higher, then this bit shall be set to one.  If the serial interface is implemented, this bit shall be set to one.

Bit 10 of word 49 is used to indicate a device’s ability to enable or disable the use of IORDY.  If this bit is set to one, then the device supports the disabling of IORDY.  Disabling and enabling of IORDY is accomplished using the SET FEATURES command.  If the serial interface is implemented, this bit shall be set to one.

Bit 9 of word 49 shall be set to one.

Bit 8 of word 49 indicates that DMA is supported.  Devices which require the DMADIR bit in the Packet command shall clear this bit to 0

        - Word 50: Capabilities

Word 50 shall have the content described for word 50 of the IDENTIFY DEVICE command.  Support of this word is mandatory if the STANDBY command is supported.

        - Word 51: Obsolete

        - Word 52: Reserved

        - Word 53: Field validity

Word 53 shall have the content described for word 53 of the IDENTIFY DEVICE command.

        - Words (61:54): Reserved

        - Word 62: DMADIR

ATAPI devices that use a serial ATA bridge chip for connection to a serial ATA host may require use of the DMADIR bit to indicate transfer direction for Packet DMA commands.  Word 62 is used to indicate if such support is required.

If bit 15 of word 62 is set to one, then DMADIR bit in the Packet Command  is required by the device for Packet DMA and Bits 2:0 of word 63, bits 15 and 8 in word 49, and bits 6:0 of word 88 shall be cleared to 0,.

If bit 15 of word 62 is cleared to 0, DMADIR bit in the PACKET command is not required.  If bit 15 of word 62 is cleared to zero, then all bits of word 62 shall be cleared to zero.

Bits (14:11) are reserved.

Bits (10:1) indicate DMA mode support.  Since the DMADIR bit is only used for a Serial ATAPI device, all of these bits are set to 1.

        - Word 63: Multiword DMA transfer

Word 63 identifies the Multiword DMA transfer modes supported by the device and indicates the mode that is currently selected.  Only one DMA mode shall be selected at any given time.  If an Ultra DMA mode is enabled, then no Multiword DMA mode shall be enabled.  If a Multiword DMA mode is enabled then no Ultra DMA mode shall be enabled.

Bit (15:11) are Reserved.

Bits 10:8 shall have the content described for word 63 of the IDENTIFY DEVICE command.

Bits (7:3) are Reserved

If bit 2 of Word 63 is set to one, then Multiword DMA modes 2 and below are supported.  If this bit is cleared to zero, then Multiword DMA mode 2 is not supported.  If Multiword DMA mode 2 is supported, then Multiword DMA modes 1 and 0 shall also be supported.

If bit 2 of Word 63 is set to one, bits (1:0) shall be set to one.  If the serial interface is implemented, this bit shall be set to one except this bit shall be cleared 0 for Serial ATAPI devices requiring the DMADIR bit in the PACKET command.

If bit 1 of Word 63 is set to one, then Multiword DMA modes 1 and below are supported.  If this bit is cleared to zero, then Multiword DMA mode 1 is not supported.  If Multiword DMA mode 1 is supported, then Multiword DMA mode 0 shall also be supported.

If bit 1 of Word 63 is set to one, bit 0 shall be set to one.  If the serial interface is implemented, this bit shall be set to one except this bit shall be cleared to 0 for Serial ATAPI devices which require the DMADIR bit in the PACKET command.

If bit 0 of word 63 is set to one, then Multiword DMA mode 0 is supported.  If the serial interface is implemented, this bit shall be set to one except this bit shall be cleared to 0 for Serial ATAPI devices which require the DMADIR bit in the PACKET command.

        - Word 64: PIO transfer mode supported

Word 64 shall have the content described for word 64 of the IDENTIFY DEVICE command.

        - Word 65: Minimum multiword DMA transfer cycle time per word

Word 65 shall have the content described for word 65 of the IDENTIFY DEVICE command.

        - Word 66: Device recommended multiword DMA cycle time

Word 66 shall have the content described for word 66 of the IDENTIFY DEVICE command.

        - Word 67: Minimum PIO transfer cycle time without flow control

Word 67 shall have the content described for word 67 of the IDENTIFY DEVICE command.

        - Word 68: Minimum PIO transfer cycle time with IORDY

Word 68 shall have the content described for word 68 of the IDENTIFY DEVICE command.

        - Word (70:69): Reserved

        - Word 71: PACKET to bus release time

Word 71 shall contain the time (for 99.7% of the occurances) in microseconds from the receipt of a PACKET command until the device performs a bus release.  Support of this word is mandatory if the Overlap or Queuing feature set is supported.

        - Word 72: SERVICE to bus release time

Word 72 shall contain the time (for 99.7% of the occurances) in microseconds from the receipt of a SERVICE command until the device performs a bus release.  Support of this word is mandatory if the Overlap or Queuing feature set is supported.

        - Word (74:73): Reserved

        - Word 75: Queue depth

Bits (4:0) of word 75 shall have the content described for word 75 of the IDENTIFY DEVICE command.  Support of this word is mandatory if the Queuing feature set is supported.

        - Words (79:76): Reserved for Serial ATA

        - Word 80: Major revision number

Word 80 shall have the content described for word 80 of the IDENTIFY DEVICE command.

        - Word 81: Minor revision number

Word 81 shall have the content described for word 81 of the IDENTIFY DEVICE command.

        - Words (84:82): Features/command sets supported

Words (84:82) shall have the content described for words (84:82) of the IDENTIFY DEVICE command except that bit 4 of word 82 shall be set to one to indicate that the PACKET Command feature set is supported.

        - Words (87:85): Features/command sets enabled

Words (87:85) shall have the content described for words (87:85) of the IDENTIFY DEVICE command except that bit 4 of word 85 shall be set to one to indicate that the PACKET Command feature set is supported.

        - Word 88:Ultra DMA modes

Word 88 shall have the content described for word 88 of the IDENTIFY DEVICE command, except bits 6:0 shall be cleared to 0 for Serial ATAPI devices which require the DMADIR bit in the Packet command.

        - Word 89: Time required for Security erase unit completion

Word 89 shall have the content described for word 89 of the IDENTIFY DEVICE command.

        - Word 90: Time required for Enhanced security erase unit completion

Word 90 shall have the content described for word 90 of the IDENTIFY DEVICE command.

        - Word (92:91): Reserved

        - Word 93: Hardware reset results

Word 93 shall have the content described for word 93 of the IDENTIFY DEVICE command.  Support of bits (13:15) is mandatory.  Support of bits (12:0) is optional.

        - Word 94: Current automatic acoustic management value

Word 94 shall have the content described for word 94 of the IDENTIFY DEVICE command.

        - Word (124:95): Reserved

        - Word 125 ATAPI byte count=0 behavior.

If the contents of word 125 are 0000h and the value of the byte count limit is zero, the device shall return command aborted.

If the contents of word 125 are non-zero and the value of the byte count limit is zero, the device shall use the contents of word 125 as the actual byte count limit for the current command and shall not abort.

The device may be reconfigured to report a new value.  However, after the device is reconfigured, the content of word 125 reported shall not change until after the next hardware reset or power-on reset event.

        - Word 126: Obsolete

        - Word 127: Removable Media Status Notification feature set support

Word 127 shall have the content described for word 127 of the IDENTIFY DEVICE command.  Support of this word is mandatory if the Removable Media Status Notification feature set is supported.

        - Word 128: Security status

Word 128 shall have the content described for word 128 of the IDENTIFY DEVICE command.  Support of this word is mandatory if the Security feature set is supported.

        - Words (160:129): Reserved

        - Words (175:161): Reserved for assignment by the CompactFlash Association

        - Words (254:176): Reserved

        - Word 255: Integrity Word

Word 255 shall have the content described for word 255 of the IDENTIFY DEVICE command.  Word 255 should be implemented.

### IDLE - E3h, Non-data

      - Feature Set

This command is mandatory for devices implementing the Power Management feature set.  The Power Management feature set is mandatory for devices not implementing the PACKET Command set  This command is mandatory when power management is not implemented by the PACKET Command set implemented by the device.

      - Description

The IDLE command allows the host to place the device in the Idle mode and also set the Standby timer.  INTRQ may be asserted even though the device may not have fully transitioned to Idle mode.

If the Count field is non-zero then the Standby timer shall be enabled.  The value in the Count field shall be used to determine the time programmed into the Standby timer (See 4.5).  If the Count field is zero then the Standby timer is disabled.

      - Inputs

Values other than zero in the Sector Count register when the IDLE command is issued shall determine the time period programmed into the Standby timer.  Table 18 defines these values.

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | Reserved |
| 01h | Count | Timer period value (TPV) -The TPV shall determine the time period programmed into the Standby timer. Table 18 defines these values. |
| 02h-04h | LBA | Reserved |
| 05h | Command | E3h |

Table 18 - Automatic Standby timer periods18

| Sector Count register contents |  | Corresponding timeout period |
| --- | --- | --- |
| 0 | (00h) | Timeout disabled |
| 1-240 | (01h-F0h) | (value  5) seconds |
| 241-251 | (F1h-FBh) | ((value - 240) 30) minutes |
| 252 | (FCh) | 21 minutes |
| 253 | (FDh) | Period between 8 hours and 12 hours |
| 254 | (Feh) | Reserved |
| 255 | (FFh) | 21 min 15 seconds |
| NOTE  Times are approximate. |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79

### IDLE IMMEDIATE - E1h, Non-data

      - Feature Set

This command is mandatory for devices implementing the Power Management feature set.  However, the Unload Feature of the command is optional.  The Power Management feature set is mandatory for devices not implementing the PACKET Command set  This command is mandatory when power management is not implemented by the PACKET Command set implemented by the device.

      - Description

Default Function:

The IDLE IMMEDIATE command allows the host to immediately place the device in the Idle mode.  Command completion may occur even though the device has not fully transitioned into the Idle mode.

Unload Feature:

The UNLOAD FEATURE of the IDLE IMMEDIATE command allows the host to immediately unload/park the heads.  The device shall stop read look-ahead if it is in process.  If the device is performing a write operation, the device shall suspend writing cached data onto the media as soon as possible, and keep unwritten sectors stored in the buffer until receiving a new command.

A device that supports load/unload technology shall retract the head(s) onto the ramp position as soon as receiving this command.  INTRQ shall be asserted and BSY shall be cleared after the head(s) is(are) completely retracted onto the ramp position and latched if available.  The time to complete the unload operation is vendor specific, this typically would be within 500 milliseconds of receiving the command.  The unload controlling method by the Unload Feature of the Idle Immediate command shall be the same as that by Power mode transition, and shall not effect the specification of normal load/unload times per device life.

A device that supports contact start/stop technology shall seek to the landing zone. The device shall not indicate command completion until it has completed the seek operation.  The time to complete the seek operation is vendor specific, this typically would be within 300 milliseconds of receiving this command.

The device shall stay at Low Power Idle mode, shall not go into Standby mode and shall not load the head(s) onto the media until receiving a new command.  Power consumption of the device is not an issue for this case.  If a device receives this command while the head(s) is(are) currently on ramp/parked no physical action is needed.

The device shall retain data in the write cache and resume writing the cached data onto the media after receiving a Software Reset, a Hardware Reset, or any new command except IDLE IMMEDIATE with UNLOAD FEATURE.

      - Inputs (Default Fuction)

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A except when the unload feature is requested, see 7.20.4 |
| 01h | Count | N/A except when the unload feature is requested, see 7.20.4 |
| 02h-04h | LBA | N/A except when the unload feature is requested, see 7.20.4 |
| 05h | Command | E1h |

      - Inputs (Unload Feature)

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | 0044h |
| 01h | Count | 0044h |
| 02h-04h | LBA | 000000554E4Ch |
| 05h | Command | E1h |

      - Normal outputs (Default Fuction)

See Table 65

      - Normal outputs (Unload Feature)

See Table 71

      - Error outputs

See Table 79

### MEDIA EJECT - EDh, Non-data

      - Feature Set

This command is mandatory for devices implementing the Removable Media Status Notification feature set or the Removable Media feature set.

      - Description

This command causes any pending operations to complete, spins down the device if needed, unlocks the media if locked, and ejects the media.  The device keeps track of only one level of media lock.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | EDh |

      - Normal outputs

See Table 65

      - Error outputs

See Table 83.

### MEDIA LOCK - DEh, Non-data

      - Feature Set

This command is optional for devices implementing the Removable Media Status Notification feature set.  This command is mandatory for devices implementing the Removable Media feature set.

      - Description

This command shall be used to lock the media, if Media Status Notification is disabled.  If Media Status Notification is enabled, this command shall return good status (no Error bit in the Status field) and perform no action.

If the media is unlocked and media is present, the media shall be set to the LOCKED state and no Error register bit shall be set to one.  The device keeps track of only one level of media lock.  Subsequent MEDIA LOCK commands, while the media is in the LOCKED state, do not set additional levels of media locks.

If the media is locked, the status returned shall indicate whether a media change request has been detected by the device.  If a media change request has been detected, the MCR bit in the Error register and the ERR bit in the Status register shall be set to one.

When media is in the LOCKED state, the device shall respond to the media change request button, by setting the MCR bit in the Error register and the ERR bit in the Status register to one, until the media LOCKED condition is cleared.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | DEh |

      - Normal outputs

See Table 65

      - Error outputs

See Table 84

### MEDIA UNLOCK - DFh, Non-data

      - Feature Set

This command is optional for devices implementing the Removable Media Status Notification feature set.  This command is mandatory for devices implementing the Removable Media feature set.

      - Description

This command can be used to unlock the device, if Media Status Notification is disabled.  If Media Status Notification is enabled, this command will return good status (no Error bit in the Status field) and perform no action.

If the media is present, the media shall be set to the UNLOCKED state and no Error register bit shall be set to one.  The device keeps track of only one level of media lock.  A single MEDIA UNLOCK command unlocks the media.

If a media change request has been detected by the device prior to the issuance of this command, the media shall be ejected at MEDIA UNLOCK command completion.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | DFh |

      - Normal outputs

See Table 65

      - Error outputs

See Table 83.

### NOP - 00h, Non-data

      - Feature Set

This command is mandatory for devices implementing the PACKET Command feature set.  This command is mandatory for devices implementing the Overlapped feature set.

      - Description

The device shall respond with command aborted.  For devices implementing the Overlapped feature set, subcommand code 00h in the Features register shall abort any outstanding queue.  Subcommand codes 01h through FFh in the Features register shall not affect the status of any outstanding queue.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | Subcommand Code (See Table 19) |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | 00h |

Table 19 - NOP Subcommand Code 19Error! Bookmark not defined.

| Subcommand code | Description | Action |
| --- | --- | --- |
| 00h | NOP | Return command aborted and abort any outstanding queued commands. |
| 01h | NOP Auto Poll | Return command aborted and do not abort any outstanding queued commands. |
| 02h-FFh | Reserved | Return command aborted and do not abort any outstanding queued commands. |

      - Normal outputs

This command always fails with an error.

      - Error outputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |
|  |  |  |  | 15:8 | Reserved |  |  |
|  |  |  |  | 7:3 | N/A |  |  |
|  |  |  |  | 2 | Abort - See clause 6.3.1. |  |  |
|  |  |  |  | 1:0 | Obsolete |  |  |
| 01h | Count | Initial Value |  |  |  |  |  |
| 02h-04h | LBA | Initial Value |  |  |  |  |  |
| 05h | Status |  |  |  |  |  |  |
|  |  |  | Bit |  |  | Description |  |
|  |  |  | 15:8 |  |  | Reserved |  |
|  |  |  | 7:6 |  |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 |  |  | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 |  |  | N/A |  |
|  |  |  | 3 |  |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 |  |  | N/A |  |
|  |  |  | 0 |  |  | Error - See clause 6.2.3 |  |

The Command Block registers, other than the Error and Status registers, are not changed by this command.  This command always fails with the device returning command aborted.

### PACKET - A0h, Packet

      - Feature Set

This command is mandatory for devices implementing the PACKET Command feature set.

      - Description

[Editors Note: Is it time to obsolete ATAPI overlap and queuing?]

The PACKET command is used to transfer a device command via a command packet.  If the native form of the encapsulated command is shorter than the packet size reported in bits (1:0) of word 0 of the IDENTIFY PACKET DEVICE response, the encapsulated command shall begin at byte 0 of the packet.  Packet bytes beyond the end of the encapsulated command are reserved.

If the device supports overlap, the OVL bit is set to one in the Features register and the Release interrupt has been disabled via the SET FEATURES command, the device may or may not perform a bus release.  If the device is ready for the data transfer, the device may begin the transfer immediately as described in the non-overlapped protocol (See Clause 5.9).  If the data is not ready, the device may perform a bus release and complete the transfer after the execution of a SERVICE command.

      - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 7:3 | Reserved |  |
|  |  |  | 2 | DMADIR - See clause 7.25.4 |  |
|  |  |  | 1 | OVL - This bit is set to one to inform the device that the PACKET command is to be overlapped. |  |
|  |  |  | 0 | DMA - This bit is set to one to inform the device that the data transfer (not the command packet transfer) associated with this command is via Multiword DMA or Ultra DMA mode. |  |
| 01h | Count |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 7:3 | Tag - If the device supports command queuing, this field contains the command Tag for the command being delivered. A Tag may have any value between 0 and 31 regardless of the queue depth supported. If queuing is not supported, this field is not applicable. |  |
|  |  |  | 2:0 | N/A |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | Byte Count - See clause 7.25.5 |  |
|  |  |  | 7:0 | Reserved |  |
| 05h | Command | A0h |  |  |  |

      - DMADIR

This bit indicates Packet DMA direction and is used only for devices that implement the Packet Command feature set with a Serial ATA bridge that require direction indication from the host.  Support for this bit is determined by reading bit 15 of word 62 in the IDENTIFY PACKET DEVICE data.  If bit 15 of word 62 is set to 1, the device requires the use of the DMADIR bit for Packet DMA commands.

If the device requires the DMADIR bit to be set for Packet DMA operations and the current operations is DMA (i.e.  bit 0, the DMA bit, is set), this bit indicates the direction of data transfer (0 = transfer to the device; 1 = transfer to the host).  If the device requires the DMADIR bit to be set for Packet DMA operations but the current operations is PIO (i.e.  bit 0, the DMA bit, is cleared), this bit is ignored.

Since the data transfer direction will be set by the host as the command is constructed, the DMADIR bit should not conflict with the data transfer direction of the command.  If a conflict between the command transfer direction and the DMADIR bit occurs, the device should return with an ABORTED command, and the sense key set to ILLEGAL REQUEST.

If the device does not require the DMADIR bit for Packet DMA operations, this bit should be cleared to 0.

A device that does not support the DMADIR feature may abort a command if the DMADIR bit is set to 1.

      - Byte Count

This is the maximum byte count that is to be transferred in any single DRQ assertion for PIO transfers.  The byte count does not apply to the command packet transfer.  If the PACKET command does not transfer data, the byte count is ignored.

If the PACKET command results in a data transfer:

- the host should not set the byte count limit to zero.  If the host sets the byte count limit to zero, the contents of IDENTIFY PACKET DEVICE data word 125 determines the expected behavior;

- the value set into the byte count limit shall be even if the total requested data transfer length is greater than the byte count limit;

- the value set into the byte count limit may be odd if the total requested data transfer length is equal to or less than the byte count limit;

The value FFFFh is interpreted by the device as though the value were FFFEh.

      - Normal outputs

        - Awaiting command

When the device is ready to accept the command packet from the host the return structure shall be set according to Table 72.  Release shall be cleared to zero, Input/Output shall be cleared to zero, and Command/Data shall be set to one.  Byte Count shall reflect the value set by the host when the command was issued

        - Data transmission

If overlap is not supported or not specified by the command, data transfer shall occur after the receipt of the command packet.  If overlap is supported and the command specifies that the command may be overlapped, data transfer may occur after receipt of the command packet or may occur after the receipt of a SERVICE command.  See Table 72 for the return structure when the device is ready to transfer data requested by a data transfer command.  Release shall be cleared to zero, Input/Output is ignored, and Command/Data shall be set to zero.  See clause 7.25.6.3 for a desciption of how Byte Count operates during data transmission.

        - Byte Count

If the transfer is to be in PIO mode, the byte count of the data to be transferred for this DRQ assertion shall be presented.

Valid byte count values are as follows:

- the byte count shall be less than or equal to the byte count limit value from the host;

- the byte count shall not be zero;

- the byte count shall be less than or equal to FFFEh;

- the byte count shall be even except for the last transfer of a command;

- if the byte count is odd, the last valid byte transferred is on DD(7:0) and the data on DD(15:8) is a pad byte of undefined value;

if the last transfer of a command has a pad byte, the byte count shall be odd.

        - Bus release (overlap feature set only)

After receiving the command packet, the device sets BSY to one and clears DRQ to zero.  If the command packet requires a data transfer, the OVL bit is set to one, the Release interrupt is disabled, and the device is not prepared to immediately transfer data, the device may perform a bus release by placing the following register content in the Command Block registers.  If the command packet requires a data transfer, the OVL bit is set to one, and the Release interrupt is enabled, the device shall perform a bus release by setting the register content as follows.

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | N/A |  |  |  |  |
| 01h | Interrupt Reason |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4 |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be set to one |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. Shall be cleared to zero |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be cleared to zero |  |
| 02h-04h | LBA | N/A |  |  |  |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |
|  |  |  | 15:8 |  | Reserved |  |
|  |  |  | 7 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 6 |  | N/A |  |
|  |  |  | 5 |  | DMA Ready - See clause 6.2.8. Shallbe cleared to zero. |  |
|  |  |  | 4 |  | Service - See clause 6.2.9. |  |
|  |  |  | 3 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 |  | N/A |  |
|  |  |  | 0 |  | Check Condition - See clause 6.2.2. Shall be cleared to zero |  |

        - Service request (overlap feature set only)

When the device is ready to transfer data or complete a command after the command has performed a bus release, the device shall set the SERV bit and not change the state of any other register bit (See 6.2.9).  When the SERVICE command is received, the device shall set outputs as described in data transfer, successful command completion, or error outputs depending on the service the device requires.

        - Successful command completion

When the device has command completion without error, the device returns the data structure found in Table 72.  Release shall be cleared to zero, Input/Output shall be set to one, Command/Data shall be set to one.  Byte Count is reserved at command completion.

      - Error outputs

The device shall not terminate the PACKET command with an error before the last byte of the command packet has been written (See Clause 5.9).

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |
|  |  |  |  | 7:4 | Sense Key - See clause 6.3.11 |  |
|  |  |  |  | 3 | N/A |  |
|  |  |  |  | 2 | Abort - See clause 6.3.1 |  |
|  |  |  |  | 1 | End of Media - See clause 6.3.3 |  |
|  |  |  |  | 0 | Illegal Length Indicator - See clause 6.3.5 |  |
| 01h | Interrupt Reason |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4 |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be cleared to zero |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. Shall be set to one |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be set to one |  |
| 02h-04h | LBA | N/A |  |  |  |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |
|  |  |  | 15:8 |  | Reserved |  |
|  |  |  | 7:6 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 |  | N/A |  |
|  |  |  | 4 |  | Service - See clause 6.2.9. |  |
|  |  |  | 3 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 |  | N/A |  |
|  |  |  | 0 |  | Check Condition - See clause 6.2.2. |  |

### READ BUFFER - E4, PIO data-in

      - Feature Set

This command is optional for devices not implementing the PACKET Command feature set.

      - Description

The READ BUFFER command enables the host to read the current contents of the device’s sector buffer.

The READ BUFFER and WRITE BUFFER commands shall be synchronized such that sequential WRITE BUFFER and READ BUFFER commands access the same 512 bytes within the buffer.

The command prior to a READ BUFFER command shall be a WRITE BUFFER command.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | E4h |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79

### READ DMA - C8h, DMA

      - Feature Set

This command is mandatory for devices not implementing the PACKET Command feature set.

      - Description

The READ DMA command allows the host to read data using the DMA data transfer protocol.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C8h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The LBA field contains the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  LBA bits 47:28 shall be cleared to zero.  See Table 85.

### READ DMA EXT - 25h, DMA

      - Feature Set

This command is mandatory for devices implementing the 48-bit Address feature set.

      - Description

The READ DMA EXT command allows the host to read data using the DMA data transfer protocol.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65536 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C8h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The Command Block registers contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.

### READ DMA QUEUED - C7h, DMA Queued

      - Feature Set

This command is mandatory for devices implementing the Overlapped feature set [Editors Note: This seems strange since we have a Queued command set as well].

      - Description

This command executes in a similar manner to a READ DMA command.  The device may perform a bus release or may execute the data transfer without performing a bus release if the data is ready to transfer.

      - Inputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |  |  |  |
| 01h | Count |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:3 | Tag - See clause 6.4.4 |  |  |  |
|  |  |  | 2:0 | N/A |  |  |  |
| 02h | LBA | MSB |  |  | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  | LSB |  |
| 05h | Command | C7h |  |  |  |  |  |

      - Normal outputs

        - Data transmission

Data transfer may occur after receipt of the command or may occur after the receipt of a SERVICE command.  When the device is ready to transfer data requested by a data transfer command, the device returns the data structure described in Table 73.  Release shall be cleared to zero, Input/Output shall be set to one, and Command/Data shall be cleared to zero.

        - Bus release

If the device performs a bus release before transferring data for this command, the device returns the data structure described in Table 73.  Release shall be set to one, Input/Output shall be cleared to zero, and Command/Data shall be cleared to zero.

        - Service request

When the device is ready to transfer data or complete a command after the command has performed a bus release, the device shall set the SERV bit and not change the state of any other status bit.  When the SERVICE command is received, the device shall set outputs as described in data transfer, command completion, or error outputs depending on the service the device requires.

        - Command completion

When the transfer of all requested data has occurred without error, the device returns the data structure described in Table 73.  Release shall be cleared to zero, Input/Output shall be set to one, and Command/Data shall be set to one.

      - Error outputs

The Count field contains the Tag for this command if the device supports command queuing.  The device shall return command aborted if the command is not supported or if the device has not had overlapped interrupt enabled.  The device shall return command aborted if the device supports command queuing and the Tag is invalid.  An unrecoverable error encountered during the execution of this command results in the termination of the command and the Command Block registers contain the sector where the first unrecoverable error occurred.  If a queue existed, the unrecoverable error shall cause the queue to abort.

| Word | Name | Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |  |
|  |  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |  |  |
|  |  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |  |
|  |  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |  |
|  |  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |  |
|  |  |  |  | 2 | Abort - See clause 6.3.1 |  |  |  |
|  |  |  |  | 1 | No Media - See clause 6.3.10 |  |  |  |
|  |  |  |  | 0 | Obsolete |  |  |  |
| 01h | Interrupt Reasont |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4. If the device supports command queuing, this field shall contain the Tag of the command being bus released. |  |  |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be cleared to zero |  |  |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. Shall be set to one |  |  |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be set to one |  |  |  |
| 02h | LBA | MSB |  |  |  | Address of first unrecoverable error Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |  |  |
|  |  |  | Bit |  |  | Description |  |  |
|  |  |  | 15:8 |  |  | Reserved |  |  |
|  |  |  | 7:6 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 |  |  | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 |  |  | Service - See clause 6.2.9 |  |  |
|  |  |  | 3 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 |  |  | N/A |  |  |
|  |  |  | 0 |  |  | Error - See clause 6.2.3 |  |  |

### READ DMA QUEUED EXT- 26h, DMA Queued

      - Feature Set

This command is mandatory for devices implementing both the Overlapped and 48-bit feature sets [Editors Note: This seems strange since we have a Queued command set as well].

      - Description

This command executes in a similar manner to a READ DMA command.  The device may perform a bus release or may execute the data transfer without performing a bus release if the data is ready to transfer.

      - Inputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |  |  |  |
| 01h | Count |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:3 | Tag - See clause 6.4.4 |  |  |  |
|  |  |  | 2:0 | N/A |  |  |  |
| 02h | LBA | MSB |  |  | Address of first sector to be transferred. |  |  |
| 03h |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  | LSB |  |
| 05h | Command | 26h |  |  |  |  |  |

      - Normal outputs

See clause 7.29.4

      - Error outputs

The Count field contains the Tag for this command if the device supports command queuing.  The device shall return command aborted if the command is not supported or if the device has not had overlapped interrupt enabled.  The device shall return command aborted if the device supports command queuing and the Tag is invalid.  An unrecoverable error encountered during the execution of this command results in the termination of the command and the Command Block registers contain the sector where the first unrecoverable error occurred.  If a queue existed, the unrecoverable error shall cause the queue to abort.

| Word | Name | Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |  |
|  |  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |  |  |
|  |  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |  |
|  |  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |  |
|  |  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |  |
|  |  |  |  | 2 | Abort - See clause 6.3.1 |  |  |  |
|  |  |  |  | 1 | No Media - See clause 6.3.10 |  |  |  |
|  |  |  |  | 0 | Obsolete |  |  |  |
| 01h | Interrupt Reasont |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4. If the device supports command queuing, this field shall contain the Tag of the command being bus released. |  |  |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be cleared to zero |  |  |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. Shall be set to one |  |  |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be set to one |  |  |  |
| 02h | LBA | MSB |  |  |  | Address of first unrecoverable error |  |  |
| 03h |  |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |  |  |
|  |  |  | Bit |  |  | Description |  |  |
|  |  |  | 15:8 |  |  | Reserved |  |  |
|  |  |  | 7:6 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 |  |  | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 |  |  | Service - See clause 6.2.9 |  |  |
|  |  |  | 3 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 |  |  | N/A |  |  |
|  |  |  | 0 |  |  | Error - See clause 6.2.3 |  |  |

### READ LOG EXT - 2Fh, PIO data-in

      - Feature Set

This command is mandatory for devices implementing the General Purpose Logging feature set

      - Description

This command returns the specified log to the host.

        - General Purpose Log Directory

Table 20 defines the 512 bytes that make up the General Purpose Log Directory.

Table 20 - General Purpose Log Directory 20

| Byte | Descriptions |
| --- | --- |
| 0-1 | General Purpose Logging Version |
| 2 | Number of sectors in the log at log address 01h (7:0) |
| 3 | Number of sectors in the log at log address 01h (15:8) |
| 4 | Number of sectors in the log at log address 02h (7:0) |
| 5 | Number of sectors in the log at log address 02h (15:8) |
| … |  |
| 256 | 10h sectors in the log at log address 80h |
| 257 | 00h sectors in the log at log address 80h |
| … |  |
| 510-511 | Number of sectors in the log at log address FFh |

The value of the General Purpose Logging Version word shall be 0001h.  A value of 0000h indicates that no General Purpose log Directory exists.

The logs at log addresses 80-9Fh shall each be defined as 16 sectors long.

        - Extended Comprehensive SMART Error log

Table 21 defines the format of each of the sectors that comprise the Extended Comprehensive SMART error log.  The maximum size of the Extended Comprehensive SMART error log is 65,536 sectors.  Devices may support fewer than 65,535 sectors.  All multi-byte fields shown in this structure follow the byte ordering described in Volume 1, Clause 3.  Error log data structures shall include UNC errors, IDNF errors for which the address requested was valid, servo errors, write fault errors, etc.  Error log data structures shall not include errors attributed to the receipt of faulty commands such as command codes not implemented by the device or requests with invalid parameters or invalid addresses.

All 28-bit entries contained in the Comprehensive SMART log, defined under section 7.54.7.2.4, shall also be included in the Extended Comprehensive SMART error log with the 48-bit entries.

Table 21 - Extended Comprehensive SMART error log 21

| Byte | First sector | Subsequent sectors |
| --- | --- | --- |
| 0 | SMART error log version | Reserved |
| 1 | Reserved | Reserved |
| 2 | Error log index (7:0) | Reserved |
| 3 | Error log index (15:8) | Reserved |
| 4-127 | First error log data structure | Data structure 4n+1 |
| 128-251 | Second error log data structure | Data structure 4n+2 |
| 252-375 | Third error log data structure | Data structure 4n+3 |
| 376-499 | Fourth error log data structure | Data structure 4n+4 |
| 500-501 | Device error count | Reserved |
| 502-510 | Reserved | Reserved |
| 511 | Data structure checksum | Data structure checksum |
| n is the sector number within the log. The first sector is sector zero |  |  |

###### Error log version

The value of the SMART error log version byte shall be 01h.

###### Error log index

The error log index indicates the error log data structure representing the most recent error.  If there have been no error log entries, the error log index is cleared to zero.  Valid values for the error log index are zero to  65,536.

###### Extended Error log data structure

The error log is viewed as a circular buffer.  When the last supported error log sector has been filled, the next error shall create an error log data structure that replaces the first error log data structure in sector zero.  The next error after that shall create an error log data structure that replaces the second error log data structure in sector zero.  The fifth error after the log has filled shall replace the first error log data structure in sector one, and so on.

The error log index indicates the most recent error log data structure.  Unused error log data structures shall be filled with zeros.

The content of the error log data structure entries is defined in Table 22.

Table 22 - Extended Error log data structure 22

| Byte | Descriptions |
| --- | --- |
| n thru n+17 | First command data structure |
| n+18 thru n+35 | Second command data structure |
| n+36 thru n+53 | Third command data structure |
| n+54 thru n+71 | Fourth command data structure |
| n+72 thru n+89 | Fifth command data structure |
| n+90 thru n+123 | Error data structure |

            - Command data structure

The fifth command data structure shall contain the command or reset for which the error is being reported.  The fourth command data structure should contain the command or reset that preceded the command or reset for which the error is being reported, the third command data structure should contain the command or reset preceding the one in the fourth command data structure, etc.  If fewer than four commands and resets preceded the command or reset for which the error is being reported, the unused command data structures shall be zero filled, for example, if only three commands and resets preceded the command or reset for which the error is being reported, the first command data structure shall be zero filled.  In some devices, the hardware implementation may preclude the device from reporting the commands that preceded the command for which the error is being reported or that preceded a reset.  In this case, the command data structures are zero filled.

If the command data structure represents a command or software reset, the content of the command data structure shall be as shown in Table 23.  If the command data structure represents a hardware reset, the content of byte n shall be FFh, the content of bytes n+1 through n+13 are vendor specific, and the content of bytes n+14 through n+17 shall contain the timestamp.

Table 23 - Command data structure23

| Byte | Descriptions |
| --- | --- |
| n | Content of the Device Control register when the Command register was written. |
| n+1 | Content of the Features register (7:0) when the Command register was written. (See note) |
| n+2 | Content of the Features register (15:8) when the Command register was written. |
| n+3 | Content of the Sector Count register (7:0) when the Command register was written. |
| n+4 | Content of the Sector Count register (15:8) when the Command register was written. |
| n+5 | Content of the LBA Low register (7:0) when the Command register was written. |
| n+6 | Content of the LBA Lowregister (15:8) when the Command register was written. |
| n+7 | Content of the LBA Mid register (7:0) when the Command register was written. |
| n+8 | Content of the LBA Mid register (15:8) when the Command register was written. |
| n+9 | Content of the LBA High register (7:0) when the Command register was written. |
| n+10 | Content of the LBA High register (15:8) when the Command register was written. |
| n+11 | Content of the Device/Head register when the Command register was written. |
| n+12 | Content written to the Command register. |
| n+13 | Reserved |
| n+14 | Timestamp (least significant byte) |
| n+15 | Timestamp (next least significant byte) |
| n+16 | Timestamp (next most significant byte) |
| n+17 | Timestamp (most significant byte) |
| NOTE - bits (7:0) refer to the most recently written contents of the register. Bits (15:8) refer to the contents of the register prior to the most recent write to the register. |  |

Timestamp shall be the time since power-on in milliseconds when command acceptance occurred.  This timestamp may wrap around.

            - Error data structure

The error data structure shall contain the error description of the command for which an error was reported as described in Table 24.  If the error was logged for a hardware reset, the content of bytes n+1 through n+11 shall be vendor specific and the remaining bytes shall be as defined in Table 24.

Table 24 - Error data structure24

| Byte | Descriptions |
| --- | --- |
| n | Reserved |
| n+1 | Content of the Error register after command completion occurred. |
| n+2 | Content of the Sector Count register (7:0) after command completion occurred. (see note) |
| n+3 | Content of the Sector Count register (15:8) after command completion occurred. (see note) |
| n+4 | Content of the LBA Low register (7:0) after command completion occurred. |
| n+5 | Content of the LBA Low register (15:8) after command completion occurred. |
| n+6 | Content of the LBA Mid register (7:0) after command completion occurred. |
| n+7 | Content of the LBA Mid register (15:8) after command completion occurred. |
| n+8 | Content of the LBA High register (7:0) after command completion occurred. |
| n+9 | Content of the LBA High register (15:8) after command completion occurred. |
| n+10 | Content of the Device/Head register after command completion occurred. |
| n+11 | Content written to the Status register after command completion occurred. |
| n+12 through n+30 | Extended error information |
| n+31 | State |
| n+32 | Life timestamp (least significant byte) |
| n+33 | Life timestamp (most significant byte) |
| NOTE - bits (7:0) refer to the contents if the register were read with bit 7 of the Device Control register cleared to zero. Bits (15:8) refer to the contents if the register were read with bit 7 of the Device Control register set to one. |  |

Extended error information shall be vendor specific.

State shall contain a value indicating the state of the device when the command was written to the Command register or the reset occurred as described in Table 25.

Table 25 - State field values25

| Value | State |
| --- | --- |
| x0h | Unknown |
| x1h | Sleep |
| x2h | Standby |
| x3h | Active/Idle with BSY cleared to zero |
| x4h | Executing SMART off-line or self-test |
| x5h-xAh | Reserved |
| xBh-xFh | Vendor unique |
| The value of x is vendor specific and may be different for each state. |  |

Sleep indicates the reset for which the error is being reported was received when the device was in the Sleep mode.

Standby indicates the command or reset for which the error is being reported was received when the device was in the Standby mode.

Active/Idle with BSY cleared to zero indicates the command or reset for which the error is being reported was received when the device was in the Active or Idle mode and BSY was cleared to zero.

Executing SMART off-line or self-test indicates the command or reset for which the error is being reported was received when the device was in the process of executing a SMART off-line or self-test.

Life timestamp shall contain the power-on lifetime of the device in hours when command completion occurred.

###### Device error count

The device error count field shall contain the total number of errors attributable to the device that have been reported by the device during the life of the device.  These errors shall include UNC errors, IDNF errors for which the address requested was valid, servo errors, write fault errors, etc.  This count shall not include errors attributed to the receipt of faulty commands such as commands codes not implemented by the device or requests with invalid parameters or invalid addresses.  If the maximum value for this field is reached, the count shall remain at the maximum value when additional errors are encountered and logged.

###### Data structure checksum

The data structure checksum is the two’s complement of the sum of the first 511 bytes in the data structure.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all 512 bytes will be zero when the checksum is correct.  The checksum is placed in byte 511.

        - Extended Self-test log sector

Table 26 defines the format of each of the sectors that comprise the Extended SMART Self-test log.  The maximum size of the self-test log is 65,535 sectors.  Devices may support fewer than 65,536 sectors.  All multi-byte fields shown in this structure follow the byte ordering described in Volume 1, Clause 3.

The Extended SMART self-test log sector shall support 48-bit and 28-bit addressing.  All 28-bit entries contained in the SMART self-test log, defined under section 7.54.7.2.5 shall also be included in the Extended SMART self-test log with all 48-bit entries.

Table 26 - Extended Self-test log data structure26

| Byte | First sector | Subsequent sectors |
| --- | --- | --- |
| 0 | Self-test log data structure revision number | Reserved |
| 1 | Reserved | Reserved |
| 2 | Self-test descriptor index (7:0) | Reserved |
| 3 | Self-test descriptor index (15:8) | Reserved |
| 4-29 | Descriptor entry 1 | Descriptor entry 18n+1 |
| 30-55 | Descriptor entry 2 | Descriptor entry 18n+2 |
| …. | .... | .... |
| 472-497 | Descriptor entry 18 | Descriptor entry 18n+18 |
| 498-499 | Vendor specific | Vendor specific |
| 500-510 | Reserved | Reserved |
| 511 | Data structure checksum | Data structure checksum |
| n is the sector number within the log. The first sector is sector zero |  |  |

This log is viewed as a circular buffer.  When the last supported Self-test log sector has been filled, the next self-test shall create a descriptor that replaces descriptor entry 1 in sector 0.  The next self-test after that shall create a descriptor that replaces descriptor entry 2 in sector 0, and so on.  All unused self-test descriptors shall be filled with zeros.

###### Self-test descriptor index

The Self-test descriptor index indicates the most recent self-test descriptor.  If there have been no self-tests, the Self-test descriptor index is set to zero.  Valid values for the Self-test descriptor index are zero to 65,535.

###### Self-test log data structure revision number

The value of the self-test log data structure revision number shall be 01h.

###### Extended Self-test log descriptor entry

The content of the self-test descriptor entry is shown in Table 27.

Table 27 - Extended Self-test log descriptor entry 27Error! Bookmark not defined.

| Byte | Descriptions |
| --- | --- |
| n | Content of the LBA Low register. |
| n+1 | Content of the self-test execution status byte. |
| n+2 | Life timestamp (least significant byte). |
| n+3 | Life timestamp (most significant byte). |
| n+4 | Content of the self-test failure checkpoint byte. |
| n+5 | Failing LBA (7:0). |
| n+6 | Failing LBA (15:8). |
| n+7 | Failing LBA (23:16). |
| n+8 | Failing LBA (31:24). |
| n+9 | Failing LBA (39:32). |
| n+10 | Failing LBA (47:40). |
| n+1 - n+23 | Vendor specific. |

Content of the LBA Low register shall be the content of the LBA Low register when the nth self-test subcommand was issued (See 7.54.5.1).

Content of the self-test execution status byte shall be the content of the self-test execution status byte when the nth self-test was completed (See 7.54.6.4).

Life timestamp shall contain the power-on lifetime of the device in hours when the nth self-test subcommand was completed.

Content of the self-test failure checkpoint byte may contain additional information about the self-test that failed.

The failing LBA shall be the LBA of the sector that caused the test to fail.  If the device encountered more than one failed sector during the test, this field shall indicate the LBA of the first failed sector encountered.  If the test passed or the test failed for some reason other than a failed sector, the value of this field is undefined.

###### Data structure checksum

The data structure checksum is the two's complement of the sum of the first 511 bytes in the data structure.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all 512 bytes is zero when the checksum is correct.  The checksum is placed in byte 511.

        - Read Stream Error Log

Table 28 defines the format of the Read Stream Error log.  Entries are placed into the Read Stream Error log only when the SE bit is set to one in the Status register.  The 512 bytes returned shall contain a maximum of 31 error entries.  The Read Stream Error Count shall contain the total number of Read Stream Errors detected since the last successful completion of the READ LOG EXT command with LBA Low register set to 22h.  This error count may be greater than 31, but only the most recent 31 errors are represented by entries in the log.  If the Read Stream Error Count reaches the maximum value that can be represented, after the next error is detected the Read Stream Error Count shall remain at the maximum value.  After successful completion of a READ LOG EXT command with the LBA Low Register set to 22h, the Read Stream Error Log shall be reset to a power-on or hardware reset condition, with the Error Log Index and Read Stream Error Count cleared to zero.  The Read Stream Error Log is not preserved across power cycles and hardware reset.

Table 28 - Read Stream Error Log 28Error! Bookmark not defined.

| Byte | Content |
| --- | --- |
| 0 | Structure Version |
| 1 | Error Log Index |
| 2-3 | Read Stream Error Log Count |
| 4-15 | Reserved |
| 16-31 | Read Stream Error Log Entry #1 |
| 32-47 | Read Stream Error Log Entry #2 |
| 48-63 | Read Stream Error Log Entry #3 |
| 64-511 | Read Stream Error Log Entries #4 through #31 |

The Data Structure Version field shall contain a value of 02h indicating the second revision of the structure format.

The Read Stream Error Log Count field shall contain the number of uncorrected sector entries currently reportable to the host.  This value may exceed 31.

The Error Log Index indicates the error log data structure representing the most recent error.  Only values (31:1) are valid.

Table 29 defines the format of each entry in the Read Stream Error Log.

Table 29 - Error Log Entry  29Error! Bookmark not defined.

| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Feature Register Contents Value (current) |  |  |  |  |  |  |  |
| 1 | Feature Register Contents Value (previous) |  |  |  |  |  |  |  |
| 2 | Status Register Contents Value |  |  |  |  |  |  |  |
| 3 | Error Register Contents Value |  |  |  |  |  |  |  |
| 4 | LBA (7:0) |  |  |  |  |  |  |  |
| 5 | LBA (15:8) |  |  |  |  |  |  |  |
| 6 | LBA (23:16) |  |  |  |  |  |  |  |
| 7 | LBA (31:24) |  |  |  |  |  |  |  |
| 8 | LBA (39:32) |  |  |  |  |  |  |  |
| 9 | LBA (47:40) |  |  |  |  |  |  |  |
| 10-11 | Reserved |  |  |  |  |  |  |  |
| 12 | Sector Count (LSB) |  |  |  |  |  |  |  |
| 13 | Sector Count (MSB) |  |  |  |  |  |  |  |
| 14 | Reserved |  |  |  |  |  |  |  |
| 15 | Reserved |  |  |  |  |  |  |  |

Byte (1:0) (Feature Register Contents Value) contains the contents of the Feature Register when the error occurred.  This value shall be set to 0FFFFh for a deferred write error.

Byte 2 (Status Register Contents Value) contains the contents of the Status Register when the error occurred.

Byte 3 (Error Register Contents Value) contains the contents of the Error Register when the error occurred.

Bytes (9:4) (LBA) indicate the starting LBA of the error.

Bytes (13:12) (Sector Count) indicate the length of the error.  Therefore, each entry may describe a range of sectors starting at the given address and spanning the specified number of sectors.

        - Write Stream Error Log

Table 30 defines the format of the Write Stream Error log.  Entries are placed into the Write Stream Error log only when the SE bit is set to one in the Status register.  The 512 bytes returned shall contain a maximum of 31 error entries.  The Write Stream Error Count shall contain the total number of Write Stream Errors detected since the last successful completion of the READ LOG EXT command with LBA Low register set to 21h.  This error count may be greater than 31, but only the most recent 31 errors are represented by entries in the log.  If the Write Stream Error Count reaches the maximum value that can be represented, after the next error is detected the Write Stream Error Count shall remain at the maximum value.  After successful completion of a READ LOG EXT command with the LBA Low Register set to 21h, the Write Stream Error Log shall be reset to a power-on or hardware reset condition, with the Error Log Index and Write Stream Error Count cleared to zero.  The Write Stream Error Log is not preserved across power cycles and hardware reset.

Table 30 - Write Stream Error Log 30Error! Bookmark not defined.

| Byte | Content |
| --- | --- |
| 0 | Structure Version |
| 1 | Error Log Index |
| 2-3 | Write Stream Error Log Count |
| 4-15 | Reserved |
| 5-7 | Reserved |
| 16-31 | Write Stream Error Log Entry #1 |
| 32-47 | Write Stream Error Log Entry #2 |
| 48-63 | Write Stream Error Log Entry #3 |
| 64-511 | Write Stream Error Log Entries #4 through #31 |

The Data Structure Version field shall contain a value of 02h indicating the second revision of the structure format.

The Write Stream Error Log Count field shall contain the number of WRITE STREAM command entries since the last power on, since this log was last read, or since a hardware reset was executed.

The Error Log Index indicates the error log data structure representing the most recent error.  Only values (31:0) are valid.

Table 29 defines the format of each entry in the Error Log.

        - Streaming Performance Log

Table 31, Table 32, Table 33, and Table 34 define the format of the log returned by the READ LOG EXT command, when the LBA Low register is 20h.  This data set is referred to as the Streaming Performance Parameters log, the length of which (in sectors) is statically indicated in READ LOG EXT log address 00h (Log Directory).

The contents of Streaming Performance Parameters Log may be affected by the host issuing a SET FEATURES subcommand 42h, C2h, or 43h and may also affect the Delayed LBA log.

NOTE  The host should check the content of the Streaming Performance Parameters log and the Delayed LBA log after issuing SET FEATURES subcommands 42h, 43h, or C2h.

The host should base its calculations on the larger of its Typical Host Interface Sector Time and the device reported Sector Time values, and on the sum of the device reported Access Time values and any additional latency that only the host is aware of (e.g., host command overhead, etc).

Table 31 - Streaming Performance Parameters Log 31Error! Bookmark not defined.

| Bytes | Description |
| --- | --- |
| 2 | Stream Performance Parameters log version |
| 2 | K, Number of Regions in Sector Time Array |
| 2 | L, Number of Positions in Position Array |
| 2 | M, Number of Position-differences in Access Time Array |
| K*8 | Sector Time Array (See Table 32) |
| L*8 | Position Array (See Table 33) |
| M*4 | Access Time Array (See Table 34) |
| Last sector remainder | Reserved |

Table 32 - Sector Time Array Entry (Linearly Interpolated) 32Error! Bookmark not defined.

| Byte | Descriptions |
| --- | --- |
| n-(n+5) | Logical Block Address of reference location (LBA(7:0)…LBA(47:40)) |
| (n+6)-(n+7) | (IDENTIFY DEVICE data words (99:98))/65536 time units per sector at the reference location |

Table 33 - Position Array Entry (Linearly Interpolated) 33Error! Bookmark not defined.

| Byte | Descriptions |
| --- | --- |
| n-(n+5) | Logical Block Address of start of region (LBA(7:0)…LBA(47:40)) |
| (n+6)-(n+7) | Position number in the range 0…32767 |

Table 34 - Access Time Array Entry (Linearly Interpolated) 34Error! Bookmark not defined.

| Byte | Descriptions |
| --- | --- |
| n-(n+1) | Difference in position from last stream access to new stream access |
| (n+2)-(n+3) | Time that may be required to begin access at new stream access position, in (IDENTIFY DEVICE data words (89:88))/256 time units. |

        - Delayed LBA Log

Table 35 defines the format of each of the sectors that comprise the Delayed LBA Log.  The maximum size of the Delayed LBA Log is vendor specific.  The alternate physical location, access method, or access time for a Delayed LBA are vendor specific.

If the maximum size of the Delayed LBA Log is reached and an additional Delayed LBA is detected by the device, the most recent Delayed LBA shall not be added to the log.

The device may add entries to the log at any time.  The device shall not remove entries from this log.  The log is returned to the host ordered by timestamp, the most recently added entry is last.

The Delayed LBA Log is non-volatile, it is preserved across power cycles and hardware reset.

Table 35 -  Delayed LBA log 35Error! Bookmark not defined.

| Byte | First Sector | Subsequent Sectors |
| --- | --- | --- |
| 0 | Delayed LBA Log Version | Reserved |
| 1 | Reserved | Reserved |
| 2-3 | Number of Delayed LBA entries | Reserved |
| 4-9 | LBA (7:0) of 1st Delayed LBA | LBA of (63n+1) Delayed LBA |
|  | LBA (15:8) |  |
|  | LBA (23:16) |  |
|  | LBA (31:24) |  |
|  | LBA (39:32) |  |
|  | LBA (47:40) |  |
| 10 | Life Timestamp (7:0) of 1st Delayed LBA | Life Timestamp (7:0) of (63n+1) Delayed LBA |
| 11 | Life Timestamp (15:8) of 1st Delayed LBA | Life Timestamp (15:8) of (63n+1) Delayed LBA |
| 12-17 | LBA of 2nd Delayed LBA | LBA of (63n+2) Delayed LBA |
| 18-19 | Life Timestamp of 2nd Delayed LBA | Reserved |
| ... | ... | ... |
| 500-505 | LBA of 63rd Delayed LBA | LBA of (63n+63) Delayed LBA |
| 506-507 | Life Timestamp of 63rd Delayed LBA | Life Timestamp of (63n+63) Delayed LBA |
| 508-510 | Reserved | Reserved |
| 511 | Data Structure Checksum | Data Structure Checksum |
| n is the sector number within the log. The first sector is sector 0. |  |  |

###### Delayed LBA Log Version

The value of the Delayed LBA Log Version shall be set to 01h.

###### Number of Delayed LBA entries

The Number of Delayed LBA entries shall contain the total count of Delayed LBA entries, and shall be consistent with the number of LBA entries in the Delayed LBA Log.  If the maximum value for this field is reached (corresponding to the vendor-specific maximum number of sectors in the log), the count shall remain at the maximum value when additional Delayed LBA entries are added.

###### Life Timestamp

The Life Timestamp shall contain the power-on lifetime of the device, in hours, when the sector was entered into the log.

      - Inputs

All Log Addresses in this standard reserve the Features register.

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | Log Address Specific [Editors Note, I think this means that all the log pages need to be modified to state the value of the features register] |  |  |  |
| 01h | Count | Sector Count - Specifies the number of sectors to be read from the specified log. The log transferred by the drive shall start at the sector in the specified log at the specified offset, regardless of the sector count requested |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:32 | Reserved |  |
|  |  |  | 31:16 | Sector Offset - Specifies the first sector of the log to be read. |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:0 | Log Address - Specifies the log to be returned as described in Table 36. A device may support a subset of the available logs. Support for individual logs is determined by support for the associated feature set. Support of the associated log(s) is mandatory for devices implementing the associated feature set. The host vendor specific logs may be used by the host to store any data desired. If a host vendor specific log has never been written by the host, when read the content of the log shall be zeros. Device vendor specific logs may be used by the device vendor to store any data and need only be implemented if used. |  |
| 05h | Command | 2Fh |  |  |  |

Table 36 - Log address definition 36

| Log address | Content | Feature set | R/W | Access |
| --- | --- | --- | --- | --- |
| 00h | Log directory | na | RO | GPL,SL |
| 01h | Summary SMART error log | SMART | RO | SL |
| 02h | Comprehensive SMART error log | SMART error logging | RO | SL |
| 03h | Extended Comprehensive SMART error log | SMART error logging | RO | GPL |
| 04h-05h | Reserved | na | Reserved | GPL |
| 06h | SMART self-test log | SMART self-test | RO | SL |
| 07h | Extended SMART self-test log | SMART self-test | RO | GPL |
| 08h | Reserved | na | Reserved |  |
| 09h | Selective Self-test log | SMART Self-test | R/W | SL |
| 0Ah-0Fh | Reserved | na | Reserved |  |
| 10h-17h | Reserved for Serial ATA | na | Reserved |  |
| 18h-1Fh | Reserved | na | Reserved |  |
| 20h | Streaming performance log | Streaming | RO | GPL |
| 21h | Write stream error log | Streaming | RO | GPL |
| 22h | Read stream error log | Streaming | RO | GPL |
| 23h | Delayed sector log | General Purpose Logging | RO | GPL |
| 24h-7Fh | Reserved | na | Reserved |  |
| 80h-9Fh | Host vendor specific | SMART | R/W | GPL,SL |
| A0h-BFh | Device vendor specific | SMART | VS | GPL,SL |
| C0h-EFh | Reserved | na | Reserved |  |
| E0h-E1h | Reserved for SCT [Editors Note: Need to fill in project number] | na | Reserved |  |
| E2h-FFh | Reserved | na |  |  |
| Key  RO - Log is only read by the host. R/W - Log is read or written by the host. VS - Log is vendor specific thus read/write ability is vendor specific. GPL - General Purpose Logging SL - SMART Logging NOTE  Command Abort shall be returned if a GPL command is used to access a log page that is marked with SL only. Command Abort shall be returned if an SL command is used to access a log page that is marked with GPL only. |  |  |  |  |

The Comprehensive SMART error log and the SMART self-test log are defined in 7.54.7 and 7.54.9.  If log address 02h or log address 06h are accessed using the READ LOG EXT or WRITE LOG EXT commands, command abort shall be returned.

All 28-bit entries contained in the Comprehensive SMART log shall also be included in the Extended Comprehensive SMART error log with the 48-bit entries.

The Extended SMART self-test log sector shall support 48-bit and 28-bit addressing.  All 28-bit entries contained in the SMART self-test log sector shall also be included in the Comprehensive SMART self-test log sector with the 48-bit entries.

      - Normal outputs

See Table 65

      - Error outputs

If the device does not support this command, if the feature set associated with the log specified in the LBA Low register is not supported or enabled, or if the values in the Features, Sector Count, LBA Mid, or LBA High registers are invalid, the device shall return command aborted.  Abort shall be set to one if the feature associated with the log specified in the  LBA bits 7:0 is not supported, or if other register values are invalid.  Abort may be set to one if the device is not able to complete the action requested by the command.  Abort shall be set to one if the Sector Count is larger than the log size reported in the Log Directory.  Abort shall be set to one if the host issues a READ LOG EXT or WRITE LOG EXT command with a value of zero as Sector Count.  See Table 86.

### READ MULTIPLE - C4h, PIO data-in

      - Feature Set

This command is mandatory for devices implementing the General Feature Set.

      - Description

This command reads the number of sectors specified in the Sector Count register.

The number of sectors per block is defined by the content of word 59 in the IDENTIFY DEVICE data.  The device shall interrupt for each DRQ block transferred.

When the READ MULTIPLE command is issued, the Sector Count register contains the number of sectors (not the number of blocks) requested.

If the number of requested sectors is not evenly divisible by the block count, as many full blocks as possible are transferred, followed by a final, partial block transfer.  The partial block transfer shall be for n sectors, where n = remainder (sector count/ block count).

If the READ MULTIPLE command is received when READ MULTIPLE commands are disabled, the READ MULTIPLE operation shall be rejected with command aborted.

Device errors encountered during READ MULTIPLE commands are posted at the beginning of the block or partial block transfer, but the DRQ bit is still set to one and the data transfer shall take place, including transfer of corrupted data, if any.  The contents of the Command Block Registers following the transfer of a data block that had a sector in error are undefined.  The host should retry the transfer as individual requests to obtain valid error information.

Subsequent blocks or partial blocks are transferred only if the error was a correctable data error.  All other errors cause the command to stop after transfer of the block that contained the error.

If bit 8 of IDENTIFY DEVICE data word 59 is cleared to zero, a successful SET MULTIPLE MODE command shall precede a READ MULTIPLE command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C4h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The LBA field contains the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  LBA bits 47:28 shall be cleared to zero.  See Table 87.

### READ MULTIPLE EXT - 29h, PIO data-in

      - Feature Set

This command is mandatory for all devices implementing the 48-bit Address feature set.

      - Description

This command reads the number of sectors specified in the Sector Count register.

The number of sectors per block is defined by a successful SET MULTIPLE command.  If no successful SET MULTIPLE command has been issued, the block is defined by the device’s default value for number of sectors per block as defined in bits (7:0) in word 47 in the IDENTIFY DEVICE data.  The device shall interrupt for each DRQ block transferred.

When the READ MULTIPLE EXT command is issued, the Sector Count register contains the number of sectors (not the number of blocks) requested.

If the number of requested sectors is not evenly divisible by the block count, as many full blocks as possible are transferred, followed by a final, partial block transfer.  The partial block transfer shall be for n sectors, where n = remainder (sector count/ block count).

If the READ MULTIPLE EXT command is received when READ MULTIPLE commands are disabled, the READ MULTIPLE operation shall be rejected with command aborted.

Device errors encountered during READ MULTIPLE EXT commands are posted at the beginning of the block or partial block transfer, but the DRQ bit is still set to one and the data transfer shall take place, including transfer of corrupted data, if any.  The contents of the Command Block Registers following the transfer of a data block that had a sector in error are undefined.  The host should retry the transfer as individual requests to obtain valid error information.

Subsequent blocks or partial blocks are transferred only if the error was a correctable data error.  All other errors cause the command to stop after transfer of the block that contained the error.

If bit 8 of IDENTIFY DEVICE data word 59 is cleared to zero, a successful SET MULTIPLE MODE command shall precede a READ MULTIPLE command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 29h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The LBA field contains the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  See Table 87

### READ NATIVE MAX ADDRESS - F8h, Non-data

      - Feature Set

This command is mandatory for devices implementing the Host Protected Area feature set.  Use of this command is prohibited for devices implementing the Removable feature set.

      - Description

This command returns the native maximum address.  The native maximum address is the highest address accepted by the device in the factory default condition.  The native maximum address is the maximum address that is valid when using the SET MAX ADDRESS command.

If the 48-bit Address feature set is supported and the 48-bit native max address is greater than 268,435,455, the READ NATIVE MAX ADDRESS command shall return a maximum value of 268,435,454.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F8h |

      - Normal outputs

See Table 74.  LBA contains the Native Max Address.  Bits 47:28 of LBA shall be cleared to zero.

      - Error outputs

See Table 79

### READ NATIVE MAX ADDRESS EXT - 27h, Non-data

      - Feature Set

This command is mandatory for devices implementing both the Host Protected Area feature set and the 48-bit Address feature set.  Use of this command is prohibited for devices implementing the Removable feature set.

      - Description

This command returns the native maximum address.  The native maximum address is the highest address accepted by the device in the factory default condition.  The native maximum address is the maximum address that is valid when using the SET MAX ADDRESS EXT command.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | 27h |

      - Normal outputs

See Table 74.  LBA contains the Native Max Address.

      - Error outputs

See Table 79.

### READ SECTOR(S) - 20h, PIO data-in

      - Feature Set

This command is mandatory for all devices implementing the General Feature Set.

      - Description

This command reads from 1 to 256 sectors as specified in the Sector Count register.  A sector count of 0 requests 256 sectors.  The transfer shall begin at the sector specified in the LBA Low, LBA Mid, LBA High, and Device registers.  The device shall interrupt for each DRQ block transferred.

The DRQ bit is always set to one prior to data transfer regardless of the presence or absence of an error condition.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 20h |  |  |

      - Outputs

        - Normal outputs

See Table 65

        - Outputs for PACKET Command feature set devices

In response to this command, devices that implement the PACKET Command feature set shall post command aborted and place the PACKET Command feature set signature in the LBA High and the LBA Mid register (See 7.11 for a list of the possible signatures).

      - Error outputs

LBA bits 47:28 shall be cleared to zero.  See Table 87.

### READ SECTOR(S) EXT - 24h, PIO data-in

      - Feature Set

This command is mandatory for devices implementing the 48-bit Address feature set

      - Description

This command reads from 1 to 65,536 sectors as specified in the Sector Count register.  A sector count of 0000h requests 65,536 sectors.  The transfer shall begin at the sector specified in the LBA Low, LBA Mid, and LBA High registers.  The device shall interrupt for each DRQ block transferred.

The DRQ bit is always set to one prior to data transfer regardless of the presence or absence of an error condition.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 24h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 87

### READ STREAM DMA EXT - 2Ah, DMA

      - Feature Set

This command is mandatory for devices that implement the Streaming feature set.

      - Description

The command reads from 1 to 65536 sectors as specified in the Sector Count register.  A value of 0000h in the Sector Count register requests 65536 sectors.

The RC bit indicates that the drive operate in a continuous read mode for the Read Stream command.  When RC is cleared to zero the drive shall operate in normal Streaming read mode.

When the Read Continuous mode is enabled, the device shall transfer data of the requested length without setting the ERR bit to one.  The SE bit shall be set to one if the data transferred includes errors.  The data may be erroneous in this case.  If an error is encountered, it may be necessary for the device to pad the data being transferred in order to fulfill the host’s requested transfer size.  The implementation of the padding is vendor specific.

If the Read Continuous bit is set to one,the device shall not stop execution of the command due to errors.  If thr RC bit is set to one and errors occur in reading or transfer of the data, the device shall continue to transfer the amount of data requested and then provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and the type of error, ICRC, UNC, IDNF, or ABRT, reported in the error log.  If the RC bit is set to one and the Command Completion Time Limit expires, the device shall stop execution of the command and provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and report the fact that the Command Completion Time Limit expired by setting the CCTO bit in the error log to one.  In all cases, the device shall attempt to transfer the amount of data requested within the Comand Completion Time Limit even if some data transferred is in error.

      - Inputs

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Command Completion Time Limit (CCTO)- See clause 7.38.4. |  |  |
|  |  |  | 7 | Urgent - The Urgent bit specifies that the command should be completed in the minimum possible time by the device and shall be completed within the specified Command Completion Time Limit. This bit is optional (See 7.17.7.42) |  |  |
|  |  |  | 6 | Read Continuous (RC) - See clause 7.38.5 |  |  |
|  |  |  | 5 | Not Sequential (NS) - If the NS bit is set to one the next read stream command with the same Stream ID may not be sequential in LBA space. Any read of the device media or internal device buffer management as a result of the state of the NS bit is device vendor specific. |  |  |
|  |  |  | 4 | Handle Streaming Error (HSE) - If HSE is set to one this command starts at the LBA of the last reported error for this stream, so the device may attempt to continue its corresponding error recovery sequence where it left off earlier |  |  |
|  |  |  | 3 | Reserved |  |  |
|  |  |  | 2:0 | Stream ID - The stream to be read. The device shall operate according to the Stream ID set by the READ STREAM command. Any read of the device media or internal device buffer management as a result of the Stream ID is device vendor specific. |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65536 sectors are to be transferred. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first sector to be transferred. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Command | 2Ah |  |  |  |  |

      - Command Completion Time Limit (CCTO)

CCTO is the time allowed for the current command’s completion.  CCTO is calculated as follows:

CCTO = (content of the Features register Previous)  (IDENTIFY DEVICE data words (99:98)) microseconds

If the value is zero, the device shall use the Default Command Completion Time Limit supplied with a previous CONFIGURE STREAM command for this Stream ID.  If the Default Command Completion Time Limit is zero, or no previous Configure Stream command was defined for this Stream ID, the result is vendor specific.  The time is measured from the write of the command register to the final INTRQ for command completion.

      - Read Continuous

If the RC bit is set to one,the device shall not stop execution of the command due to errors.

If the RC bit is set to one and errors occur in reading or transfer of the data, the device shall continue to transfer the amount of data requested and then provide ending status with the Stream Error bit set to one, the Error bit cleared to zero, and the type of error, ICRC, UNC, IDNF, or ABRT, reported in the error log.

If the RC bit is set to one and the Command Completion Time Limit expires, the device shall stop execution of the command and provide ending status with the Stream Error bit set to one, the Error bit cleared to zero, and report the fact that the Command Completion Time Limit expired by setting the CCTO bit in the error log to one.

In all cases, the device shall attempt to transfer the amount of data requested within the Comand Completion Time Limit even if some data transferred is in error.

      - Normal outputs

See Table 69

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  The Error bit shall be set to one if an Error field bit is set to one and the RC bit is cleared to zero.  If the RC bit is set to one when the command is issued and an ICRC UNC, IDNF, ABRT, or CCTO error occurs, the SE bit shall be set to one, the ERR bit shall be cleared to zero, and the bits that would normally be set in the Error register shall be set in the error log.  Bits 47:28 of the LBA field shall be cleared to zero.  See Table 88

### READ STREAM EXT - 2Bh, PIO data-in

[Editors Note: We could save a tree and refer to 7.38.  This is a duplicate other than the DMA part which only resides in the header and as a bit in the error return section]

      - Feature Set

This command is mandatory for devices that implement the Streaming feature set.

      - Description

The command reads from 1 to 65536 sectors as specified in the Sector Count register.  A sector count of  value 0000h requests 65,536 sectors.  The transfer shall begin at the sector specified in the Sector Number register.

The RC bit specifies that the drive operate in a continuous read mode for the Read Stream command.  When RC is cleared to zero the drive shall operate in normal Streaming read mode.

When the Read Continuous mode is enabled, the device shall transfer data of the requested length without setting the error bit.  The SE bit shall be set to one if the data transferred includes errors.  The data may be erroneous in this case.  If an error is encountered, it may be necessary for the device to pad the data being transferred in order to fulfill the host’s requested transfer size.  The implementation of the padding is vendor specific.

The DRQ bit is always set to one prior to data transfer regardless of the presence or absence of an error condition.

If the Read Continuous bit is set to one,the device shall not stop execution of the command due to errors.  If thr RC bit is set to one and errors occur in reading or transfer of the data, the device shall continue to transfer the amount of data requested and then provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and the type of error, UNC, IDNF, or ABRT, reported in the error log.  If the RC bit is set to one and the Command Completion Time Limit expires, the device shall stop execution of the command and provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and report the fact that the Command Completion Time Limit expired by setting the CCTO bit in the error log to one.  In all cases, the device shall attempt to transfer the amount of data requested within the Comand Completion Time Limit even if some data transferred is in error.

      - Inputs

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Command Completion Time Limit (CCTO)- See clause 7.38.4. |  |  |
|  |  |  | 7 | Urgent - The Urgent bit specifies that the command should be completed in the minimum possible time by the device and shall be completed within the specified Command Completion Time Limit. This bit is optional (See 7.17.7.42) |  |  |
|  |  |  | 6 | Read Continuous (RC) - See clause 7.38.5 |  |  |
|  |  |  | 5 | Not Sequential (NS) - If the NS bit is set to one the next read stream command with the same Stream ID may not be sequential in LBA space. Any read of the device media or internal device buffer management as a result of the state of the NS bit is device vendor specific. |  |  |
|  |  |  | 4 | Handle Streaming Error (HSE) - If HSE is set to one this command starts at the LBA of the last reported error for this stream, so the device may attempt to continue its corresponding error recovery sequence where it left off earlier |  |  |
|  |  |  | 3 | Reserved |  |  |
|  |  |  | 2:0 | Stream ID - The stream to be read. The device shall operate according to the Stream ID set by the READ STREAM command. Any read of the device media or internal device buffer management as a result of the Stream ID is device vendor specific. |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65536 sectors are to be transferred. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first sector to be transferred. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Command | 2Bh |  |  |  |  |

      - Normal outputs

See Table 69

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  The Error bit shall be set to one if an Error field bit is set to one and the RC bit is cleared to zero.  If the RC bit is set to one when the command is issued and an ICRC UNC, IDNF, ABRT, or CCTO error occurs, the SE bit shall be set to one, the ERR bit shall be cleared to zero, and the bits that would normally be set in the Error register shall be set in the error log.  See Table 88.

### READ VERIFY SECTOR(S) - 40h, Non-data

      - Feature Set

This command is mandatory for all devices that implement the General Feeature set

      - Description

This command is identical to READ SECTOR(S) command, except that no data is transferred from the device to the host.  The device shall read the data from the media and verify that there are no errors.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 40h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

LBA bits 47:28 shall be cleared to zero.  See Table 87.

### READ VERIFY SECTOR(S) EXT - 42h, Non-data

      - Feature Set

This command is mandatory for devices implementing the 48-bit Address feature set

      - Description

This command is identical to READ SECTOR(S) EXT command, except that no data is transferred from the device to the host.  The device shall read the data from the media and verify that there are no errors.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | 42h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 87.

### SECURITY DISABLE PASSWORD - F6h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the Security Mode feature set.

      - Description

The SECURITY DISABLE PASSWORD command transfers 512 bytes of data from the host.  Table 37 defines the content of this information.  If the password selected by word 0 matches the password previously saved by the device, the device shall disable the Lock mode.  This command shall not change the Masterpassword.  The Master password shall be reactivated when a User password is set(See 4.7).

Table 37 - Security password content 37Error! Bookmark not defined.

| Word | Content |  |  |
| --- | --- | --- | --- |
| 0 | Control word |  |  |
|  | Bit 0 | Identifier | 0=compare User password |
|  |  |  | 1=compare Masterpassword |
|  | Bit (15:1) | Reserved |  |
| 1-16 | Password (32 bytes) |  |  |
| 17-255 | Reserved |  |  |

This command shall only complete successfully if the Device is in Unlocked mode.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F6h |

      - Normal outputs

See Table 65

      - Error outputs

The device shall return command aborted if the command is not supported, the device is in Locked mode, or the device is in Frozen mode.  See Table 79.

### SECURITY ERASE PREPARE - F3h, Non-data

      - Feature Set

This command is mandatory for devices that implement the Security Mode feature set.

      - Description

The SECURITY ERASE PREPARE command shall be issued immediately before the SECURITY ERASE UNIT command to enable device erasing and unlocking.  This command prevents accidental loss of data on the device.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F3h |

      - Normal outputs

See Table 65

      - Error outputs

Abort shall be set to one if the device is in Frozen mode.  See Table 79.

### SECURITY ERASE UNIT - F4h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the Security Mode feature set.

      - Description

This command transfers 512 bytes of data from the host.  Table 38 defines the content of this information.  If the password does not match the password previously saved by the device, the device shall reject the command with command aborted.

The SECURITY ERASE PREPARE command shall be completed immediately prior to the SECURITY ERASE UNIT command.  If the device receives a SECURITY ERASE UNIT command without an immediately prior SECURITY ERASE PREPARE command, the device shall command abort the SECURITY ERASE UNIT command.

When Normal Erase mode isspecified, the SECURITY ERASE UNIT command shall write binary zeroes to all user data areas.  The Enhanced Erase mode is optional.  When Enhanced Erase mode isspecified, the device shall write predetermined data patterns to all user data areas.  In Enhanced Erase mode, all previously written user data shall be overwritten, including sectors that are no longer in use due to reallocation.

This command shall disable the device Lock mode, however, the Masterpassword shall still be stored internally within the device and may be reactivated later when a new User password is set.

Table 38 - SECURITY ERASE UNIT password 38Error! Bookmark not defined.

| Word | Content |  |  |
| --- | --- | --- | --- |
| 0 | Control word |  |  |
|  | Bit 0 | Identifier | 0=Compare User password |
|  |  |  | 1=Compare Master password |
|  | Bit 1 | Erase mode | 0=Normal Erase |
|  |  |  | 1=Enhanced Erase |
|  | Bit (15:2) | Reserved |  |
| 1-16 | Password (32 bytes) |  |  |
| 17-255 | Reserved |  |  |

This command shall be immediately preceded by a SECURITY ERASE PREPARE command.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F4h |

      - Normal outputs

See Table 65

      - Error outputs

Abort shall be set to one if the device is in Frozen mode, not preceded by a SECURITY ERASE PREPARE command, or if the data area is not successfully overwritten.  See Table 79.

### SECURITY FREEZE LOCK - F5h, Non-data

      - Feature Set

This command is mandatory for devices that implement Security Mode feature set.

      - Description

The SECURITY FREEZE LOCK command shall set the device to Frozen mode.  After command completion any other commands that update the device Lock mode shall be command aborted.  Frozen mode shall be disabled by power-off or hardware reset.  If SECURITY FREEZE LOCK is issued when the device is in Frozen mode, the command executes and the device shall remain in Frozen mode.

Commands disabled by SECURITY FREEZE LOCK are:

- SECURITY SET PASSWORD

- SECURITY UNLOCK

- SECURITY DISABLE PASSWORD

- SECURITY ERASE PREPARE

- SECURITY ERASE UNIT

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F5h |

      - Normal outputs

See Table 65

      - Error outputs

Abort shall be set to one if the device is in Frozen mode.  See Table 79.

### SECURITY SET PASSWORD - F1h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the Security Mode feature set.

      - Description

This command transfers 512 bytes of data from the host.  Table 39 defines the content of this information.  The data transferred controls the function of this command.  Table 40 defines the interaction of the identifier and security level bits.

The revision code field shall be returned in the IDENTIFY DEVICE data word 92.  The valid revision codes are 0001h through FFFEh.  A value of 0000h or FFFFh indicates that the Master Password Revision Code is not supported.

Table 39 - SECURITY SET PASSWORD data content 39Error! Bookmark not defined.

| Word | Content |  |  |
| --- | --- | --- | --- |
| 0 | Control word |  |  |
|  | Bit 0 | Identifier | 0=set User password |
|  |  |  | 1=set Master password |
|  | Bits (7:1) | Reserved |  |
|  | Bit 8 | Security level | 0=High |
|  |  |  | 1=Maximum |
|  | Bits (15:9) | Reserved |  |
| 1-16 | Password (32 bytes) |  |  |
| 17 | Master Password Revision Code (valid if word 0 bit 0 = 1) |  |  |
| 18-255 | Reserved |  |  |

Table 40 - Identifier and security level bit interaction 40Error! Bookmark not defined.

| Identifier | Level | Command result |
| --- | --- | --- |
| User | High | The password supplied with the command shall be saved as the new User password. The Lock mode shall be enabled from the next power-on or hardware reset. The device shall then be unlocked by either the User password or the previously set Master password. |
| User | Maximum | The password supplied with the command shall be saved as the new User password. The Lock mode shall be enabled from the next power-on or hardware reset. The device shall then be unlocked by only the User password. The Master password previously set is still stored in the device but shall not be used to unlock the device. |
| Master | High or Maximum | This combination shall set a Master password but shall not enable or disable the Lock mode. The security level is not changed. Master password revision code set to the value in Master Password Revision Code field. |

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F1h |

      - Normal outputs

See Table 65

      - Error outputs

Abort shall be set to one if the device is in Frozen mode.  See Table 79.

### SECURITY UNLOCK - F2h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the Security Mode feature set.

      - Description

This command transfers 512 bytes of data from the host.  Table 37 defines the content of this information.

If the Identifier bit is set to Master and the device is in high security level, then the password supplied shall be compared with the stored Master password.  If the device is in maximum security level then the unlock shall be rejected.

If the Identifier bit is set to user then the device shall compare the supplied password with the stored User password.

If the password compare fails then the device shall return command aborted to the host and decrements the unlock counter.  This counter shall be initially set to five and shall be decremented for each password mismatch when SECURITY UNLOCK is issued and the device is locked.  When this counter reaches zero then SECURITY UNLOCK and SECURITY ERASE UNIT commands shall be command aborted until a power-on reset or a hardware reset.  SECURITY UNLOCK commands issued when the device is unlocked have no effect on the unlock counter.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F2h |

      - Normal outputs

See Table 65

      - Error outputs

Abort shall be set to one if the device is in Frozen mode.  See Table 79.

### SERVICE - A2h, Packet or DMA queued

      - Feature Set

This command is mandatory for devices that implement the Overlapped

      - Description

The SERVICE command is used to provide data transfer and/or status of a command that was previously bus released.

The device shall have performed a bus release for a previous overlap PACKET, READ DMA QUEUED, READ DMA QUEUED EXT, WRITE DMA QUEUED, or WRITE DMA QUEUED EXT command and shall have set the Service (see 6.2.9) bit to one to request the SERVICE command be issued to continue data transfer and/or provide command status (See 7.49.17).

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | A2h |

      - Outputs

Outputs as a result of a SERVICE command are described in the command description for the command for which SERVICE is being requested.

### SET FEATURES - EFh, Non-data

      - Feature Set

This command is mandatory for all devices.

#### Description

The set transfer mode subcommand is mandatory.  Enable/disable write cache subcommands are mandatory when a write cache is implemented.  Enable/disable Media Status Notification sub commands are mandatory if the Removable Media feature set is implemented.  All other subcommands are optional.

This command is used by the host to establish parameters that affect the execution of certain device features.  Table 41 defines these features.

At power-on, or after a hardware reset, the default settings of the functions specified by the subcommands are vendor specific.

Table 41 - SET FEATURES register definitions 41Error! Bookmark not defined.

| Value | [Editors Note: Sync with historical annex] |
| --- | --- |
| 00h | Reserved |
| 01h | Enable 8-bit PIO transfer mode (CFA feature set only) |
| 02h | Enable write cache |
| 03h | Set transfer mode based on value in Sector Count register. Table 42 defines values. |
| 04h | Obsolete |
| 05h | Enable advanced power management |
| 06h | Enable Power-Up In Standby feature set. |
| 07h | Power-Up In Standby feature set device spin-up. |
| 08h | Reserved |
| 09h | Reserved for Address offset reserved area boot method technical report [Editors Note: This actually has a function in CFA, add the definition] |
| 0Ah | Enable CFA power mode 1 |
| 0Bh | Enable Write Read Verify feature set |
| 0Ch-0Fh | Reserved |
| 10h | Reserved for Serial ATA |
| 11h-1Fh | Reserved |
| 20h | Reserved for technical report INCITS TR-37-2004 (TLC) |
| 21h | Reserved for technical report INCITS TR-37-2004 (TLC) |
| 22h-30h | Reserved |
| 31h | Disable Media Status Notification |
| 32h | Reserved |
| 33h | Obsolete |
| 34h-41h | Reserved |
| 42h | Enable Automatic Acoustic Management feature set |
| 43h | Set Maximum Host Interface Sector Times |
| 44h | Obsolete |
| 45h-53h | Reserved |
| 54h | Obsolete |
| 55h | Disable read look-ahead feature |
| 56h-5Ch | Vendor Specific |
| 5Dh | Enable release interrupt |
| 5Eh | Enable SERVICE interrupt |
| 5Fh | Reserved for DRQ technical report |
| 60h-65h | Reserved |
| 66h | Disable reverting to power-on defaults |
| 67h-68h | Reserved |
| 69h | [Editors Note: CFA 2.1 defines this as a NOP for ?backwards compatability?] |
| 6Ah-76h | Reserved |
| 77h | Obsolete |
| 78h-80h | Reserved |
| 81h | Disable 8-bit PIO transfer mode (CFA feature set only) |
| 82h | Disable write cache |
| 83h | Reserved |
| 84h | Obsolete |
| 85h | Disable advanced power management |
| 86h | Disable Power-Up In Standby feature set. |
| 87h | Reserved |
| 88h | Obsolete |
| 89h | Reserved for Address offset reserved area boot method technical report. [Editors Note: This actually has a function in CFA, add both definitions] |
| 8Ah | Disable CFA power mode 1 |
| 8Bh | Disable Write Read Verify feature set |
| 8Ch-8Fh | Reserved |
| 90h | Reserved for Serial ATA |
| 91h-94h | Reserved |
| 95h | Enable Media Status Notification |
| 96h-99h | Reserved |
| 99h | Obsolete |
| 9Ah | Obsolete |
| 9Bh-A9h | Reserved |
| AAh | Enable read look-ahead feature |
| ABh | Obsolete |
| ACh-BAh | Reserved |
| BBh | Obsolete |
| BCh-C1h | Reserved |
| C2h | Disable Automatic Acoustic Management feature set |
| C3h-CBh | Reserved |
| CCh | Enable reverting to power-on defaults |
| CDh-D5h | Reserved |
| D6h-DCh | Vendor Specific |
| DDh | Disable release interrupt |
| DEh | Disable SERVICE interrupt |
| DFh | Reserved for DRQ technical report |
| E0h | Vendor Specific |
| E1h-EFh | Reserved |
| F0h-FFh | Reserved for assignment by the CompactFlash Association |

#### Enable/disable 8-bit PIO data transfer

Devices implementing the CFA feature set shall support 8-bit PIO data transfers.  Devices not implementing the CFA feature set shall not support 8-bit PIO data transfers.  When 8-bit PIO data transfer is enabled the Data register is 8-bits wide using only DD7 to DD0.

#### Enable/disable write cache

Subcommand codes 02h and 82h allow the host to enable or disable write cache in devices that implement write cache.  When the subcommand disable write cache is issued, the device shall initiate the sequence to flush cache to non-volatile memory before command completion (See 7.14).  This subcommand does not apply to commands that have a Flush to Disk bit.

#### Set transfer mode

A host selects the transfer mechanism by Set Transfer Mode, subcommand code 03h, and specifying a value in the Count field.  The upper 5 bits define the type of transfer and the low order 3 bits encode the mode value.  The host may change the selected modes by the SET FEATURES command.

Table 42 - Transfer mode values42

| Mode | Bits (7:3) | Bits (2:0) |
| --- | --- | --- |
| PIO default mode | 00000b | 000b |
| PIO default mode, disable IORDY | 00000b | 001b |
| PIO flow control transfer mode | 00001b | mode |
| Retired | 00010b | na |
| Multiword DMA mode | 00100b | mode |
| Ultra DMA mode | 01000b | mode |
| Reserved | 10000b | na |
| mode = transfer mode number |  |  |

If a device supports this standard, and receives a SET FEATURES command with a Set Transfer Mode parameter and a Sector Count register value of “00000000b”, the device shall set the default PIO mode.  If the value is “00000001b” and the device supports disabling of IORDY, then the device shall set the default PIO mode and disable IORDY.  A device shall support all PIO modes below the highest mode supported, e.g., if PIO mode 1 is supported PIO mode 0 shall be supported.

Support of IORDY is mandatory when PIO mode 3 or above is the current mode of operation.

A device shall support all Multiword DMA modes below the highest mode supported, e.g., if Multiword DMA mode 1 is supported Multiword DMA mode 0 shall be supported.

A device shall support all Ultra DMA modes below the highest mode supported, e.g., if Ultra DMA mode 1 is supported Ultra DMA mode 0 shall be supported.

If an Ultra DMA mode is enabled any previously enabled Multiword DMA mode shall be disabled by the device.  If a Multiword DMA mode is enabled any previously enabled Ultra DMA mode shall be disabled by the device.

For PATA systems using a cable assembly, the host shall determine that an 80-conductor cable assembly is connecting the host with the device(s) before enabling any Ultra DMA mode greater than 2 in the device(s).

#### Enable/disable advanced power management

Subcommand code 05h allows the host to enable Advanced Power Management.  To enable Advanced Power Management, the host writes the Sector Count register with the desired advanced power management level and then executes a SET FEATURES command with subcommand code 05h.  The power management level is a scale from the lowest power consumption setting of 01h to the maximum performance level of FEh.  Table 43 shows these values.

Table 43 - Advanced power management levels 43

| Level | Count Field |
| --- | --- |
| Maximum performance | FEh |
| Intermediate power management levels without Standby | 81h-FDh |
| Minimum power consumption without Standby | 80h |
| Intermediate power management levels with Standby | 02h-7Fh |
| Minimum power consumption with Standby | 01h |
| Reserved | FFh |
| Reserved | 00h |

Device performance may increase with increasing power management levels.  Device power consumption may increase with increasing power management levels.  The power management levels may contain discrete bands.  For example, a device may implement one power management method from 80h to A0h and a higher performance, higher power consumption method from level A1h to FEh.  Advanced power management levels 80h and higher do not permit the device to spin down to save power.

Subcommand code 85h disables Advanced Power Management.  Subcommand 85h may not be implemented on all devices that implement SET FEATURES subcommand 05h.

#### Enable/disable Power-Up In Standby feature set

Subcommand code 06h enables the Power-Up In Standby feature set.  When this feature set is enabled, the device shall power-up into Standby mode, i.e., the device shall be ready to receive commands but shall not spinup (See 4.12).  Having been enabled, this feature shall remain enabled through power-down, hardware reset and software rest.

Subcommand code 86h disables the Power-Up In Standby feature set.  When this feature set is disabled, the device shall power-up into Active mode.  The factory default for this feature set shall be disabled.

#### Power-Up In Standby feature set device spin-up

Subcommand code 07h shall cause a device that has powered-up into Standby to go to the Active state (See 4.12 and Figure 4).

      - Enable/disable CFA power mode 1

Subcommand code 0Ah enables CFA Power Mode 1.  CFA devices may consume up to 500 mA maximum average RMS current for either 3.3 V or 5 V operation in Power Mode 1.  CFA devices revert to Power Mode 1 on hardware or power-on reset.  CFA devices revert to Power Mode 1 on software reset except when Set Features disable reverting to power-on defaults is set (See 7.17.7.83).  Enabling CFA Power Mode 1 does not spin up rotating media devices.

Subcommand 8Ah disables CFA Power Mode 1, placing the device to CFA Power Mode 0.  CFA devices may consume up to 75 mA maximum average RMS current for 3.3 V or 100 mA maximum average RMS current for 5 V operation in Power Mode 0.

A device in Power Mode 0 the device shall accept the following commands:

- IDENTIFY DEVICE

- SET FEATURES (function codes 0Ah and 8Ah)

- STANDBY

- STANDBY IMMEDIATE

- SLEEP

- CHECK POWER MODE

- EXECUTE DEVICE DIAGNOSTICS

- CFA REQUEST EXTENDED ERROR

A device in Power Mode 0 may accept any command that the device is capable of executing within the Power Mode 0 current restrictions.  Commands that require more current than specified for Power Mode 0 shall be rejected with an abort error.

#### Enable/Disable Write Read Verify feature set

Subcommand code 0Bh enables the Write Read Verify feature set. When this feature set is supported and enabled, the device shall exhibit the following behavior. When this feature set is not supported, the command shall be aborted.

Sector Count register shall specify the maximum number of sectors that shall be verified after any power-on reset or spin-up.  LBA (7:0) contains the mode.  See Table 44.

Table 44 – Write-Read-Verify Sector Counts 44

| Mode | Description |
| --- | --- |
| 00h | Enabled Always. The device shall perform a Write Read Verify for all write command received from the host. |
| 01h | The first 65,536 sectors (located ANYWHERE, not just the LBA=0 thru LBA=65535) written by the host after every spinup (or reset) or after issuing this set features will be verified. |
| 02h | Device may determine, for itself, how many sectors to do (‘automatic’ setting) |
| 03h | The first (Verify Sector Count x 1024) sectors written by the host after every spinup or after issuing this set features will be verified |
| 04h-FFh | Reserved |
| Note: When Mode 3 is not selected, the Verify Sector Count is defined as Ignored |  |

Subcommand code 8Bh disables the Write Read Verify feature set.  When this feature set is not supported, the command shall be aborted. A subsequent IDENTIFY DEVICE or IDENTIFY PACKET DEVICE command shall reflect the disabled state of the feature set.

#### Enable/disable Media Status Notification

Subcommand code 31h disables Media Status Notification and leaves the media in an unlocked state.  If Media Status Notification is disabled when this subcommand is received, the subcommand has no effect.

Subcommand code 95h enables Media Status Notification and clears any previous media lock state.  This subcommand returns the device capabilities for media eject, media lock, previous state of Media Status Notification and the current version of Media Status Notification supported in the LBA field as described below.

| Bits | Description |
| --- | --- |
| 47:19 | Reserved |
| 18 | Power Ejection (PEJ) - shall be set to one if the device has a power eject mechanism that is capable of physically ejecting the media when a MEDIA EJECT command is sent to the device. This bit shall be set to zero if the device only unlocks the media when the device receives a MEDIA EJECT command. |
| 17 | Lock - shall be set to one if the device is capable of locking the media preventing manual ejection. |
| 16 | Previously Enabled (PENA) - shall be set to one if Media Status Notification was enabled prior to the receipt of this command. |
| 15:8 | Version - Shall be set to the Media Status Notification version supported by the device (currently 0x00h). |
| 7:0 | Reserved |

#### Enable/disable Automatic Acoustic Management

Subcommand code 42h allows the host to enable the Automatic Acoustic Management feature set.  To enable the Automatic Acoustic Management feature set, the host writes the Sector Count register with the requested automatic acoustic management level and executes a SET FEATURES command with subcommand code 42h.  The acoustic management level is selected on a scale from 01h to FEh.  Table 45 shows the acoustic management level values.

Enabling or disabling of the Automatic Acoustic Management feature set, and the current automatic acoustic management level setting shall be preserved by the device across all forms of reset, i.e.  power-on, hardware, and software resets.

Table 45 - Automatic acoustic management levels 45

| Level | Count Field |
| --- | --- |
| Reserved | FFh |
| Maximum performance | FEh |
| Intermediate acoustic management levels | 81h-FDh |
| Minimum acoustic emanation level | 80h |
| Retired | 01h-7Fh |
| Vendor Specific | 00h |

Device performance may increase with increasing acoustic management levels.  Device power consumption may decrease with decreasing acoustic management levels.  The acoustic management levels may contain discrete bands.  For example, a device may implement one acoustic management method from 80h to BFh and a higher performance, higher acoustic management method from level C0h to FEh.

Upon successful completion of this SET FEATURES subcommand, IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data word 94, bits (7:0) shall be updated by the device.  If the command is aborted by the device, the previous automatic acoustic management state shall be retained.

Subcommand code C2h disables the Automatic Acoustic Management feature set.  Devices that implement SET FEATURES subcommand 42h are not required to implement subcommand C2h.  If device successfully completes execution of this subcommand, then the acoustic behavior of the device shall be vendor-specific, and the device shall return zeros in bits (7:0) of word 94 and bit 9 of word 86 of the IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data words.

Upon completion of SET FEATURES subcommands 42h and C2h, the device may update words (97:96) and word 104 in IDENTIFY DEVICE or IDENTIFY PACKET DEVICE data words, and the contents of the Stream Performance Parameters Log in the READ LOG EXT command.

#### Set Maximum Host Interface Sector Times

Subcommand code 43h allows the host to inform the device of a host interface rate limitation.  This information shall be used by the device to meet the Command Completion Time Limits of the commands of the streaming feature set.  To inform the device of a host interface rate limitation, the host writes the LSB and MSB value of its Typical PIO Host Interface Sector Time to the Sector Count and LBA Low registers and writes the LSB and MSB value of its Typical DMA Host Interface Sector Time to the LBA Mid and LBA High registers.  The Typical Host Interface Sector Times have the same units as IDENTITY DEVICE data word 96 for DMA and word 104 for PIO.  A value of zero indicates that the host interface shall be capable of transferring data at the maximum rate allowed by the selected transfer mode.  The Typical PIO Mode Host Interface Sector Time includes the host’s interrupt service time.

Upon completion of SET FEATURES subcommand 43h, the device may adjust IDENTIFY DEVICE data words (97:96), and the contents of the Stream Performance Parameters Log in the READ LOG EXT command to allow for the specified host interface sector time.

| Field | Bits | Description |
| --- | --- | --- |
| Count | 15:8 | Reserved |
|  | 7:0 | Typical PIO Mode Host Interface Sector Time (7:0) |
| LBA | 47:24 | Reserved |
|  | 23:8 | Typical DMA Mode Host Interface Sector Time |
|  | 7:0 | Typical PIO Mode Host Interface Sector Time (15:8) |

#### Enable/disable read look-ahead

Subcommand codes AAh and 55h allow the host to request the device to enable or disable read look-ahead.  Error recovery performed by the device is vendor specific.

#### Enable/disable release interrupt

Subcommand codes 5Dh and DDh allow a host to enable or disable the asserting of Interrupt Pending when a device releases the bus for an overlapped PACKET command.

#### Enable/disable SERVICE interrupt

Subcommand codes 5Eh and DEh allow a host to enable or disable the asserting of an Interrupt Pending when DRQ is set to one in response to a SERVICE command.

#### Enable/disable reverting to defaults

Subcommand codes CCh and 66h allow the host to enable or disable the device from reverting to power-on default values.  A setting of 66h allows settings that may have been modified since power-on to remain at the same setting after a software reset.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | Subcommand Code - Table 41 defines the value of the subcommand. |
| 01h | Count | Subcommand specific |
| 02h-04h | LBA | Subcommand specific |
| 05h | Command | EFh |

      - Normal outputs

See the subcommand descriptions.  [Editors Note: Is this statement right?  I see places where the text describes the subcommand specific fields.  I do not see much in the way of normal output returns.]

See Table 65

      - Error outputs

Abort shall be set to one if any subcommand input value is not supported or is invalid.  See Table 79.

### SET MAX

#### SET MAX Overview

Individual SET MAX commands are identified by the value placed in the Features field.  Table 46 shows these Features register values.

Table 46 - SET MAX Features register values 46

| Value | Command |
| --- | --- |
| 00h | SET MAX ADDRESS |
| 01h | SET MAX SET PASSWORD |
| 02h | SET MAX LOCK |
| 03h | SET MAX UNLOCK |
| 04h | SET MAX FREEZE LOCK |
| 05h-FFh | Reserved |

#### SET MAX ADDRESS - F9h

        - Feature Set

This command is mandatory for devices that implement the Host Protected Area feature set.  This command shall not be implemented if the Removable feature set is implemented.

        - Description

After successful command completion, all read and write access attempts to addresses greater than specified by the successful SET MAX ADDRESS command shall be rejected with an ID Not Found error (see clause 6.3.4).  IDENTIFY DEVICE data words (61:60) shall reflect the maximum address set with this command.

If the 48-bit Address feature set is supported, the value placed in IDENTIFY DEVICE data words (103:100) shall be the same as the value placed in words (61:60).

Hosts shall not issue more than one non-volatile SET MAX ADDRESS or SET MAX ADDRESS EXT command after a power-on or hardware reset.  Devices should report an IDNF error upon receiving a second non-volatile SET MAX ADDRESS command after a power-on or hardware reset.

The contents of IDENTIFY DEVICE data words and the max address shall not be changed if a SET MAX ADDRESS command fails.

After a successful SET MAX ADDRESS command using a new maximum LBA the content of all IDENTIFY DEVICE data words shall comply with 4.2.1 and the content of words (61:60) shall be equal to the new Maximum LBA + 1.

A successful READ NATIVE MAX ADDRESS command shall immediately precede a SET MAX ADDRESS command.

        - Inputs

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | N/A |  |  |  |  |
| 01h | Count |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 16:1 | N/A |  |  |
|  |  |  | 0 | Volatile Value (VV) - If VV is set to one, the device shall preserve the maximum values over power-up or hardware reset. If VV is cleared to zero, the device shall revert to the most recent non-volatile maximum address value setting over power-up or hardware reset. |  |  |
| 02h | LBA | MSB |  | SET MAX LBA Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Command | F9h |  |  |  |  |

        - Normal outputs

See Table 74.  LBA bits 47:28 shall be cleared to zero.

        - Error outputs

If this command is not supported, the maximum value to be set exceeds the capacity of the device, a host protected area has been established by a SET MAX ADDRESS EXT command, or the device is in the Set_Max_Locked or Set_Max_Frozen state, then the device shall return command aborted.  ID Not Found shall be set to one if the command was the second non-volatile SET MAX ADDRESS or SET MAX ADDRESS EXT command after power-on or hardware reset.    Abort shall be set to one if the maximum value requested exceeds the device capacity, a host protected area has been estabished by a SET MAX ADDRESS EXT command, the device is in the Set_Max_Locked or Set_Max_Frozen state, or the command is not immediately preceded by a READ NATIVE MAX ADDRESS command.

#### SET MAX SET PASSWORD - F9h/01h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the Host Protected Area feature set.  This command shall not be implemented if the Removable feature set is implemented.

        - Description

This command requests a transfer of a single sector of data from the host.  Table 47 defines the content of this sector of information.  The password is retained by the device until the next power cycle.  When the device accepts this command the device is in Set_Max_Unlocked state.

Table 47 - SET MAX SET PASSWORD data content 47

| Word | Content |
| --- | --- |
| 0 | Reserved |
| 1-16 | Password (32 bytes) |
| 17-255 | Reserved |

This command shall not be immediately preceded by a READ NATIVE MAX ADDRESS command.  If this command is immediately preceded by a READ NATIVE MAX ADDRESS command, it shall be interpreted as a SET MAX ADDRESS command.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | 01h - SET MAX PASSWORD |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F9h |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if the device is in the Set_Max_Locked or Set_Max_Frozen state.  See Table 89. ]

#### SET MAX LOCK - F9h/02h, Non-data

        - Feature Set

This command is mandatory for devices that implement the Host Protected Area feature set.  This command shall not be implemented if the Removable feature set is implemented.

        - Description

The SET MAX LOCK command sets the device into Set_Max_Locked state.  After this command is completed any other SET MAX commands except SET MAX UNLOCK and SET MAX FREEZE LOCK shall be command aborted.  The device shall remain in this state until a power cycle or command completion without error of a SET MAX UNLOCK or SET MAX FREEZE LOCK command.

This command shall not be immediately preceded by a READ NATIVE MAX ADDRESS command.  If this command is immediately preceded by a READ NATIVE MAX ADDRESS command, it shall be interpreted as a SET MAX ADDRESS command.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | 02h - SET MAX LOCK |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F9h |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if the device is in the Set_Max_Locked or Set_Max_Frozen state.  See Table 79

#### SET MAX UNLOCK - F9h/03h, PIO data-out

        - Feature Set

This command is mandatory for devices that implement the Host Protected Area feature set.  This command shall not be implemented if the Removable feature set is implemented.

        - Description

This command requests a transfer of a single sector of data from the host.  Table 47 defines the content of this sector of information.

The password supplied in the sector of data transferred shall be compared with the stored SET MAX password.

If the password compare fails, then the device shall return command aborted and decrement the unlock counter.  On the acceptance of the SET MAX LOCK command, this counter is set to a value of five and shall be decremented for each password mismatch when SET MAX UNLOCK is issued and the device is locked.  When this counter reaches zero, then the SET MAX UNLOCK command shall return command aborted until a power cycle.

If the password compare matches, then the device shall make a transition to the Set_Max_Unlocked state and all SET MAX commands shall be accepted.

This command shall not be immediately preceded by a READ NATIVE MAX ADDRESS command.  If this command is immediately preceded by a READ NATIVE MAX ADDRESS command, it shall be interpreted as a SET MAX ADDRESS command.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | 03h - SET MAX UNLOCK |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F9h |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if the device is not in the Set_Max_Locked state.  See Table 79

SET MAX FREEZE LOCK - F9h/04h, Non-data

        - Feature Set

This command is mandatory for devices that implement the Host Protected Area feature set.  This command shall not be implemented if the Removable feature set is implemented.

        - Description

The SET MAX FREEZE LOCK command sets the device to Set_Max_Frozen state.  After command completion any subsequent SET MAX commands shall be command aborted.

Commands disabled by SET MAX FREEZE LOCK are:

- SET MAX ADDRESS

- SET MAX SET PASSWORD

- SET MAX LOCK

- SET MAX UNLOCK

This command shall not be immediately preceded by a READ NATIVE MAX ADDRESS command.  If this command is immediately preceded by a READ NATIVE MAX ADDRESS command, it shall be interpreted as a SET MAX ADDRESS command.

        - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | 04h - SET MAX FREEZE LOCK |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | F9h |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if the device is not in the Set_Max_Unlocked state..  See Table 79.]

### SET MAX ADDRESS EXT - 37h, Non-data

      - Feature Set

This command is mandatory for devices that implement both the Host Protected Area feature set and 48-bit Address feature set.  This command shall not be implemented if the Removable feature set is implemented.

      - Description

After successful command completion, all read and write access attempts to addresses greater than specified by the successful SET MAX ADDRESS EXT command shall be rejected with an IDNF error.

Hosts shall not issue more than one non-volatile SET MAX ADDRESS or SET MAX ADDRESS EXT command after a power-on or hardware reset.  Devices shall report an IDNF error upon receiving a second non-volatile SET MAX ADDRESS EXT command after a power-on or hardware reset.

The contents of IDENTIFY DEVICE data words and the max address shall not be changed if a SET MAX ADDRESS EXT command fails.

After a successful SET MAX ADDRESS EXT command using a new maximum LBA the content of all IDENTIFY DEVICE data words shall comply with 6.2.1.

A successful READ NATIVE MAX ADDRESS EXT command shall immediately precede a SET MAX ADDRESS EXT command.

      - Inputs

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |  |  |
| 01h | Count |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 16:1 | Reserved |  |  |
|  |  |  | 0 | Volatile Value (VV) - If VV is set to one, the device shall preserve the maximum values over power-up or hardware reset. If VV is cleared to zero, the device shall revert to the most recent non-volatile maximum address value setting over power-up or hardware reset. |  |  |
| 02h | LBA | MSB |  | SET MAX LBA |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Command | 37h |  |  |  |  |

      - Normal outputs

See Table 74

      - Error outputs

If this command is not supported, the maximum value to be set exceeds the capacity of the device, a host protected area has been established by a SET MAX ADDRESS command, the command is not immediately preceded by a READ NATIVE MAX ADDRESS EXT command, or the device is in the Set_Max_Locked or Set_Max_Frozen state, then the device shall return command aborted.  ID Not Found shall be set to one if the command was the second non-volatile SET MAX ADDRESS or SET MAX ADDRESS EXT command after power-on or hardware reset.  Abort shall be set to one if the maximum value requested exceeds the device capacity, a host protected area has been established by a SET MAX ADDRESS command, or the command is not immediately preceded by a READ NATIVE MAX ADDRESS EXT command.  See Table 89.

### SET MULTIPLE MODE - C6h, Non-data

      - Feature Set

This command is mandatory for devices implementing the General feature set

      - Description

This command establishes the block count for READ MULTIPLE, READ MULTIPLE EXT, WRITE MULTIPLE, and WRITE MULTIPLE EXT commands.

Devices shall support the block size specified in the IDENTIFY DEVICE parameter word 47, bits (7:0), and may also support smaller values.

Upon receipt of the command, the device checks the Sector Count register.  If the content of the Sector Count register is not zero, the Sector Count register contains a valid value, and the block count is supported, then the value in the Sector Count register is used for all subsequent READ MULTIPLE, READ MULTIPLE EXT, WRITE MULTIPLE, and WRITE MULTIPLE EXT commands and their execution is enabled.  If the content of the Sector Count register is zero, the device may:

- disable multiple mode and respond with command aborted to all subsequent READ MULTIPLE, READ MULTIPLE EXT, WRITE MULTIPLE, and WRITE MULTIPLE EXT commands;

- respond with command aborted to the SET MULTIPLE MODE command;

- retain the previous multiple mode settings.

After a successful SET MULTIPLE command the device shall report the valid value set by that command in bits (7:0) in word 59 in the IDENTIFY DEVICE data.

After a power-on or hardware reset, if bit 8 is set to one and bits (7:0) are cleared to zero in word 59 of the IDENTIFY DEVICE data, a SET MULTIPLE command is required before issuing a READ MULTIPLE, READ MULTIPLE EXT, WRITE MULTIPLE, or WRITE MULTIPLE EXT command.  If bit 8 is set to one and bits (7:0) are not cleared to zero, a SET MULTIPLE command may be issue to change the multiple value required before issuing a READ MULTIPLE, READ MULTIPLE EXT, WRITE MULTIPLE, or WRITE MULTIPLE EXT command.

      - Inputs

[Editors Note: Shouldn’t this be part of the description?]

If the content of the Count field is not zero, then the Sector Count register contains the number of sectors per block for the device to be used on all following READ/WRITE MULTIPLE commands.  The content of the Count field shall be less than or equal to the value in bits (7:0) in word 47 in the IDENTIFY DEVICE data.  The host should set the content of the Sector Count register to 1, 2, 4, 8, 16, 32, 64 or 128.

If the content of the Sector Count register is zero and the SET MULTIPLE command completes without error, then the device shall respond to any subsequent READ MULTIPLE or WRITE MULTIPLE command with command aborted until a subsequent successful SET MULTIPLE command completion where the Sector Count register is not set to zero.

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | Sectors per block |
| 02h-04h | LBA | N/A |
| 05h | Command | C6h |

      - Normal outputs

See Table 65

      - Error outputs

Abort shall be set to one if the block count is not supported.  See Table 79.]

### SLEEP - E6h, Non-data

      - Feature Set

This command is mandatory for devices implementing the Power Management feature set.  The Power Management feature set is mandatory for devices not implementing the PACKET Command set  This command is mandatory when power management is not implemented by the PACKET Command set implemented by the device.

      - Description

This command is the only way to cause the device to enter Sleep mode.

This command shall cause the device to set the BSY bit to one, prepare to enter Sleep mode, clear the BSY bit to zero and assert INTRQ.  The host shall read the Status register in order to clear the Interrupt Pending and allow the device to enter Sleep mode.  In Sleep mode, the device shall only respond to the assertion of the RESET- signal and the writing of the SRST bit in the Device Control register and shall release the device driven signal lines (See Figure 4).  The host shall not attempt to access the Command Block registers while the device is in Sleep mode.

Because some host systems may not read the Status register and clear the Interrupt Pending, a device may release INTRQ and enter Sleep mode after a vendor specific time period of not less than 2 s.

The only way to recover from Sleep mode is with a software reset, a hardware reset, or a DEVICE RESET command.

A device shall not power-on in Sleep mode nor remain in Sleep mode following a reset sequence.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | E6h |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79]

### SMART

#### Overview

Individual SMART commands are identified by the value placed in the Feature register.  Table 48 shows these Feature register values.

Table 48 - SMART Feature register values 48

| Value | Command |
| --- | --- |
| 00h-CFh | Reserved |
| D0h | SMART READ DATA |
| D1h | Obsolete |
| D2h | SMART ENABLE/DISABLE ATTRIBUTE AUTOSAVE |
| D3h | Obsolete |
| D4h | SMART EXECUTE OFF-LINE IMMEDIATE |
| D5h | SMART READ LOG |
| D6h | SMART WRITE LOG |
| D7h | Obsolete |
| D8h | SMART ENABLE OPERATIONS |
| D9h | SMART DISABLE OPERATIONS |
| DAh | SMART RETURN STATUS |
| DBh | Obsolete |
| DCh-DFh | Reserved |
| E0h-FFh | vendor specific |

#### SMART DISABLE OPERATIONS - B0h/D9h, Non-data

        - Feature Set

This command is mandatory for devices that implement the SMART feature set.

        - Description

This command disables all SMART capabilities within the device including any and all timer and event count functions related exclusively to this feature.  After command acceptance the device shall disable all SMART operations.  SMART data shall no longer be monitored or saved by the device.  The state of SMART, either enabled or disabled, shall be preserved by the device across power cycles.

After receipt of this command by the device, all other SMART commands including SMART DISABLE OPERATIONS commands, with the exception of SMART ENABLE OPERATIONS, are disabled and invalid and shall be command aborted by the device.

        - Inputs

The Features register shall be set to D9h.  The LBA Mid register shall be set to 4Fh.  The LBA High register shall be set to C2h.

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D9h - SMART DISABLE OPERATIONS |  |  |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | N/A |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if SMART is not enabled, or if an input value is invalid.  See Table 79]

#### SMART ENABLE/DISABLE ATTRIBUTE AUTOSAVE - B0h/D2h, Non-data

        - Feature Set

This command is mandatory for devices that implement the SMART feature set.

        - Description

This command enables and disables the optional attribute autosave feature of the device.  This command may either allow the device, after some vendor specified event, to save the device updated attribute values to non-volatile memory; or this command may cause the autosave feature to be disabled.  The state of the attribute autosave feature (either enabled or disabled) shall be preserved by the device across power cycles.

A value of zero written by the host into the device’s Sector Count register before issuing this command shall cause this feature to be disabled.  Disabling this feature does not preclude the device from saving SMART data to non-volatile memory during some other normal operation such as during a power-on or power-off sequence or during an error recovery sequence.

A value of F1h written by the host into the device’s Sector Count register before issuing this command shall cause this feature to be enabled.  Any other meaning of this value or any other non-zero value written by the host into this register before issuing this command may differ from device to device.  The meaning of any non-zero value written to this register at this time shall be preserved by the device across power cycles.

If this command is not supported by the device, the device shall return command aborted upon receipt from the host.

During execution of the autosave routine the device shall not set BSY to one nor clear DRDY to zero.  If the device receives a command from the host while executing the autosave routine the device shall begin processing the command within two seconds.

        - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D2h - SMART ENABLE/DISABLE ATTRIBUTE AUTOSAVE |  |  |  |
| 01h | Count |  |  |  |  |
|  |  |  | Value | Description |  |
|  |  |  | 00h | Disable attribute autosave |  |
|  |  |  | 01h-F0h | Reserved |  |
|  |  |  | F1h | Enable attribute autosave |  |
|  |  |  | F2h-FFh | Reserved |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | N/A |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 65

        - Error outputs

Abort shall be set to one if SMART is not enabled, or if an input value is invalid.  See Table 79.]

#### SMART ENABLE OPERATIONS - B0h/D8h, Non-data

        - Feature Set

This command is mandatory for devices that implement the SMART feature set.

        - Description

This command enables access to all SMART capabilities within the device.  Prior to receipt of this command SMART data are neither monitored nor saved by the device.  The state of SMART (either enabled or disabled) shall be preserved by the device across power cycles.  Once enabled, the receipt of subsequent SMART ENABLE OPERATIONS commands shall not affect any SMART data or functions.

        - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D8h - SMART ENABLE OPERATIONS |  |  |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | N/A |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 65

        - Error outputs

See Table 79. ]

#### SMART EXECUTE OFF-LINE IMMEDIATE - B0h/D4h, Non-data

        - Feature Set

This command is optional for devices that implement the SMART feature set.

        - Description

This command causes the device to immediately initiate the optional set of activities that collect SMART data in an off-line mode and then save this data to the device's non-volatile memory, or execute a self-diagnostic test routine in either captive or off-line mode.

Table 49 - SMART EXECUTE OFF-LINE IMMEDIATE Subcommands49

| Value | Description of subcommand to be executed |
| --- | --- |
| 0 | Execute SMART off-line routine immediately in off-line mode |
| 1 | Execute SMART Short self-test routine immediately in off-line mode |
| 2 | Execute SMART Extended self-test routine immediately in off-line mode |
| 3 | Execute SMART Conveyance self-test routine immediately in off-line mode |
| 4 | Execute SMART Selective self-test routine immediately in off-line mode |
| 5-63 | Reserved |
| 64-126 | Vendor specific |
| 127 | Abort off-line mode self-test routine |
| 128 | Reserved |
| 129 | Execute SMART Short self-test routine immediately in captive mode |
| 130 | Execute SMART Extended self-test routine immediately in captive mode |
| 131 | Execute SMART Conveyance self-test routine immediately in captive mode |
| 132 | Execute SMART Selective self-test routine immediately in captive mode |
| 133-191 | Reserved |
| 192-255 | Vendor specific |

        - Off-line mode

The following describes the protocol for executing a SMART EXECUTE OFF-LINE IMMEDIATE subcommand routine (including a self-test routine) in the off-line mode.

- The device shall execute command completion before executing the subcommand routine.

- After clearing BSY to zero and setting DRDY to one after receiving the command, the device shall not set BSY nor clear DRDY during execution of the subcommand routine.

- If the device is in the process of performing the subcommand routine and is interrupted by any new command from the host except a SLEEP, SMART DISABLE OPERATIONS, SMART EXECUTE OFF-LINE IMMEDIATE, or STANDBY IMMEDIATE command, the device shall suspend or abort the subcommand routine and service the host within two seconds after receipt of the new command.  After servicing the interrupting command from the host the device may immediately re-initiate or resume the subcommand routine without any additional commands from the host (See 7.54.6.6).

- If the device is in the process of performing a subcommand routine and is interrupted by a SLEEP command from the host, the device may abort the subcommand routine and execute the SLEEP command.  If the device is in the process of performing any self-test routine and is interrupted by a SLEEP command from the host, the device shall abort the subcommand routine and execute the SLEEP command.

- If the device is in the process of performing the subcommand routine and is interrupted by a SMART DISABLE OPERATIONS command from the host, the device shall suspend or abort the subcommand routine and service the host within two seconds after receipt of the command.  Upon receipt of the next SMART ENABLE OPERATIONS command the device may, either re-initiate the subcommand routine or resume the subcommand routine from where it had been previously suspended.

- If the device is in the process of performing the subcommand routine and is interrupted by a SMART EXECUTE OFF-LINE IMMEDIATE command from the host, the device shall abort the subcommand routine and service the host within two seconds after receipt of the command.  The device shall then service the new SMART EXECUTE OFF-LINE IMMEDIATE subcommand.

- If the device is in the process of performing the subcommand routine and is interrupted by a STANDBY IMMEDIATE or IDLE IMMEDIATE command from the host, the device shall suspend or abort the subcommand routine, and service the host within two seconds after receipt of the command.  After receiving a new command that causes the device to exit a power saving mode, the device shall initiate or resume the subcommand routine without any additional commands from the host unless these activities were aborted by the host (See 7.54.6.1).

- While the device is performing the subcommand routine it shall not automatically change power states (e.g., as a result of its Standby timer expiring).

- If a test failure occurs while a device is performing a self-test routine the device may discontinue the testing and place the test results in the Self-test execution status byte (See Table 50).

        - Captive mode

When executing a self-test in captive mode, the device sets BSY to one and executes the self-test routine after receipt of the command.  At the end of the routine the device places the results of this routine in the Self-test execution status byte (See Table 50) and executes command completion.  If an error occurs while a device is performing the routine the device may discontinue its testing, place the results of this routine in the Self-test execution status byte, and complete the command.

        - SMART off-line routine

This routine shall only be performed in the off-line mode.  The results of this routine are placed in the Off-line data collection status byte (See Table 51).

        - SMART Short self-test routine

Depending on the value in the LBA Low register, this self-test routine may be performed in either the captive or the off-line mode.  This self-test routine should take on the order of ones of minutes to complete (See 7.54.6.1).

        - SMART Extended self-test routine

Depending on the value in the LBA Low register, this self-test routine may be performed in either the captive or the off-line mode.  This self-test routine should take on the order of tens of minutes to complete (See 7.54.6.1).

        - SMART Conveyance self-test routine

Depending on the value in the LBA Low register, this self-test routine may be performed in either the captive or the off-line mode.  This self-test routine is intended to identify damage incurred during transporting of the device.  This self-test routine should take on the order of minutes to complete (See 7.54.6.1).

        - SMART Selective self-test routine

The SMART Selective self-test routine is an optional self-test routine.  If the routine is implemented, all features of the routine shall be implemented.  Support for the routine is indicated in off-line data collection capabilities (See 7.54.6.6).

When the value in the LBA Low register is 4 or 132, the Selective self-test routine shall be performed.  This self-test routine shall include the initial tests performed by the Extended self-test routine plus a selectable read scan.  The host shall not write the Selective self-test log while the execution of a Selective self-test command is in progress.

The user may choose to do read scan only on specific areas of the media.  To do this, user shall set the test spans desired in the Selective self-test log and set the flags in the Feature flags field of the Selective self-test log to indicate do not perform off-line scan.  In this case, the test spans defined shall be read scanned in their entirety.  The Selective self-test log is updated as the self-test proceeds indicating test progress.  When all specified test spans have been completed, the test is terminated and the appropriate self-test execution status is reported in the SMART READ DATA response depending on the occurrence of errors.  Figure 9 shows an example of a Selective self-test definition with three test spans defined.  In this example, the test terminates when all three test spans have been scanned.

Figure 9 - Selective self-test test span example 9

After the scan of the selected spans described above, a user may wish to have the rest of media read scanned as an off-line scan.  In this case, the user shall set the flag to enable off-line scan in addition to the other settings.  If an error occurs during the scanning of the test spans, the error is reported in the self-test execution status in the SMART READ DATA response and the off-line scan is not executed.  When the test spans defined have been scanned, the device shall then set the off-line scan pending and active flags in the Selective self-test log to one, the span under test to a value greater than five, the self-test execution status in the SMART READ DATA response to 00h, set a value of 03h in the off-line data collection status in the SMART READ DATA response and shall proceed to do an off-line read scan through all areas not included in the test spans.  This off-line read scan shall completed as rapidly as possible, no pauses between block reads, and any errors encountered shall not be reported to the host.  Instead error locations may be logged for future reallocation.  If the device is powered-down before the off-line scan is completed, the off-line scan shall resume when the device is again powered up.  From power-up, the resumption of the scan shall be delayed the time indicated in the Selective self-test pending time field in the Selective self-test log.  During this delay time the pending flag shall be set to one and the active flag shall be set to zero in the Selective self-test log.  Once the time expires,  the active flag shall be set to one, and the off-line scan shall resume.  When the entire media has been scanned, the off-line scan shall terminate, both the pending and active flags shall be cleared to zero, and the off-line data collection status in the SMART READ DATA response shall be set to 02h indicating completion.

During execution of the Selective self-test, the self-test executions time byte in the Device SMART Data Structure may be updated but the accuracy may not be exact because of the nature of the test span segments.  For this reason, the time to complete off-line testing and the self-test polling times are not valid.  Progress through the test spans is indicated in the selective self-test log.

A hardware or software reset shall abort the Selective self-test except when the pending bit is set to one in the Selective self-test log (See 7.54.7.2.6).  The receipt of a SMART EXECUTE OFF-LINE IMMEDIATE command with 0Fh, Abort off-line test routine, in the LBA Low register shall abort Selective self-test regardless of where the device is in the execution of the command.  If a second self-test is issued while a selective self-test is in progress, the selective self-test is aborted and the newly requested self-test is executed.

        - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D4h - SMART EXECUTE OFF-LINE IMMEDIATE |  |  |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | Table 49 defines the subcommand that shall be executed |  |
| 05h | Command | B0h |  |  |  |

        - Normal Outputs

See Table 75.

        - Error Outputs

ID Not Found shall be set to one if SMART data sector’s ID field could not be found.  Abort shall be set to one if SMART is not enabled or if a self-test fails while executing a sequence in captive mode.  See Table 90.

#### SMART READ DATA - B0h/D0h, PIO data-in

        - Feature Set

This command is optional for devices that implement the SMART feature set.

        - Description

This command returns the Device SMART data structure to the host.

Table 50 defines the 512 bytes that make up the Device SMART data structure.  All multi-byte fields shown in this structure follow the byte ordering described in Volume 1, Clause 3.

Table 50 - Device SMART data structure50

| Byte | F/V | Descriptions |  |
| --- | --- | --- | --- |
| 0-361 | X | Vendor specific |  |
| 362 | V | Off-line data collection status |  |
| 363 | X | Self-test execution status byte |  |
| 364-365 | V | Total time in seconds to complete off-line data collection activity |  |
| 366 | X | Vendor specific |  |
| 367 | F | Off-line data collection capability |  |
| 368-369 | F | SMART capability |  |
| 370 | F | Error logging capability |  |
|  |  | 7-1 | Reserved |
|  |  | 0 | 1=Device error logging supported |
| 371 | X | Vendor specific |  |
| 372 | F | Short self-test routine recommended polling time (in minutes) |  |
| 373 | F | Extended self-test routine recommended polling time (7:0) in minutes. If FFh, use bytes 375 and 376 for the polling time. |  |
| 374 | F | Conveyance self-test routine recommended polling time (in minutes) |  |
| 375 | F | Extended self-test routine recommended polling time (7:0) in minutes |  |
| 376 | F | Extended self-test routine recommended polling time (15:8) in minutes |  |
| 377-385 | R | Reserved |  |
| 386-510 | X | Vendor specific |  |
| 511 | V | Data structure checksum |  |
| Key: F=the content of the byte is fixed and does not change. V=the content of the byte is variable and may change depending on the state of the device or the commands executed by the device. X=the content of the byte is vendor specific and may be fixed or variable. R=the content of the byte is reserved and shall be zero. |  |  |  |

        - Off-line collection status byte

The value of the off-line data collection status byte defines the current status of the off-line activities of the device.  Table 51 lists the values and their respective definitions.

Table 51 - Off-line data collection status byte values51

| Value | Definition |
| --- | --- |
| 00h or 80h | Off-line data collection activity was never started. |
| 01h | Reserved |
| 02h or 82h | Off-line data collection activity was completed without error. |
| 03h | Off-line activity in progress. |
| 04h or 84h | Off-line data collection activity was suspended by an interrupting command from host. |
| 05h or 85h | Off-line data collection activity was aborted by an interrupting command from host. |
| 06h or 86h | Off-line data collection activity was aborted by the device with a fatal error. |
| 07h-3Fh | Reserved |
| 40h-7Fh | Vendor specific |
| 81h | Reserved |
| 83h | Reserved |
| 87h-BFh | Reserved |
| C0h-FFh | Vendor specific |

        - Self-test execution status byte

The self-test execution status byte reports the execution status of the self-test routine.

- Bits (3:0) (Percent Self-Test Remaining) The value in these bits indicates an approximation of the percent of the self-test routine remaining until completion in ten percent increments.  Valid values are 9 through 0.  A value of 0 indicates the self-test routine is complete.   A value of 9 indicates 90% of total test time remaining.

- Bits (7:4) (Self-test Execution Status) The value in these bits indicates the current Self-test Execution Status (See Table 52).

Table 52 - Self-test execution status values52

| Value | Description |
| --- | --- |
| 0 | The previous self-test routine completed without error or no self-test has ever been run |
| 1 | The self-test routine was aborted by the host |
| 2 | The self-test routine was interrupted by the host with a hardware or software reset |
| 3 | A fatal error or unknown test error occurred while the device was executing its self-test routineand the device was unable to complete the self-test routine. |
| 4 | The previous self-test completed having a test element that failed and the test element that failed is not known. |
| 5 | The previous self-test completed having the electrical element of the test failed. |
| 6 | The previous self-test completed having the servo (and/or seek) test element of the test failed. |
| 7 | The previous self-test completed having the read element of the test failed. |
| 8 | The previous self-test completed having a test element that failed and the device is suspected of having handling damage. |
| 9-14 | Reserved. |
| 15 | Self-test routine in progress. |

        - Total time to complete off-line data collection

The total time in seconds to complete off-line data collection activity word specifies how many seconds the device requires to complete the sequence of off-line data collection activity.  Valid values for this word are from 0001h to FFFFh.

        - Off-line data collection capabilities

The following describes the definition for the off-line data collection capability bits.  If the value of all of these bits is cleared to zero, then no off-line data collection is implemented by this device.  [Editors Note: would this be better as a table? It would be clearerm yes]

- Bit 0 (EXECUTE OFF-LINE IMMEDIATE implemented bit) - If this bit is set to one, then the SMART EXECUTE OFF-LINE IMMEDIATE command is implemented by this device.  If this bit is cleared to zero, then the SMART EXECUTE OFF-LINE IMMEDIATE command is not implemented by this device.

- Bit 1 (vendor specific).

- Bit 2 (abort/restart off-line by host bit) - If this bit is set to one, then the device shall abort all off-line data collection activity initiated by an SMART EXECUTE OFF-LINE IMMEDIATE command upon receipt of a new command within 2 seconds of receiving the new command.  If this bit is cleared to zero, the device shall suspend off-line data collection activity after an interrupting command and resume off-line data collection activity after some vendor-specified event.

- Bit 3 (off-line read scanning implemented bit) - If this bit is cleared to zero, the device does not support off-line read scanning.  If this bit is set to one, the device supports off-line read scanning.

- Bit 4 (self-test implemented bit) - If this bit is cleared to zero, the device does not implement the Short and Extended self-test routines.  If this bit is set to one, the device implements the Short and Extended self-test routines.

- Bit 5 (conveyance self-test implemented bit) - If this bit is cleared to zero, the device does not implement the Conveyance self-test routines.  If this bit is set to one, the device implements the Conveyance self-test routines.

- Bit 6 (Selective self-test implemented bit) - If this bit is cleared to zero, the device does not implement the Selective self-test routine.  If this bit is set to one, the device implements the Selective self-test routine.

- Bit 7 (Reserved).

        - SMART capablilities

The following describes the definition for the SMART capabilities bits.

- Bit 0  - If this bit is set to one, the device saves SMART data prior to going into a power saving mode (Idle, Standby, or Sleep) or immediately upon return to Active or Idle mode from a Standby mode.  If this bit is cleared to zero, the device does not save SMART data prior to going into a power saving mode (Idle, Standby, or Sleep) or immediately upon return to Active or Idle mode from a Standby mode.

- Bit 1 - This bit shall be set to one to indicate that the device supports the SMART ENABLE/DISABLE ATTRIBUTE AUTOSAVE command.

- Bits (15:2) (Reserved).

        - Self-test routine recommended polling time

The self-test routine recommended polling time shall be equal to the estimated number of minutes that is the minimum recommended time before which the host should first poll check for test completion status.  Actual test time could be several times this value. The host should wait at least this long before sending the first SMART READ DATA command to check for test completion status. Polling Checking before this time could extend the self-test execution time or abort the test depending on the state of bit 2 of the offline data capability bits. Subsequent checking by the host shall be at a vendor specific interval.

        - Data structure checksum

The data structure checksum is the two's complement of the sum of the first 511 bytes in the data structure.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all 512 bytes will be zero when the checksum is correct.  The checksum is placed in byte 511.

        - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D0h - SMART READ DATA |  |  |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | N/A |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 65.

        - Error outputs

Uncorrectable Error shall be set to one if SMART data is uncorrectable.  ID Note Found shall be set to one if SMART data sector’s ID field could not be found or a data structure checksum occurred.  Abort shall be set to one if SMART is not enabled, or if register values are invalid.  See Table 86.

#### SMART READ LOG - B0h/D5h

        - Feature Set

This command is optional for devices that implement the SMART featureset.

        - Description

[Editors Note: I believe that there is quite a bit of overlap with the READ LOG EXT description.  There is also many references to registers that need to be fixed.]

###### Overview

This command returns the specified log to the host.

###### SMART Log Directory

Table 53 defines the 512 bytes that make up the SMART Log Directory, which is optional.  If implemented, the SMART Log Directory is SMART Log address zero, and is defined as one sector long.

Table 53 - SMART Log Directory 53

| Byte | Descriptions |
| --- | --- |
| 0-1 | SMART Logging Version |
| 2 | Number of sectors in the log at log address 1 |
| 3 | Reserved |
| 4 | Number of sectors in the log at log address 2 |
| 5 | Reserved |
| … | … |
| 510 | Number of sectors in the log at log address 255 |
| 511 | Reserved |

The value of the SMART Logging Version word shall be 01h if the drive supports multi-sector SMART logs.  In addition, if the drive supports multi-sector logs, then the logs at log addresses 80-9Fh (Host Vendor Specific addresses) shall each be defined as 16 sectors long.  The content of the Host Vendor Specific log addresses shall be common to both Smart Log Commands and General Purpose Log Commands.  This means that if the host places data in a Host Vendor Specific page using SMART WRITE LOG, and then issues a READ LOG EXT to the same page, that the host receives the same data that was originally stored by SMART WRITE LOG.  [Editor’s Note: should this be in its own section.  It is kind of buried here.  Do it]

If the drive does not support multi-sector SMART logs, then log number zero is defined as reserved, and the drive shall return a command aborted response to the host’s request to read log number zero.

###### Summary error log sector

Table 54 defines the 512 bytes that make up the SMART summary error log sector.  All multi-byte fields shown in this structure follow the byte ordering described in Volume 1, Clause 3.  Summary error log data structures shall include UNC errors, IDNF errors for which the address requested was valid, servo errors, write fault errors, etc.  Summary error log data structures shall not include errors attributed to the receipt of faulty commands such as command codes not implemented by the device or requests with invalid parameters or invalid addresses.  If the device supports comprehensive error log (address 02h), then the summary error log sector duplicates the last five error entries in the comprehensive error log.  The summary error log supports 28-bit addressing only.

Table 54 - SMART summary error log sector54

| Byte | Descriptions |
| --- | --- |
| 0 | SMART error log version |
| 1 | Error log index |
| 2-91 | First error log data structure |
| 92-181 | Second error log data structure |
| 182-271 | Third error log data structure |
| 272-361 | Fourth error log data structure |
| 362-451 | Fifth error log data structure |
| 452-453 | Device error count |
| 454-510 | Reserved |
| 511 | Data structure checksum |

            - Error log version

The value of the SMART summary error log version byte shall be 01h.

            - Error log index

The error log index indicates the error log data structure representing the most recent error.  Only values 5 through 0 are valid.  If there are no error log entries, the value of the error log index shall be zero.

            - Error log data structure

An error log data structure shall be presented for each of the last five errors reported by the device.  These error log data structure entries are viewed as a circular buffer.  That is, the first error shall create the first error log data structure; the second error, the second error log structure; etc.  The sixth error shall create an error log data structure that replaces the first error log data structure; the seventh error replaces the second error log structure, etc.  The error log pointer indicates the most recent error log structure.  If fewer than five errors have occurred, the unused error log structure entries shall be zero filled.  Table 55 describes the content of a valid error log data structure.

Table 55 - Error log data structure55

| Byte | Descriptions |
| --- | --- |
| n thru n+11 | First command data structure |
| n+12 thru n+23 | Second command data structure |
| n+24 thru n+35 | Third command data structure |
| n+36 thru n+47 | Fourth command data structure |
| n+48 thru n+59 | Fifth command data structure |
| n+60 thru n+89 | Error data structure |

              - Command data structure

The fifth command data structure shall contain the command or reset for which the error is being reported.  The fourth command data structure should contain the command or reset that preceded the command or reset for which the error is being reported, the third command data structure should contain the command or reset preceding the one in the fourth command data structure, etc.  If fewer than four commands and resets preceded the command or reset for which the error is being reported, the unused command data structures shall be zero filled, for example, if only three commands and resets preceded the command or reset for which the error is being reported, the first command data structure shall be zero filled.  In some devices, the hardware implementation may preclude the device from reporting the commands that preceded the command for which the error is being reported or that preceded a reset.  In this case, the command data structures are zero filled.

If the command data structure represents a command or software reset, the content of the command data structure shall be as shown in Table 56.  If the command data structure represents a hardware reset, the content of byte n shall be FFh, the content of bytes n+1 through n+7 are vendor specific, and the content of bytes n+8 through n+11 shall contain the timestamp.

Table 56 - Command data structure56

| Byte | Descriptions |
| --- | --- |
| n | Content of the Device Control register when the Command register was written. |
| n+1 | Content of the Features register when the Command register was written. |
| n+2 | Content of the Sector Count register when the Command register was written. |
| n+3 | Content of the LBA Low register when the Command register was written. |
| n+4 | Content of the LBA Mid register when the Command register was written. |
| n+5 | Content of the LBA High register when the Command register was written. |
| n+6 | Content of the Device register when the Command register was written. |
| n+7 | Content written to the Command register. |
| n+8 | Timestamp (least significant byte) |
| n+9 | Timestamp (next least significant byte) |
| n+10 | Timestamp (next most significant byte) |
| n+11 | Timestamp (most significant byte) |

Timestamp shall be the time since power-on in milliseconds when command acceptance occurred.  This timestamp may wrap around.

              - Error data structure

The error data structure shall contain the error description of the command for which an error was reported as described in Table 57.  If the error was logged for a hardware reset, the content of bytes n+1 through n+7 shall be vendor specific and the remaining bytes shall be as defined in Table 57.

Table 57 - Error data structure57

| Byte | Descriptions |
| --- | --- |
| n | Reserved |
| n+1 | Content of the Error register after command completion occurred. |
| n+2 | Content of the Sector Count register after command completion occurred. |
| n+3 | Content of the LBA Low register after command completion occurred. |
| n+4 | Content of the LBA Mid register after command completion occurred. |
| n+5 | Content of the LBA High register after command completion occurred. |
| n+6 | Content of the Device register after command completion occurred. |
| n+7 | Content written to the Status register after command completion occurred. |
| n+8 thru n+26 | Extended error information |
| n+27 | State |
| n+28 | Life timestamp (least significant byte) |
| n+29 | Life timestamp (most significant byte) |

Extended error information shall be vendor specific.

State shall contain a value indicating the state of the device when command was written to the Command register or the reset occurred as described in Table 58.

Table 58 - State field values58

| Value | State |
| --- | --- |
| x0h | Unknown |
| x1h | Sleep |
| x2h | Standby |
| x3h | Active/Idle with BSY cleared to zero |
| x4h | Executing SMART off-line or self-test |
| x5h-xAh | Reserved |
| xBh-xFh | Vendor unique |
| The value of x is vendor specific and may be different for each state. |  |

Sleep indicates the reset for which the error is being reported was received when the device was in the Sleep mode.

Standby indicates the command or reset for which the error is being reported was received when the device was in the Standby mode.

Active/Idle with BSY cleared to zero indicates the command or reset for which the error is being reported was received when the device was in the Active or Idle mode and BSY was cleared to zero.

Executing SMART off-line or self-test indicates the command or reset for which the error is being reported was received when the device was in the process of executing a SMART off-line or self-test.

Life timestamp shall contain the power-on lifetime of the device in hours when command completion occurred.

            - Device error count

The device error count field shall contain the total number of errors attributable to the device that have been reported by the device during the life of the device.  These errors shall include UNC errors, IDNF errors for which the address requested was valid, servo errors, write fault errors, etc.  This count shall not include errors attributed to the receipt of faulty commands such as commands codes not implemented by the device or requests with invalid parameters or invalid addresses.  If the maximum value for this field is reached, the count shall remain at the maximum value when additional errors are encountered and logged.

            - Data structure checksum

The data structure checksum is the two's complement of the sum of the first 511 bytes in the data structure.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all 512 bytes will be zero when the checksum is correct.  The checksum is placed in byte 511.

###### Comprehensive error log

Table 59 defines the format of each of the sectors that comprise the SMART comprehensive error log.  The SMART Comprehensive error log provides logging for 28-bit addressing only.  For 48-bit addressing See 7.31.2.2.  The maximum size of the SMART comprehensive error log  shall be 51 sectors.  Devices may support fewer than 51 sectors.  All multi-byte fields shown in this structure follow the byte ordering described in volume 1, Clause 3.  The comprehensive error log data structures shall include UNC errors, IDNF errors for which the address requested was valid, servo errors, write fault errors, etc.  Comprehensive error log data structures shall not include errors attributed to the receipt of faulty commands such as command codes not supported by the device or requests with invalid parameters or invalid addresses.

Table 59 - Comprehensive error log 59

| Byte | First sector | Subsequent sectors |
| --- | --- | --- |
| 0 | SMART error log version | Reserved |
| 1 | Error log index | Reserved |
| 2-91 | First error log data structure | Data structure 5n+1 |
| 92-181 | Second error log data structure | Data structure 5n+2 |
| 182-271 | Third error log data structure | Data structure 5n+3 |
| 272-361 | Fourth error log data structure | Data structure 5n+4 |
| 362-451 | Fifth error log data structure | Data structure 5n+5 |
| 452-453 | Device error count | Reserved |
| 454-510 | Reserved | Reserved |
| 511 | Data structure checksum | Data structure checksum |
| n is the sector number within the log. The first sector is sector zero |  |  |

            - Error log version

The value of the error log version byte shall be set to 01h.

            - Error log index

The error log index indicates the error log data structure representing the most recent error.  If there have been no error log entries, the error log index is set to zero.  Valid values for the error log index are zero to 255.

            - Error log data structure

The error log is viewed as a circular buffer.  The device may support from two to 51 error log sectors.  When the last supported error log sector has been filled, the next error shall create an error log data structure that replaces the first error log data structure in sector zero.  The next error after that shall create an error log data structure that replaces the second error log data structure in sector zero.  The sixth error after the log has filled shall replace the first error log data structure in sector one, and so on.

The error log index indicates the most recent error log data structure.  Unused error log data structures shall be filled with zeros.

The content of the error log data structure entries is defined in 7.54.7.2.3.3.

            - Device error count

The device error count field is defined in 7.54.7.2.3.4.

            - Data structure checksum

The data structure checksum is defined in 7.54.7.2.3.5.

###### Self-test log sector

Table 60 defines the 512 bytes that make up the SMART self-test log sector.  All multi-byte fields shown in this structure follow the byte ordering described in Volume 1, Clause 3.  The self-test log sector supports 28-bit addressing only.

Table 60 - Self-test log data structure60

| Byte | Descriptions |
| --- | --- |
| 0-1 | Self-test log data structure revision number |
| 2-25 | First descriptor entry |
| 26-49 | Second descriptor entry |
| ..... | ............ |
| 482-505 | Twenty-first descriptor entry |
| 506-507 | Vendor specific |
| 508 | Self-test index |
| 509-510 | Reserved |
| 511 | Data structure checksum |

This log is viewed as a circular buffer.  The first entry shall begin at byte 2, the second entry shall begin at byte 26, and so on until the twenty-second entry, that shall replace the first entry.  Then, the twenty-third entry shall replace the second entry, and so on.  If fewer than 21 self-tests have been performed by the device, the unused descriptor entries shall be filled with zeroes.

            - Self-test log data structure revision number

The value of the self-test log data structure revision number shall be 0001h.

            - Self-test log descriptor entry

The content of the self-test descriptor entry is shown in Table 61.

Table 61 - Self-test log descriptor entry61

| Byte | Descriptions |
| --- | --- |
| n | Content of the LBA Low register. |
| n+1 | Content of the self-test execution status byte. |
| n+2 | Life timestamp (least significant byte). |
| n+3 | Life timestamp (most significant byte). |
| n+4 | Content of the self-test failure checkpoint byte. |
| n+5 | Failing LBA (least significant byte). |
| n+6 | Failing LBA (next least significant byte). |
| n+7 | Failing LBA (next most significant byte). |
| n+8 | Failing LBA (most significant byte). |
| n+9 to n+23 | Vendor specific. |

Content of the LBA Low register shall be the content of the LBA Low register when the nth self-test subcommand was issued (See 7.54.5.1).

Content of the self-test execution status byte shall be the content of the self-test execution status byte when the nth self-test was completed (See 7.54.6.4).

Life timestamp shall contain the power-on lifetime of the device in hours when the nth self-test subcommand was completed.

Content of the self-test failure checkpoint byte may contain additional information about the self-test that failed.

The failing LBA shall be the LBA of the uncorrectable sector that caused the test to fail.  If the device encountered more than one uncorrectable sector during the test, this field shall indicate the LBA of the first uncorrectable sector encountered.  If the test passed or the test failed for some reason other than an uncorrectable sector, the value of this field is undefined.

            - Self-test index

The self-test index shall point to the most recent entry.  Initially, when the log is empty, the index shall be set to zero.  It shall be set to one when the first entry is made, two for the second entry, etc., until the 22nd entry, when the index shall be reset to one.

            - Data structure checksum

The data structure checksum is the two's complement of the sum of the first 511 bytes in the data structure.  Each byte shall be added with unsigned arithmetic, and overflow shall be ignored.  The sum of all 512 bytes is zero when the checksum is correct.  The checksum is placed in byte 511.

###### Selective self-test log

The Selective self-test log is a log that may be both written and read by the host.  This log allows the host to select the parameters for the self-test and to monitor the progress of the self-test.  Table 62 defines the content of the Selective self-test log.

Table 62 - Selective self-test log 62

| Byte | Description | Read/write |
| --- | --- | --- |
| 0-1 | Data structure revision number | R/W |
| 2-9 | Starting LBA for test span 1 | R/W |
| 10-17 | Ending LBA for test span 1 | R/W |
| 18-25 | Starting LBA for test span 2 | R/W |
| 26-33 | Ending LBA for test span 2 | R/W |
| 34-41 | Starting LBA for test span 3 | R/W |
| 42-49 | Ending LBA for test span 3 | R/W |
| 50-57 | Starting LBA for test span 4 | R/W |
| 58-65 | Ending LBA for test span 4 | R/W |
| 66-73 | Starting LBA for test span 5 | R/W |
| 74-81 | Ending LBA for test span 5 | R/W |
| 82-337 | Reserved | Reserved |
| 338-491 | Vendor specific | Vendor specific |
| 492-499 | Current LBA under test | Read |
| 500-501 | Current span under test | Read |
| 502-503 | Feature flags | R/W |
| 504-507 | Vendor specific | Vendor specific |
| 508-509 | Selective self-test pending time | R/W |
| 510 | Reserved | Reserved |
| 511 | Data structure checksum | R/W |

            - Data structure revision number

The value of the data structure revision number filed shall be 01h.  This value shall be written by the host and returned unmodified by the device.

            - Test span definition

The Selective self-test log provides for the definition of up to five test spans.  The starting LBA for each test span is the LBA of the first sector tested in the test span and the ending LBA for each test span is the last LBA tested in the test span.  If the starting and ending LBA values for a test span are both zero, a test span is not defined and not tested.  These values shall be written by the host and returned unmodified by the device.

            - Current LBA under test

The Current LBA under test field shall be written with a value of zero by the host.  As the self-test progresses, the device shall modify this value to contain the beginning LBA of the 65,536 sector block currently being tested.  When the self-test including the off-line scan between test spans has been completed, a zero value is placed in this field.

            - Current span under test

The Current span under test field shall be written with a value of zero by the host.  As the self-test progresses, the device shall modify this value to contain the test span number of the current span being tested.  If an off-line scan between test spans is selected, a value greater then five is placed in this field during the off-line scan.  When the self-test including the off-line scan between test spans has been completed, a zero value is placed in this field.

            - Feature flags

The Feature flags define the features of Selective self-test to be executed (See Table 63).

Table 63 - Selective self-test feature flags 63

| Bit | Description |
| --- | --- |
| 0 | Vendor specific |
| 1 | When set to one, perform off-line scan after selective test. |
| 2 | Vendor specific |
| 3 | When set to one, off-line scan after selective test is pending. |
| 4 | When set to one, off-line scan after selective test is active. |
| 5-15 | Reserved. |

Bit (1) shall be written by the host and returned unmodified by the device.  Bits (4:3) shall be written as zeros by the host and the device shall modify them as the test progresses.

            - Selective self-test pending time

The selective self-test pending time is the time in minutes from power-on to the resumption of the off-line testing if the pending bit is set.  At the expiration of this time, sets the active bit to one, and resumes the off-line scan that had begun before power-down.

            - Data structure checksum

The data structure checksum is defined in 7.54.7.2.3.5.

        - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D5h - SMART READ LOG |  |  |  |
| 01h | Count | Specifies the number of sectors to be read from the specified log. The log transferred by the drive shall start at the first sector in the specified log, regardless of the sector count requested. Bits 15:8 shall be cleared to zero |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | Specifies the log to be returned as described in Table 36. If this command is implemented, all address values for which the contents are defined shall be implemented and all address values defined as host vendor specific shall be implemented. The host vendor specific logs may be used by the host to store any data desired. If a host vendor specific log has never been written by the host, when read the content of the log shall be zeros. Device vendor specific logs may be used by the device vendor to store any data and need only be implemented if used. |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 65

        - Error outputs

Uncorrectable Error shall be set to one if SMART data is uncorrectable.  ID Note Found shall be set to one if SMART data sector’s ID field could not be found or a data structure checksum occurred.  Abort shall be set to one if SMART is not enabled, or if register values are invalid.  See Table 86.

#### SMART RETURN STATUS - B0h/DAh, Non-data

        - Feature Set

This command is mandatory for devices that implement the SMART feature set.

        - Description

This command causes the device to communicate the reliability status of the device to the host.  If a threshold exceeded condition is not detected by the device, the device shall set the LBA Mid register to 4Fh and the LBA High register to C2h.  If a threshold exceeded condition is detected by the device, the device shall set the LBA Mid register to F4h and the LBA High register to 2Ch.

        - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | DAh - SMART RETURN STATUS |  |  |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | Specifies the log to be returned as described in Table 36. If this command is implemented, all address values for which the contents are defined shall be implemented and all address values defined as host vendor specific shall be implemented. The host vendor specific logs may be used by the host to store any data desired. If a host vendor specific log has never been written by the host, when read the content of the log shall be zeros. Device vendor specific logs may be used by the device vendor to store any data and need only be implemented if used. |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 76

        - Error outputs

Abort shall be set to one if SMART is not enabled.  See Table 79.

#### SMART WRITE LOG - D6h, PIO data-out

        - Feature Set

This command is optional for devices that implement the SMART feature set.

        - Description

[Editors Note: This description seems a bit thin to me.  There is a reference to the log table list in the command]

This command writes the specified number of 512 byte data sectors to the specified log. [Editors Note: Should we call this a sector]

        - Inputs

The Features register shall be set to D6h.  The Sector Count register shall specify the number of sectors that shall be written to the log number specified by the LBA Low register.  The LBA Mid register shall be set to 4Fh.  The LBA High register shall be set to C2h.

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | D6h - SMART WRITE LOG |  |  |  |
| 01h | Count | Specifies the number of sectors that shall be written to the specified log. The log transferred to the drive shall be stored by the drive starting at the first sector in the specified log. Bits 15:8 shall be cleared to zero |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:24 | Reserved |  |
|  |  |  | 23:8 | C24Fh |  |
|  |  |  | 7:0 | Specifies the log to be written as described in Table 36. If this command is implemented, all address values defined as host vendor specific shall be implemented. |  |
| 05h | Command | B0h |  |  |  |

        - Normal outputs

See Table 65

        - Error outputs

ID Note Found shall be set to one if SMART log sector’s ID field could not be found.  Abort shall be set to one if SMART is not enabled, if the log sector address is not implemented.  See Table 89.

### STANDBY - E2h, Non-data

      - Feature Set

This commans is mandatory for devices that implement the Power Management feature set

      - Description

This command causes the device to enter the Standby mode.

If the Count field is non-zero then the Standby timer shall be enabled.  The value in the Count field shall be used to determine the time programmed into the Standby timer (See Table 18).

If the Count field is zero then the Standby timer is disabled.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | This value shall determine the time period programmed into the Standby timer. Table 18 defines these values. |
| 02h-04h | LBA | N/A |
| 05h | Command | E2h |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79.

### STANDBY IMMEDIATE - E0h, Non-data

      - Feature Set

This commans is mandatory for devices that implement the Power Management feature set

      - Description

This command causes the device to immediately enter the Standby mode.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | E2h |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79.

### TRUSTED RECEIVE – 5Ch

Reserved for Trusted Computing Group

### TRUSTED RECEIVE DMA – 5Dh

Reserved for Trusted Computing Group

### TRUSTED SEND – 5Eh

Reserved for Trusted Computing Group

### TRUSTED SEND DMA – 5Fh

Reserved for Trusted Computing Group

### WRITE BUFFER - E8h, PIO data-out

      - Feature Set

This command is optional for devices that implement the General feature set

      - Description

This command enables the host to write the contents of one sector in the device’s buffer.

The READ BUFFER and WRITE BUFFER commands shall be synchronized within the device such that sequential WRITE BUFFER and READ BUFFER commands access the same 512 bytes within the buffer.

      - Inputs

| Word | Name | Description |
| --- | --- | --- |
| 00h | Feature | N/A |
| 01h | Count | N/A |
| 02h-04h | LBA | N/A |
| 05h | Command | E8h |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79.

### WRITE DMA - CAh, DMA

      - Feature Set

This command is mandatory for devices that implement the General feature set

      - Description

The WRITE DMA command allows the host to write data using the DMA data transfer protocol.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C8h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79.

### WRITE DMA EXT - 35h, DMA

      - Feature Set

This command is mandatory for devices that implement the 48-bit Address feature set

      - Description

The WRITE DMA EXT command allows the host to write data using the DMA data transfer protocol.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C8h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79.

### WRITE DMA FUA EXT - 3Dh, DMA

      - Feature Set

This command is mandatory for devices that implement the 48-bit Address feature set

      - Description

The WRITE DMA FUA EXT command provides the same function as the WRITE DMA EXT command except that regardless of whether write caching in the device is enabled or not, the user data shall be written to the media before ending status for the command is reported.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C8h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

See Table 79.

### WRITE DMA QUEUED - CCh, DMA queued

      - Feature Set

This command is mandatory for devices that implement the Overlapped feature set

      - Description

This command executes in a similar manner to a WRITE DMA command.  The device may perform a bus release the bus or may execute the data transfer without performing a bus release if the data is ready to transfer.

If the device performs a bus release, the host shall reselect the device using the SERVICE command.

Once the data transfer is begun, the device shall not perform a bus release until the entire data transfer has been completed.

      - Inputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |  |  |  |
| 01h | Count |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:3 | Tag - See clause 6.4.4 |  |  |  |
|  |  |  | 2:0 | N/A |  |  |  |
| 02h | LBA | MSB |  |  | Address of first sector to be transferred. Bits 47:28 snall be cleared to zero. |  |  |
| 03h |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  | LSB |  |
| 05h | Command | CCh |  |  |  |  |  |

      - Normal outputs

        - Data transmission

Data transfer may occur after receipt of the command or may occur after the receipt of a SERVICE command.  When the device is ready to transfer data requested by a data transfer command, the device sets the following field content to initiate the data transfer.  See Table 73 for the layout of the normal outputs data structure.  Release shall be cleared to zero, Input/Output Shall be cleared to zero, Command/Data shall be cleared to zero.

        - Bus release

If the device performs a bus release before transferring data for this command, the field content upon performing a bus release shall be as shown below.  See Table 73 for the layout of the normal outputs data structure.  Release shall be set to one, Input/Output Shall be cleared to zero, Command/Data shall be cleared to zero.

        - Service request

When the device is ready to transfer data or complete a command after the command has performed a bus release, the device shall set the SERV bit and not change the state of any other register bit ([Editors Note: Was See clause 10]).  When the SERVICE command is received, the device shall set outputs as described in data transfer, command completion, or error outputs depending on the service the device requires.

        - Command completion

When the transfer of all requested data has occurred without error, the field content shall be as shown below.  See Table 73 for the layout of the normal outputs data structure.  Release shall be cleared to zero, Input/Output Shall be set to one, Command/Data shall be set to one.

      - Error outputs

The Sector Count register contains the Tag for this command if the device supports command queuing.  The device shall return command aborted if the command is not supported.  The device shall return command aborted if the device supports command queuing and the Tag is invalid.  An unrecoverable error encountered during the execution of this command results in the termination of the command and the Command Block registers contain the sector where the first unrecoverable error occurred.  If a queue existed, the unrecoverable error shall cause the queue to abort.  The device may remain BSY for some time when responding to these errors.  [Editors Note: Do we still need I/O and C/D? I am not sure they are defined correctly here]

| Word | Name | Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |  |
|  |  |  |  | 6 | Write Protect - See clause 6.3.13 |  |  |  |
|  |  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |  |
|  |  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |  |
|  |  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |  |
|  |  |  |  | 2 | Abort - See clause 6.3.1 |  |  |  |
|  |  |  |  | 1 | No Media - See clause 6.3.10 |  |  |  |
|  |  |  |  | 0 | N/A |  |  |  |
| 01h | Interrupt Reasont |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4. If the device supports command queuing, this field shall contain the Tag of the command being bus released. |  |  |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be cleared to zero |  |  |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. Shall be set to one |  |  |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be set to one |  |  |  |
| 02h | LBA | MSB |  |  |  | Address of first unrecoverable error. Bits 47:28 shall be cleared to zero. |  |  |
| 03h |  |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |  |  |
|  |  |  | Bit |  |  | Description |  |  |
|  |  |  | 15:8 |  |  | Reserved |  |  |
|  |  |  | 7:6 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 |  |  | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 |  |  | Service - See clause 6.2.9 |  |  |
|  |  |  | 3 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 |  |  | N/A |  |  |
|  |  |  | 0 |  |  | Error - See clause 6.2.3 |  |  |

### WRITE DMA QUEUED EXT - 36h, DMA queued

      - Feature Set

This command is mandatory for devices that implement both the Overlapped feature set and 48-bit Address feature set

      - Description

This command executes in a similar manner to a WRITE DMA EXT command.  The device may perform a bus release the bus or may execute the data transfer without performing a bus release if the data is ready to transfer.

If the device performs a bus release, the host shall reselect the device using the SERVICE command.

Once the data transfer is begun, the device shall not perform a bus release until the entire data transfer has been completed.

      - Inputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. |  |  |  |  |  |
| 01h | Count |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:3 | Tag - See clause 6.4.4 |  |  |  |
|  |  |  | 2:0 | N/A |  |  |  |
| 02h | LBA | MSB |  |  | Address of first sector to be transferred |  |  |
| 03h |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  | LSB |  |
| 05h | Command | 36h |  |  |  |  |  |

      - Normal outputs

See Clause 7.65.4

      - Error outputs

The Interrupt Reason field contains the Tag for this command if the device supports command queuing.  The device shall return command aborted if the command is not supported.  The device shall return command aborted if the device supports command queuing and the Tag is invalid.  An unrecoverable error encountered during the execution of this command results in the termination of the command and the Command Block registers contain the sector where the first unrecoverable error occurred.  If a queue existed, the unrecoverable error shall cause the queue to abort.  The device may remain BSY for some time when responding to these errors.  [Editors Note: Do we still need I/O and C/D? I am not sure they are defined correctly here]

| Word | Name | Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |  |
|  |  |  |  | 6 | Write Protect - See clause 6.3.13 |  |  |  |
|  |  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |  |
|  |  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |  |
|  |  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |  |
|  |  |  |  | 2 | Abort - See clause 6.3.1 |  |  |  |
|  |  |  |  | 1 | No Media - See clause 6.3.10 |  |  |  |
|  |  |  |  | 0 | N/A |  |  |  |
| 01h | Interrupt Reason |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4. If the device supports command queuing, this field shall contain the Tag of the command being bus released. |  |  |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be cleared to zero |  |  |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. Shall be set to one |  |  |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be set to one |  |  |  |
| 02h | LBA | MSB |  |  |  | Address of first unrecoverable error |  |  |
| 03h |  |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |  |  |
|  |  |  | Bit |  |  | Description |  |  |
|  |  |  | 15:8 |  |  | Reserved |  |  |
|  |  |  | 7:6 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 |  |  | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 |  |  | Service - See clause 6.2.9 |  |  |
|  |  |  | 3 |  |  | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 |  |  | N/A |  |  |
|  |  |  | 0 |  |  | Error - See clause 6.2.3 |  |  |

### WRITE DMA QUEUED FUA EXT - 3Eh, DMA queued

      - Feature Set

This command is mandatory for devices that implement both the Overlapped feature set and 48-bit Address feature set.

      - Description

This command executes in a similar manner to a WRITE DMA EXT command.  The device may perform a bus release or may execute the data transfer without performing a bus release if the data is ready to transfer.

If the device performs a bus release, the host shall reselect the device using the SERVICE command.

Once the data transfer is begun, the device shall not perform a bus release until the entire data transfer has been completed.

The WRITE DMA QUEUED FUA EXT command provides the same function as the WRITE DMA EXT command.  It is an Overlapped feature set command and when issued it shall not cause an existing queue to be aborted.  However, regardless of whether write caching in the device is enabled or not, the user data shall be written to the media before ending status for the command is reported.

      - Inputs

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. |  |  |  |  |  |
| 01h | Count |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:3 | Tag - See clause 6.4.4 |  |  |  |
|  |  |  | 2:0 | N/A |  |  |  |
| 02h | LBA | MSB |  |  | Address of first sector to be transferred |  |  |
| 03h |  |  |  |  |  |  |  |
| 04h |  |  |  |  |  | LSB |  |
| 05h | Command | 36h |  |  |  |  |  |

      - Normal outputs

See Clause 7.65.4

      - Error outputs

See Clause 7.66.5

### WRITE LOG EXT - 3Fh, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the General Purpose Logging feature set.

      - Description

This command writes a specified number of 512 byte data sectors to the specified log.  The device shall interrupt for each DRQ block transferred.

      - Inputs

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |  |
| 01h | Count | Sector Count - Specifies the number of sectors that shall be written to the specified log. If the number is greater than the number indicated in the Log directory (which is available in Log number zero), the device shall return command aborted. The log transferred to the drive shall be stored by the drive starting at the first sector in the specified log. |  |  |  |
| 02h-04h | LBA |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 47:32 | Reserved |  |
|  |  |  | 31:16 | Sector Offset - Specifies the first sector of the log to be written. |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:0 | Log Address - Specifies the log to be written as described in Table 36. A device may support a subset of the available logs. Support for individual logs is determined by support for the associated feature set. Support of the associated log(s) is mandatory for devices implementing the associated feature set. If this command is implemented, all address values defined as host vendor specific shall be implemented. These host vendor specific logs may be used by the host to store any data desired. Support for device vendor specific logs is optional. If the host attempts to write to a read only (RO) log address, the device shall return command aborted. [Editors Note: Some of this stuff looks like it would be better stated in the command description. Yes] |  |
| 05h | Command | 3Fh |  |  |  |

      - Normal outputs

See Table 65

      - Error outputs

If the device does not support this command, if the feature set associated with the log specified in the LBA Low register is not supported or enabled, or if the values in the Features, Sector Count, LBA Mid, or LBA High registers are invalid, the device shall return command aborted.  If the host attempts to write to a read only (RO) log address, the device shall return command aborted.  ID Not Found shall be set to one if the log sector’s ID field was not found or data structure checksum error occurred.  Abort shall be set to one if the feature associated with the log specified in bit 7:0 of the LBA field is not supported or not enabled.  See Table 89.

### WRITE MULTIPLE - C3h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the General feature set

      - Description

This command writes the number of sectors specified in the Sector Count register.

The number of sectors per block is defined by the content of word 59 of the IDENTIFY DEVICE response.  The device shall interrupt for each DRQ block transferred.

When the WRITE MULTIPLE command is issued, the Sector Count register contains the number of sectors (not the number of blocks) requested.

If the number of requested sectors is not evenly divisible by the block count, as many full blocks as possible are transferred, followed by a final, partial block transfer.  The partial block transfer is for n sectors, where:

n = Remainder (sector count/ block count).

If the WRITE MULTIPLE command is received when WRITE MULTIPLE commands are disabled, the Write Multiple operation shall be rejected with command aborted.

Device errors encountered during WRITE MULTIPLE commands are posted after the attempted device write of the block or partial block transferred.  The command ends with the sector in error, even if the error was in the middle of a block.  Subsequent blocks are not transferred in the event of an error.

The contents of the Command Block Registers following the transfer of a data block that had a sector in error are undefined.  The host should retry the transfer as individual requests to obtain valid error information.  Interrupt pending is set when the DRQ bit is set to one at the beginning of each block or partial block.

If bit 8 of IDENTIFY DEVICE data word 59 is cleared to zero, a successful SET MULTIPLE MODE command shall proceed a WRITE MULTIPLE command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C3h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  LBA field Bits 47:28 shall be cleared to zero.  See Table 91.

### WRITE MULTIPLE EXT - 39h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the 48-bit Address feature set

      - Description

This command writes the number of sectors specified in the Sector Count register.

The number of sectors per block is defined by the content of word 59 in the IDENTIFY DEVICE response.  The device shall interrupt for each DRQ block transferred.

When the WRITE MULTIPLE EXT command is issued, the Sector Count register contains the number of sectors (not the number of blocks) requested.

If the number of requested sectors is not evenly divisible by the block count, as many full blocks as possible are transferred, followed by a final, partial block transfer.  The partial block transfer is for n sectors, where:

n = Remainder (sector count / block count).

If the WRITE MULTIPLE EXT command is received when WRITE MULTIPLE EXT commands are disabled, the Write Multiple operation shall be rejected with command aborted.

Device errors encountered during WRITE MULTIPLE EXT commands are posted after the attempted device write of the block or partial block transferred.  The command ends with the sector in error, even if the error was in the middle of a block.  Subsequent blocks are not transferred in the event of an error.

The contents of the Command Block Registers following the transfer of a data block that had a sector in error are undefined.  The host should retry the transfer as individual requests to obtain valid error information.  Interrupt pending is set when the DRQ bit is set to one at the beginning of each block or partial block.

If bit 8 of IDENTIFY DEVICE data word 59 is cleared to zero, a successful SET MULTIPLE MODE command shall proceed a WRITE MULTIPLE EXT command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C3h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  See Table 91.

### WRITE MULTIPLE FUA EXT - CEh, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the 48-bit Address feature set

      - Description

The WRITE MULTIPLE FUA EXT command provides the same functionality as the WRITE MULTIPLE EXT command except that regardless of whether write caching in the device is enabled or not, the user data shall be written to the media before ending status for the command is reported.

If bit 8 of IDENTIFY DEVICE data word 59 is cleared to zero, a successful SET MULTIPLE MODE command shall proceed a WRITE MULTIPLE FUA EXT command.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C3h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  See Table 91

### WRITE SECTOR(S) - 30h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the General feature set

      - Description

This command writes from 1 to 256 sectors as specified in the Count field.  A count of 0 requests 256 sectors.  The device shall interrupt for each DRQ block transferred.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 00h indicates that 256 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred. Bits 47:28 shall be cleared to zero |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C3h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  LBA field Bits 47:28 shall be cleared to zero.  See Table 91.

### WRITE SECTOR(S) EXT - 34h, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the 48-bit Address feature set

      - Description

This command writes from 1 to 65,536 sectors as specified in the Sector Count register.  A sector count value of 0000h requests 65,536 sectors.  The device shall interrupt for each DRQ block transferred.

      - Inputs

| Word | Name | Description |  |  |
| --- | --- | --- | --- | --- |
| 00h | Feature | Reserved |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65,536 sectors are to be transferred. Bits 15:8 shall be cleared to zero |  |  |
| 02h | LBA | MSB | Address of first sector to be transferred |  |
| 03h |  |  |  |  |
| 04h |  |  |  | LSB |
| 05h | Command | C3h |  |  |

      - Normal outputs

See Table 65

      - Error outputs

An unrecoverable error encountered during the execution of this command results in the termination of the command.  The return fields contain the address of the sector where the first unrecoverable error occurred.  The amount of data transferred is indeterminate.  See Table 91.

### WRITE STREAM DMA EXT - 3Ah, DMA

      - Feature Set

This command is mandatory for devices that implement the Streaming feature set.

      - Description

The Write Stream DMA EXT command allows the host to write data using the DMA data transfer protocol.  This command allows for the host to specify to the device that additional actions need to be performed prior to the completion of the command if the required bits are set.

If the Write Continuous bit is set to one,the device shall not stop execution of the command due to errors.  If the WC bit is set to one and errors occur in the transfer or writing of the data, the device shall continue to transfer the amount of data requested and then provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and the type of error, ICRC, IDNF, or ABRT, reported in the error log.  If the WC bit is set to one and the Command Completion Time Limit expires, the device shall stop execution of the command and provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and report the fact that the Command Completion Time Limit expired by setting the CCTO bit in the error log to one.  In all cases, the device shall attempt to transfer the amount of data requested within the Comand Completion Time Limit even if some data transferred is in error.

      - Inputs

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Command Completion Time Limit (CCTO)- See clause 7.74.6. |  |  |
|  |  |  | 7 | Urgent - The Urgent bit specifies an urgent transfer request. The Urgent bit specifies that the command should be completed in the minimum possible time by the device and shall be completed within the specified Command Completion Time Limit. This bit is optional (See 7.17.7.42). |  |  |
|  |  |  | 6 | Write Continuous - See clause 7.74.4 |  |  |
|  |  |  | 5 | Flush - See clause 7.74.5 |  |  |
|  |  |  | 4 | Handle Streaming Error (HSE) - If HSE is set to one this command starts at the LBA of the last reported error for this stream, so the device may attempt to continue its corresponding error recovery sequence where it left off earlier |  |  |
|  |  |  | 3 | Reserved |  |  |
|  |  |  | 2:0 | Stream ID - Specifies the stream being written. The device shall operate according to the Stream ID set by the WRITE STREAM command. Any write of the device media or internal device buffer management as a result of the Stream ID is device vendor specific. |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65536 sectors are to be transferred. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first sector to be transferred. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Command | 3Ah |  |  |  |  |

      - Write Continuous (WC)

WC specifies that the Write Continuous mode enabled.  If the Write Continuous bit is set to one, the device shall not stop execution of the command due to errors.

If the WC bit is set to one and errors occur in the transfer or writing of the data, the device shall continue to transfer the amount of data requested and then provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and the type of error, ICRC, IDNF, or ABRT, reported in the error log.

If the WC bit is set to one and the Command Completion Time Limit expires, the device shall stop execution of the command and provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and report the fact that the Command Completion Time Limit expired by setting the CCTO bit in the error log to one.

In all cases, the device shall attempt to transfer the amount of data requested within the Comand Completion Time Limit even if some data transferred is in error.

      - Flush

Flush specifies that all data for the specified stream shall be flushed to the media before command complete is reported when set to one.

      - Command Completion Time Limit (CCTO)

CCTO is the time allowed for the current command’s completion is calculated as follows:

CCTO = (content of the Features register Previous)  (IDENTIFY DEVICE data words (99:98)) microseconds

If the valuCONFIGURE STREAM command for this Stream ID.  If the Default Command Completion Time Limit is zero, or no previous Configure Stream command was defined for this Stream ID, the result is vendor specific.  The time is measured from the write of the command register to the final INTRQ for command completion

      - Normal Outputs

See Table 69

      - Error Outputs

Error shall be set to one if an Error register bit is set to one and the WC bit is cleared to zero.  If the WC bit is set to one when the command is issued and an ICRC, IDNF, ABRT, or CCTO error occurs, the SE bit shall be set to one, the ERR bit shall be cleared to zero, and the bits that would normally be set in the Error register shall be set in the error log.  See Table 92.

### WRITE STREAM EXT - 3Bh, PIO data-out

      - Feature Set

This command is mandatory for devices that implement the Streaming feature set.

      - Description

This command writes from 1 to 65,536 sectors as specified in the Sector Count register.  A sector count of 0 requests 65,536 sectors.

If the Write Continuous bit is set to one,the device shall not stop execution of the command due to errors.  If the WC bit is set to one and errors occur in the transfer or writing of the data, the device shall continue to transfer the amount  of data requested and then provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and the type of error, IDNF, or ABRT, reported in the error log.  If the WC bit is set to one and the Command Completion Time Limit expires, the device shall stop execution of the command and provide ending status with the BSY bit cleared to zero, the SE bit set to one, the ERR bit cleared to zero, and report the fact that the Command Completion Time Limit expired by setting the CCTO bit in the error log to one.  In all cases, the device shall attempt to transfer the amount of data requested within the Comand Completion Time Limit even if some data transferred is in error.

      - Inputs

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Feature |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Command Completion Time Limit (CCTO)- See clause 7.74.6. |  |  |
|  |  |  | 7 | Urgent - The Urgent bit specifies an urgent transfer request. The Urgent bit specifies that the command should be completed in the minimum possible time by the device and shall be completed within the specified Command Completion Time Limit. This bit is optional (See 7.17.7.42). |  |  |
|  |  |  | 6 | Write Continuous - See clause 7.74.4 |  |  |
|  |  |  | 5 | Flush - See clause 7.74.5 |  |  |
|  |  |  | 4 | Handle Streaming Error (HSE) - If HSE is set to one this command starts at the LBA of the last reported error for this stream, so the device may attempt to continue its corresponding error recovery sequence where it left off earlier |  |  |
|  |  |  | 3 | Reserved |  |  |
|  |  |  | 2:0 | Stream ID - Specifies the stream being written. The device shall operate according to the Stream ID set by the WRITE STREAM command. Any write of the device media or internal device buffer management as a result of the Stream ID is device vendor specific. |  |  |
| 01h | Count | The number of sectors to be transferred. A value of 0000h indicates that 65536 sectors are to be transferred. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first sector to be transferred. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Command | 3Ah |  |  |  |  |

      - Normal Outputs

See Table 69

      - Error Outputs

Error shall be set to one if an Error register bit is set to one and the WC bit is cleared to zero.  If the WC bit is set to one when the command is issued and an ICRC, IDNF, ABRT, or CCTO error occurs, the SE bit shall be set to one, the ERR bit shall be cleared to zero, and the bits that would normally be set in the Error register shall be set in the error log.  See Table 93.

## Normal & Error Outputs

### Overview

The commands listed in clause 7 each have sections labeled “Normal Outputs” and “Error Outputs”.  Clauses 8.2 and 8.3 document the return data format for all the commands described in clause 7.  Each command in clause 7 may provide additional information about a normal or error output, but all the information that is referenced in clause Error: Reference source not found applies applies to the command as well.

### Normal Outputs

The following tables document all the possible normal outputs a command returns.  References to these tables are found in clause 7.

Table 64 – Extended Error Code

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Extended error code (See Table 9) |  |  |  |  |
| 01h | Count | Vendor Specific |  |  |  |  |
| 02h | LBA | MSB |  | Vendor Specific Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5:4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 65 – Device Fault w/Error Bit Defined Normal Output

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 66 – Normal Output

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 67 – Media Card Type Normal Output

| Word | Name | Description |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |  |  |  |  |  |
| 01h | Count | 55h |  |  |  |  |  |  |  |  |
| 02h-04h | LBA |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |  |  |  |  |
|  |  |  |  | 47:25 | Reserved |  |  |  |  |  |
|  |  |  |  | 24:8 | Card specific Data |  |  |  |  |  |
|  |  |  |  | 7:0 | Shall be set to AAh |  |  |  |  |  |
| 05h | Device/ Status |  |  |  |  |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |  |  |  |  |
|  |  |  | 15:12 |  | Reserved |  |  |  |  |  |
|  |  |  | 11 |  | Write Protect - shall be set to one if the device is write protected, Write Protect shall be cleared to zero if the device is not write protected. |  |  |  |  |  |
|  |  |  | 10:8 |  |  | Media Type |  |  |  |  |
|  |  |  |  |  |  | Value | Description |  |  |  |
|  |  |  |  |  |  | 000b | Reserved |  |  |  |
|  |  |  |  |  |  | 001b | SD Memory Card |  |  |  |
|  |  |  |  |  |  | 010b | MMC Card |  |  |  |
|  |  |  |  |  |  | 011b | SD-IO Card |  |  |  |
|  |  |  |  |  |  | 100b | Smart Media Card |  |  |  |
|  |  |  |  |  |  | 101b-111b | Reserved |  |  |  |
|  |  |  | Bit |  | Description |  |  |  |  |  |
|  |  |  | 7:6 |  | Transport Dependent - See clause 6.2.11. |  |  |  |  |  |
|  |  |  | 5 |  | Device Fault - See clause 6.2.4 |  |  |  |  |  |
|  |  |  | 4:1 |  | N/A |  |  |  |  |  |
|  |  |  | 0 |  | Error - See clause 6.2.3 |  |  |  |  |  |

Table 68 – Check Power Mode

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |  |
| 01h | Count |  |  |  |  |  |
|  |  |  | Value |  | Description |  |
|  |  |  | 00h |  | Device is in Standby mode. |  |
|  |  |  | 80h |  | Device is in Idle mode |  |
|  |  |  | FFh |  | Device is in Active mode or Idle mode. |  |
| 02h-04h | LBA | N/A |  |  |  |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 69 – Stream Normal

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Stream Error - See clause 6.2.8 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 70 – Device Reset and Execute Diagnostics

| Word | Name | Description |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Diagnostic Results - The diagnostic code as described in Table 14. |  |  |  |  |  |  |  |  |
|  |  |  |  | Bit | General | Packet | Reserved for SATA | Reserved for SATA | All other signatures reserved |  |
| 01h | Count |  |  | 7:0 | 01h | 01h | 01h | 01h |  |  |
| 02h-04h | LBA |  |  | 47:24 | Reserved | Reserved | Reserved | Reserved |  |  |
|  |  |  |  | 23:16 | 00h | 14h | C3h | 96h |  |  |
|  |  |  |  | 15:8 | 00h | EBh | 3Ch | 69h |  |  |
|  |  |  |  | 7:0 | 01h | 01h | 01h | 01h |  |  |
| 05h | Status |  |  |  |  |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |  |  |  |  |
|  |  |  | 15:8 |  | Reserved |  |  |  |  |  |
|  |  |  | 7:6 |  | Transport Dependent - See clause 6.2.11. |  |  |  |  |  |
|  |  |  | 5 |  | Device Fault - See clause 6.2.4 |  |  |  |  |  |
|  |  |  | 4 |  | N/A |  |  |  |  |  |
|  |  |  | 3 |  | Transport Dependent - See clause 6.2.11. |  |  |  |  |  |
|  |  |  | 2:1 |  | N/A |  |  |  |  |  |
|  |  |  | 0 |  | Error - See clause 6.2.3 |  |  |  |  |  |

Table 71 – IDLE Unload

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | 0000000000C4h |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 72 – ATAPI Normal Output

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |  |
| 01h | Interrupt Reason |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4 |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. Shall be cleared to zero |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. Shall be set to zero |  |
| 02h-04h | LBA |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |
|  |  |  |  | 47:24 | Reserved |  |
|  |  |  |  | 23:8 | Byte Count |  |
|  |  |  |  | 7:0 | Reserved |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |
|  |  |  | 15:8 |  | Reserved |  |
|  |  |  | 7 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 6 |  | N/A |  |
|  |  |  | 5 |  | DMA Ready - See clause 6.2.8. |  |
|  |  |  | 4 |  | Service - See clause 6.2.9. |  |
|  |  |  | 3 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 |  | N/A |  |
|  |  |  | 0 |  | Check Condition - See clause 6.2.2. Shall be cleared to zero |  |

Table 73 – Queued Normal Output

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | N/A |  |  |  |  |
| 01h | Interrupt Reason |  |  |  |  |  |
|  |  |  |  | Bit | Description |  |
|  |  |  |  | 7:3 | Tag - See clause 6.4.4 |  |
|  |  |  |  | 2 | Release - See clause 6.4.1. |  |
|  |  |  |  | 1 | Input/Output - See clause 6.4.2. |  |
|  |  |  |  | 0 | Command/Data - See clause 6.4.1. |  |
| 02h-04h | LBA | Reserved |  |  |  |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit |  | Description |  |
|  |  |  | 15:8 |  | Reserved |  |
|  |  |  | 7 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 6 |  | N/A |  |
|  |  |  | 5 |  | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 |  | Service - See clause 6.2.9. |  |
|  |  |  | 3 |  | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 |  | N/A |  |
|  |  |  | 0 |  | Error - See clause 6.2.3. Shall be cleared to zero |  |

Table 74 – SETMAX Normal Output

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Max address |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 75 – SMART Off-Line Immediate Normal Output

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |  |  |
| 01h | Count | Reserved |  |  |  |  |  |
| 02h-04h | LBA |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 47:24 | Reserved |  |  |  |
|  |  |  | 23:8 |  |  |  |  |
|  |  |  |  |  | Value | Description |  |
|  |  |  |  |  | C24Fh | Subcommand specified a captive self-test that has executed without failure. |  |
|  |  |  |  |  | N/A | the subcommand specified an off-line routine including an off-line self-test routine. |  |
|  |  |  | 7:0 | N/A |  |  |  |
| 05h | Status |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |  |
|  |  |  | 4 | N/A |  |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |  |
|  |  |  | 2 | N/A |  |  |  |
|  |  |  | 1 | N/A |  |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |  |

Table 76 – SMART Return Status Normal Output

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error | Reserved |  |  |  |  |  |
| 01h | Count | Reserved |  |  |  |  |  |
| 02h-04h | LBA |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 47:24 | Reserved |  |  |  |
|  |  |  | 23:8 |  |  |  |  |
|  |  |  |  |  | Value | Description |  |
|  |  |  |  |  | C24Fh | The device has not detected a threshold exceeded condition |  |
|  |  |  |  |  | 2CF4 | The device has detected a threshold exceeded condition |  |
|  |  |  | 7:0 | N/A |  |  |  |
| 05h | Status |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |  |
|  |  |  | 4 | N/A |  |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |  |
|  |  |  | 2 | N/A |  |  |  |
|  |  |  | 1 | N/A |  |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |  |

### Error Outputs

The following tables document all the possible error outputs a command returns.  References to these tables are found in clause 7

Table 77 –Error Status

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:5 | N/A |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | N/A |  |  |
|  |  |  | 2 | Abort - See Clause 6.3.1 |  |  |
|  |  |  | 1 | N/A |  |  |
|  |  |  | 0 | Media Error - See clause 6.3.5 |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 78 –Abort Only Error Status

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:3 | N/A |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1:0 | N/A |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error Bits 47:28 shall be cleared to zero |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 79 –Abort w/bit 3 Error Status

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:3 | N/A |  |
|  |  |  | 2 | Abort - See Clause 6.3.1 |  |
|  |  |  | 1:0 | N/A |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 80 – Stream Error

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:3 | N/A |  |
|  |  |  | 2 | Abort - See Clause 6.3.1. |  |
|  |  |  | 1 | N/A |  |
|  |  |  | 0 | Obsolete |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Stream Error - See clause 6.2.8 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 81 – Write Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:3 | N/A |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1:0 | N/A |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h-04h | LBA | MSB |  | Address of first unrecoverable error |  |  |
|  |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 82 –Media Status

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7 | N/A |  |
|  |  |  | 6 | Write Protect - See clause 6.3.13 |  |
|  |  |  | 5 | Media Change - 6.3.7 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Meda Change Request - See clause 6.3.8 |  |
|  |  |  | 2 | Abort - See clause 6.3.1. |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |
|  |  |  | 0 | Obsolete |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 83 –Media Eject

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:3 | N/A |  |
|  |  |  | 2 | Abort - See clause 6.3.1. |  |
|  |  |  | 1 | No Media - See clause 6.3.10. |  |
|  |  |  | 0 | Obsolete |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 84 –Media Lock

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:4 | N/A |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |
|  |  |  | 2 | Abort - See clause 6.3.1. |  |
|  |  |  | 1 | No Media - See clause 6.3.10. |  |
|  |  |  | 0 | Obsolete |  |
| 01h | Count | N/A |  |  |  |
| 02h-04h | LBA | N/A |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 85 – Read DMA Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |
|  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | Obsolete |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 86 –Read Log Ext Error

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7 | N/A |  |
|  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |
|  |  |  | 5 | N/A |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |
|  |  |  | 3 | N/A |  |
|  |  |  | 2 | Abort - See clause 6.3.1. |  |
|  |  |  | 1 | N/A |  |
|  |  |  | 0 | Obsolete |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 87 –Read PIO Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | N/A |  |  |
|  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | Obsolete |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 88 –Read Stream Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |
|  |  |  | 6 | Uncorrectable Error - See clause 6.3.12 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | Command Completion Time Limit Out - See clause 6.3.2 |  |  |
| 01h | Count | Length of Stream Error - number of contiguous sectors containing potentially bad data, beginning with the LBA of the first sector with an uncorrectable error. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Stream Error - See clause 6.2.10. |  |  |
|  |  |  | 4 | Deferred Write Error - See clause 6.2.7. |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3. |  |  |

Table 89 –Setmax Error

| Word | Name | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:5 | N/A |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4. |  |
|  |  |  | 3 | N/A |  |
|  |  |  | 2 | Abort - See Clause 6.3.1. |  |
|  |  |  | 1 | N/A |  |
|  |  |  | 0 | Obsolete [Editors Note: S/B N/A] |  |
| 01h | Count | Reserved |  |  |  |
| 02h-04h | LBA | Reserved |  |  |  |
| 05h | Status |  |  |  |  |
|  |  |  | Bit | Description |  |
|  |  |  | 15:8 | Reserved |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |
|  |  |  | 4 | N/A |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |
|  |  |  | 2:1 | N/A |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |

Table 90 –SMART Error

| Word | Name | Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:5 | N/A |  |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4. |  |  |  |
|  |  |  | 3 | N/A |  |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1. |  |  |  |
|  |  |  | 1 | N/A |  |  |  |
|  |  |  | 0 | Obsolete |  |  |  |
| 01h | Count | Reserved |  |  |  |  |  |
| 02h-04h | LBA |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 47:24 | Reserved |  |  |  |
|  |  |  | 23:8 |  |  |  |  |
|  |  |  |  |  | Value | Description |  |
|  |  |  |  |  | C24Fh | Subcommand specified a captive self-test and some error other than a self-test routine failure occurred (i.e., if the sub-command is not supported or register values are invalid) |  |
|  |  |  |  |  | 2CF4h | the subcommand specified a captive self-test routine which has failed during execution. |  |
|  |  |  |  |  | N/A | the subcommand specified an off-line routine including an off-line self-test routine. |  |
|  |  |  | 7:0 | N/A |  |  |  |
| 05h | Status |  |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |  |
|  |  |  | 15:8 | Reserved |  |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |  |
|  |  |  | 4 | N/A |  |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |  |
|  |  |  | 2:1 | N/A |  |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |  |

Table 91 –Writre Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | N/A |  |  |
|  |  |  | 6 | Write Protect - See clause 6.3.13 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1. |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | N/A |  |  |
| 01h | Count | Reserved |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Device Fault - See clause 6.2.4 |  |  |
|  |  |  | 4 | N/A |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3 |  |  |

Table 92 –Writre Stream DMA Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | Interface CRC - See clause 6.3.6 |  |  |
|  |  |  | 6 | Write Protect - See clause 6.3.13 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | Command Completion Time Limit Out - See clause 6.3.2 |  |  |
| 01h | Count | Length of Stream Error - number of contiguous sectors containing potentially bad data, beginning with the LBA of the first sector with an uncorrectable error. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Stream Error - See clause 6.2.10. |  |  |
|  |  |  | 4 | Deferred Write Error - See clause 6.2.7. |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3. |  |  |

Table 93 –Writre Stream Error

| Word | Name | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 00h | Error |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7 | N/A |  |  |
|  |  |  | 6 | Write Protect - See clause 6.3.13 |  |  |
|  |  |  | 5 | Media Change - See clause 6.3.7 |  |  |
|  |  |  | 4 | ID Not Found - See clause 6.3.4 |  |  |
|  |  |  | 3 | Media Change Request - See clause 6.3.8 |  |  |
|  |  |  | 2 | Abort - See clause 6.3.1 |  |  |
|  |  |  | 1 | No Media - See clause 6.3.10 |  |  |
|  |  |  | 0 | Command Completion Time Limit Out - See clause 6.3.2 |  |  |
| 01h | Count | Length of Stream Error - number of contiguous sectors containing potentially bad data, beginning with the LBA of the first sector with an uncorrectable error. |  |  |  |  |
| 02h | LBA | MSB |  | Address of first unrecoverable error. |  |  |
| 03h |  |  |  |  |  |  |
| 04h |  |  |  |  | LSB |  |
| 05h | Status |  |  |  |  |  |
|  |  |  | Bit | Description |  |  |
|  |  |  | 15:8 | Reserved |  |  |
|  |  |  | 7:6 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 5 | Stream Error - See clause 6.2.10. |  |  |
|  |  |  | 4 | Deferred Write Error - See clause 6.2.7. |  |  |
|  |  |  | 3 | Transport Dependent - See clause 6.2.11. |  |  |
|  |  |  | 2:1 | N/A |  |  |
|  |  |  | 0 | Error - See clause 6.2.3. |  |  |

- Bibliography (Informative)

Address Offset Reserved Area Boot, INCITS TR-27:2001

A.X.  Widmer and P.A.  Franaszek, “A DC-Balanced, Partitioned-Block, 8b/10b Transmission Code”.  IBM Journal of Research and Development, 27, no.  5: 440-451 (September, 1983)

U.S.  Patent 4,486,739.  Peter A.  Franaszek and Albert X.  Widmer.  Byte Oriented DC Balanced (0,4) 8b/10b Partitioned Block Transmission Code.  (December 4, 1984)

- Command Set summary (Informative)

The following four tables are provided to facilitate the understanding of the command set.  Table 94 provides information on which command codes are currently defined.  Table 95 provides a list of all of the commands in order of command code with the required use for each.  Table 96 provides a summary of all commands in alphabetical order with the required use for each.  Table 97 documents the assignment history of each opcode by ATA standard.  Table 98 documents the assignment history of each SET FEATURES code by ATA standard.

Table 94 - Command Matrix

|  | x0 | x1 | x2 | x3 | x4 | x5 | x6 | x7 |  | x8 | x9 | xA | xB | xC | xD | xE | xF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x | C | R | R | C | R | R | R | R |  | C | R | R | R | R | R | R | R |
| 1x | O | E | E | E | E | E | E | E |  | E | E | E | E | E | E | E | E |
| 2x | C | O | O | O | C | C | C | C |  | R | C | C | C | R | R | R | C |
| 3x | C | O | O | O | C | C | C | C |  | C | C | C | C | O | C | C | C |
| 4x | C | O | C | R | R | R | R | R |  | R | R | R | R | R | R | R | R |
| 5x | O | C* | R | R | R | R | R | R |  | R | R | R | R | T* | T* | T* | T* |
| 6x | S | S | S | S | S | S | S | S |  | R | R | R | R | R | R | R | R |
| 7x | O | E | E | E | E | E | E | E |  | E | E | E | E | E | E | E | E |
| 8x | V | V | V | V | V | V | V | F |  | V | V | V | V | V | V | V | V |
| 9x | C | O | C | R | E | E | E | E |  | E | E | V | R | R | R | R | R |
| Ax | C | C | C | R | R | R | R | R |  | R | R | R | R | R | R | R | R |
| Bx | C | C | R | R | R | R | R | R |  | A | A | A | A | A | A | A | A |
| Cx | F | V | V | V | C | C | C | C |  | C | O | C | O | C | C | C | R |
| Dx | R | C | M | M | M | R | R | R |  | R | R | C | E | E | E | C | C |
| Ex | C | C | C | C | C | C | C | C |  | C | E | C | R | C | C | O | C |
| Fx | V | C | C | C | C | C | C | V |  | C | C | V | V | V | V | V | V |
| Key: C = a defined command. R = Reserved, undefined in current specifications. V = Vendor specific commands. O = Obsolete. E=a retired command. F=If the device does not implement the CFA feature set, this command code is Vendor specific. |  |  |  |  |  |  |  |  | A=Reserved for assignment by the CompactFlash Association M=Reserved for the Media Card Pass Through Command feature set. S=Reserved for Serial ATA * indicates that the entry in this table has changed from ATA/ATAPI-7, INCITS xxx-xxxx. |  |  |  |  |  |  |  |  |

Table 95 - Command codes (sorted by command code)

| protocol | Command | Devices not implementing the PACKET Command feature set | Devices implementing the PACKET Command feature set | Command code |
| --- | --- | --- | --- | --- |
| ND | NOP | O | M | 00h |
| ND | CFA REQUEST EXTENDED ERROR | O | N | 03h |
| DR | DEVICE RESET | N | M | 08h |
| PI | READ SECTOR(S) | M | M | 20h |
| PI | READ SECTOR(S) EXT | O | N | 24h |
| DM | READ DMA EXT | O | N | 25h |
| DMQ | READ DMA QUEUED EXT | O | N | 26h |
| ND | READ NATIVE MAX ADDRESS EXT | O | N | 27h |
| PI | READ MULTIPLE EXT | O | N | 29h |
| DM | READ STREAM DMA EXT | O | N | 2Ah |
| PI | READ STREAM EXT | O | N | 2Bh |
| PI | READ LOG EXT | O | O | 2Fh |
| PO | WRITE SECTOR(S) | M | N | 30h |
| PO | WRITE SECTOR(S) EXT | O | N | 34h |
| DM | WRITE DMA EXT | O | N | 35h |
| DMQ | WRITE DMA QUEUED EXT | O | N | 36h |
| ND | SET MAX ADDRESS EXT | O | N | 37h |
| PO | CFA WRITE SECTORS WITHOUT ERASE | O | N | 38h |
| PO | WRITE MULTIPLE EXT | O | N | 39h |
| DM | WRITE STREAM DMA EXT | O | N | 3Ah |
| PO | WRITE STREAM EXT | O | N | 3Bh |
| DM | WRITE DMA FUA EXT | O | N | 3Dh |
| DMQ | WRITE DMA QUEUED FUA EXT | O | N | 3Eh |
| PO | WRITE LOG EXT | O | O | 3Fh |
| ND | READ VERIFY SECTOR(S) | M | N | 40h |
| ND | READ VERIFY SECTOR(S) EXT | O | N | 42h |
| ND | CONFIGURE STREAM | O | O | 51H |
| PI | CFA TRANSLATE SECTOR | O | N | 87h |
| DD | EXECUTE DEVICE DIAGNOSTIC | M | M | 90h |
| PO | DOWNLOAD MICROCODE | O | N | 92h |
| P | PACKET | N | M | A0h |
| PI | IDENTIFY PACKET DEVICE | N | M | A1h |
| P/DMQ | SERVICE | O | O | A2h |
| ND | SMART DISABLE OPERATIONS | O | N | B0h |
| ND | SMART ENABLE/DISABLE AUTOSAVE | O | N | B0h |
| ND | SMART ENABLE OPERATIONS | O | N | B0h |
| ND | SMART EXECUTE OFF-LINE IMMEDIATE | O | N | B0h |
| PI | SMART READ DATA | O | N | B0h |
| PI | SMART READ LOG | O | N | B0h |
| ND | SMART RETURN STATUS | O | N | B0h |
| PO | SMART WRITE LOG | O | N | B0h |
| ND | DEVICE CONFIGURATION FREEZE LOCK | O | O | B1h |
| PI | DEVICE CONFIGURATION IDENTIFY | O | O | B1h |
| ND | DEVICE CONFIGURATION RESTORE | O | O | B1h |
| PO | DEVICE CONFIGURATION SET | O | O | B1h |

(continued)

Table 95 - Command codes (sorted by command code) (continued)

| protocol | Command | Devices not implementing the PACKET Command feature set | Devices implementing the PACKET Command feature set | Command code |
| --- | --- | --- | --- | --- |
| ND | CFA ERASE SECTORS | F | N | C0h |
| PI | READ MULTIPLE | M | N | C4h |
| PO | WRITE MULTIPLE | M | N | C5h |
| ND | SET MULTIPLE MODE | M | N | C6h |
| DMQ | READ DMA QUEUED | O | N | C7h |
| DM | READ DMA | M | N | C8h |
| DM | WRITE DMA | M | N | CAh |
| DMQ | WRITE DMA QUEUED | O | N | CCh |
| PO | CFA WRITE MULTIPLE WITHOUT ERASE | O | N | CDh |
| PO | WRITE MULTIPLE FUA EXT | O | N | CEh |
| ND | CHECK MEDIA CARD TYPE | O | N | D1h |
| ND | GET MEDIA STATUS | O | O | DAh |
| ND | MEDIA LOCK | O | N | DEh |
| ND | MEDIA UNLOCK | O | N | DFh |
| ND | STANDBY IMMEDIATE | M | M | E0h |
| ND | IDLE IMMEDIATE | M | M | E1h |
| ND | STANDBY | M | O | E2h |
| ND | IDLE | M | O | E3h |
| PI | READ BUFFER | O | N | E4h |
| ND | CHECK POWER MODE | M | M | E5h |
| ND | SLEEP | M | M | E6h |
| ND | FLUSH CACHE | M | O | E7h |
| PO | WRITE BUFFER | O | N | E8h |
| ND | FLUSH CACHE EXT | O | N | EAh |
| PI | IDENTIFY DEVICE | M | M | ECh |
| ND | MEDIA EJECT | O | N | EDh |
| ND | SET FEATURES | M | M | EFh |
| PO | SECURITY SET PASSWORD | O | O | F1h |
| PO | SECURITY UNLOCK | O | O | F2h |
| ND | SECURITY ERASE PREPARE | O | O | F3h |
| PO | SECURITY ERASE UNIT | O | O | F4h |
| ND | SECURITY FREEZE LOCK | O | O | F5h |
| PO | SECURITY DISABLE PASSWORD | O | O | F6h |
| ND | READ NATIVE MAX ADDRESS | O | O | F8h |
| ND | SET MAX ADDRESS | O | O | F9h |

(continued)

Table 95 - Command codes (sorted by command code) (continued)

| protocol | Command | Devices not implementing the PACKET Command feature set | Devices implementing the PACKET Command feature set | Command code |
| --- | --- | --- | --- | --- |
| VS | Vendor specific | V | V | 9Ah,C0h-C3h,8xh, F0h,F7h, FAh-FFh |
| - | Retired | E | E | 11h-1Fh, 71h-7Fh, 94h-99h, DBh-DDh, E9h |
| - | Obsolete | B | B | 10h, 21h-23h, 31h-33h, 3Ch, 41h, 50h, 70h C9h, CBh, EEh |
| - | Reserved: all remaining codes | R | R |  |
| Key: |  |  |  |  |
| ND = Non-data command |  | M = Mandatory |  |  |
| PI = PIO data-in command |  | O = Optional |  |  |
| PO = PIO data-out command |  | N = Use prohibited |  |  |
| DM = DMA command |  | V = Vendor specific implementation |  |  |
| DMQ = DMA QUEUED command |  | E = Retired |  |  |
| DR = DEVICE RESET command |  | B = Obsolete |  |  |
| DD = EXECUTE DEVICE DIAGNOSTIC command |  | R = Reserved |  |  |
| P = PACKET command |  | F=If the device does not implement the CFA feature set, this command code is Vendor specific. |  |  |
| VS = Vendor specfic |  |  |  |  |

(concluded)

Table 96- Command codes (sorted by command)

| protocol | Command | Devices not implementing the PACKET Command feature set | Devices implementing the PACKET Command feature set | Command code |
| --- | --- | --- | --- | --- |
| ND | CFA ERASE SECTORS | F | N | C0h |
| ND | CFA REQUEST EXTENDED ERROR | O | N | 03h |
| PI | CFA TRANSLATE SECTOR | O | N | 87h |
| PO | CFA WRITE MULTIPLE WITHOUT ERASE | O | N | CDh |
| PO | CFA WRITE SECTORS WITHOUT ERASE | O | N | 38h |
| ND | CHECK MEDIA CARD TYPE | O | N | D1h |
| ND | CHECK POWER MODE | M | M | E5h |
| ND | CONFIGURE STREAM | O | O | 51H |
| ND | DEVICE CONFIGURATION FREEZE LOCK | O | O | B1h |
| PI | DEVICE CONFIGURATION IDENTIFY | O | O | B1h |
| ND | DEVICE CONFIGURATION RESTORE | O | O | B1h |
| PO | DEVICE CONFIGURATION SET | O | O | B1h |
| DR | DEVICE RESET | N | M | 08h |
| PO | DOWNLOAD MICROCODE | O | N | 92h |
| DD | EXECUTE DEVICE DIAGNOSTIC | M | M | 90h |
| ND | FLUSH CACHE | M | O | E7h |
| ND | FLUSH CACHE EXT | O | N | EAh |
| ND | GET MEDIA STATUS | O | O | DAh |
| PI | IDENTIFY DEVICE | M | M | ECh |
| PI | IDENTIFY PACKET DEVICE | N | M | A1h |
| ND | IDLE | M | O | E3h |
| ND | IDLE IMMEDIATE | M | M | E1h |
| ND | MEDIA EJECT | O | N | EDh |
| ND | MEDIA LOCK | O | N | DEh |
| ND | MEDIA UNLOCK | O | N | DFh |
| ND | NOP | O | M | 00h |
| P | PACKET | N | M | A0h |
| PI | READ BUFFER | O | N | E4h |
| DM | READ DMA | M | N | C8h |
| DM | READ DMA EXT | O | N | 25h |
| DMQ | READ DMA QUEUED | O | N | C7h |
| DMQ | READ DMA QUEUED EXT | O | N | 26h |
| PI | READ LOG EXT | O | O | 2Fh |
| PI | READ MULTIPLE | M | N | C4h |
| PI | READ MULTIPLE EXT | O | N | 29h |
| ND | READ NATIVE MAX ADDRESS | O | O | F8h |
| ND | READ NATIVE MAX ADDRESS EXT | O | N | 27h |
| PI | READ SECTOR(S) | M | M | 20h |
| PI | READ SECTOR(S) EXT | O | N | 24h |
| DM | READ STREAM DMA EXT | O | N | 2Ah |
| PI | READ STREAM EXT | O | N | 2Bh |
| ND | READ VERIFY SECTOR(S) | M | N | 40h |
| ND | READ VERIFY SECTOR(S) EXT | O | N | 42h |
| PO | SECURITY DISABLE PASSWORD | O | O | F6h |
| ND | SECURITY ERASE PREPARE | O | O | F3h |

(continued)

Table 96 - Command codes (sorted by command) (continued)

| protocol | Command | Devices not implementing the PACKET Command feature set | Devices implementing the PACKET Command feature set | Command code |
| --- | --- | --- | --- | --- |
| PO | SECURITY ERASE UNIT | O | O | F4h |
| ND | SECURITY FREEZE LOCK | O | O | F5h |
| PO | SECURITY SET PASSWORD | O | O | F1h |
| PO | SECURITY UNLOCK | O | O | F2h |
| P/DMQ | SERVICE | O | O | A2h |
| ND | SET FEATURES | M | M | EFh |
| ND | SET MAX | O | O | F9h |
| ND | SET MAX ADDRESS EXT | O | N | 37h |
| ND | SET MULTIPLE MODE | M | N | C6h |
| ND | SLEEP | M | M | E6h |
| ND | SMART DISABLE OPERATIONS | O | N | B0h |
| ND | SMART ENABLE/DISABLE AUTOSAVE | O | N | B0h |
| ND | SMART ENABLE OPERATIONS | O | N | B0h |
| ND | SMART EXECUTE OFF-LINE IMMEDIATE | O | N | B0h |
| PI | SMART READ DATA | O | N | B0h |
| PI | SMART READ LOG | O | N | B0h |
| ND | SMART RETURN STATUS | O | N | B0h |
| PO | SMART WRITE LOG | O | N | B0h |
| ND | STANDBY | M | O | E2h |
| ND | STANDBY IMMEDIATE | M | M | E0h |
| PO | WRITE BUFFER | O | N | E8h |
| DM | WRITE DMA | M | N | CAh |
| DM | WRITE DMA EXT | O | N | 35h |
| DM | WRITE DMA FUA EXT | O | N | 3Dh |
| DMQ | WRITE DMA QUEUED | O | N | CCh |
| DMQ | WRITE DMA QUEUED EXT | O | N | 36h |
| DMQ | WRITE DMA QUEUED FUA EXT | O | N | 3Eh |
| PO | WRITE LOG EXT | O | O | 3Fh |
| PO | WRITE MULTIPLE | M | N | C5h |
| PO | WRITE MULTIPLE EXT | O | N | 39h |
| PO | WRITE MULTIPLE FUA EXT | O | N | CEh |
| PO | WRITE SECTOR(S) | M | N | 30h |
| PO | WRITE SECTOR(S) EXT | O | N | 34h |
| DM | WRITE STREAM DMA EXT | O | N | 3Ah |
| PO | WRITE STREAM EXT | O | N | 3Bh |

(continued)

Table 96 - Command codes (sorted by command) (continued)

| VS | Vendor specific | V | V | 9Ah,C0h-C3h,8xh, F0h,F7h, FAh-FFh |
| --- | --- | --- | --- | --- |
| - | Retired | E | E | 11h-1Fh, 71h-7Fh, 94h-99h, DBh-DDh, E9h |
| - | Obsolete | B | B | 10h, 21h-23h, 31h-33h, 3Ch, 41h, 50h, 70h, C9h, CBh, EEh |
| - | Reserved: all remaining codes | R | R |  |
| Key: |  |  |  |  |
| ND = Non-data command |  | M = Mandatory |  |  |
| PI = PIO data-in command |  | O = Optional |  |  |
| PO = PIO data-out command |  | N = Use prohibited |  |  |
| DM = DMA command |  | V = Vendor specific implementation |  |  |
| DMQ = DMA QUEUED command |  | E = Retired |  |  |
| DR = DEVICE RESET command |  | B = Obsolete |  |  |
| DD = EXECUTE DEVICE DIAGNOSTIC command |  | R = Reserved |  |  |
| P = PACKET command |  | F=If the device does not implement the CFA feature set, this command code is Vendor specific. |  |  |
| VS = Vendor specfic |  |  |  |  |

(concluded)

| Table 97 - Historical Command Assignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Opcode | Command Name | ATA1 | ATA2 | ATA3 | ATA4 | ATA5 | ATA6 | ATA7 | ATA8 |
| 00h | NO-OP | C | C | C | C | C | C | C | C |
| 01h |  | R | R | R | R | R | R | R | R |
| 02h |  | R | R | R | R | R | R | R | R |
| 03h | CFA REQUEST EXTENDED ERROR | R | R | R | C | C | C | C | C |
| 04h |  | R | R | R | R | R | R | R | R |
| 05h |  | R | R | R | R | R | R | R | R |
| 06h |  | R | R | R | R | R | R | R | R |
| 07h |  | R | R | R | R | R | R | R | R |
| 08h | ATAPI Soft Reset / DEVICE RESET | R | R | ATAPI soft reset | DEVICE RESET | C | C | C | C |
| 09h |  | R | R | R | R | R | R | R | R |
| 0Ah |  | R | R | R | R | R | R | R | R |
| 0Bh |  | R | R | R | R | R | R | R | R |
| 0Ch |  | R | R | R | R | R | R | R | R |
| 0Dh |  | R | R | R | R | R | R | R | R |
| 0Eh |  | R | R | R | R | R | R | R | R |
| 0Fh |  | R | R | R | R | R | R | R | R |
| 10h | RECALIBRATE | C | C | C | O | O | O | O | O |
| 11h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 12h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 13h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 14h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 15h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 16h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 17h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 18h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 19h | RECALIBRATE | C | C | O | E | E | E | E | E |
| 1Ah | RECALIBRATE | C | C | O | E | E | E | E | E |
| 1Bh | RECALIBRATE | C | C | O | E | E | E | E | E |
| 1Ch | RECALIBRATE | C | C | O | E | E | E | E | E |
| 1Dh | RECALIBRATE | C | C | O | E | E | E | E | E |
| 1Eh | RECALIBRATE | C | C | O | E | E | E | E | E |
| 1Fh | RECALIBRATE | C | C | O | E | E | E | E | E |
| 20h | READ SECTORS | C | C | C | C | C | C | C | C |
| 21h | READ SECTORS WITHOUT RETRY | C | C | C | C | O | O | O | O |
| 22h | READ LONG | C | C | C | O | O | O | O | O |
| 23h | READ LONG WITHOUT RETRY | C | C | C | O | O | O | O | O |
| 24h | READ SECTORS EXT | R | R | R | R | R | C | C | C |
| 25h | READ DMA EXT | R | R | R | R | R | C | C | C |
| 26h | READ DMA QUEUED EXT | R | R | R | R | R | C | C | C |
| 27h | READ NATIVE MAX ADDRESS EXT | R | R | R | R | R | C | C | C |
| 28h |  | R | R | R | R | R | R | R | R |
| 29h | READ MULTIPLE EXT | R | R | R | R | R | C | C | C |
| 2Ah | READ STREAM DMA | R | R | R | R | R | R | C | C |
| 2Bh | READ STREAM | R | R | R | R | R | R | C | C |
| 2Ch |  | R | R | R | R | R | R | R | R |
| 2Dh |  | R | R | R | R | R | R | R | R |
| 2Eh |  | R | R | R | R | R | R | R | R |
| 2Fh | READ LOG EXT | R | R | R | R | R | C | C | C |
| 30h | WRITE SECTORS | C | C | C | C | C | C | C | C |
| 31h | WRITE SECTORS WITHOUT RETRY | C | C | C | C | O | O | O | O |
| 32h | WRITE LONG | C | C | C | O | O | O | O | O |
| 33h | WRITE LONG WITHOUT RETRY | C | C | C | O | O | O | O | O |
| 34h | WRITE SECTORS EXT | R | R | R | R | O | C | C | C |
| 35h | WRITE DMA EXT | R | R | R | R | R | C | C | C |
| 36h | WRITE DMA QUEUED EXT | R | R | R | R | R | C | C | C |
| 37h | SET NATIVE MAX ADDRESS EXT | R | R | R | R | R | C | C | C |
| 38h | CFA WRITE SECTORS WITHOUT ERASE | R | R | R | C | C | C | C | C |
| 39h | WRITE MULTIPLE EXT | R | R | R | R | R | C | C | C |
| 3Ah | WRITE STREAM DMA | R | R | R | R | R | R | C | C |
| 3Bh | WRITE STREAM | R | R | R | R | R | R | C | C |
| 3Ch | WRITE VERIFY | C | C | C | O | O | O | O | O |
| 3Dh | WRITE DMA FUA EXT | R | R | R | R | R | R | C | C |
| 3Eh | WRITE DMA QUEUED FUA EXT | R | R | R | R | R | R | C | C |
| 3Fh | WRITE LOG EXT | R | R | R | R | R | C | C | C |
| 40h | READ VERIFY SECTORS | C | C | C | C | C | C | C | C |
| 41h | READ VERIFY SECTORS WITHOUT RETRY | C | C | C | C | O | O | O | O |
| 42h | WRITE VERIFY SECTORS EXT | R | R | R | R | R | C | C | C |
| 43h |  | R | R | R | R | R | R | R | R |
| 44h |  | R | R | R | R | R | R | R | R |
| 45h |  | R | R | R | R | R | R | R | R |
| 46h |  | R | R | R | R | R | R | R | R |
| 47h |  | R | R | R | R | R | R | R | R |
| 48h |  | R | R | R | R | R | R | R | R |
| 49h |  | R | R | R | R | R | R | R | R |
| 4Ah |  | R | R | R | R | R | R | R | R |
| 4Bh |  | R | R | R | R | R | R | R | R |
| 4Ch |  | R | R | R | R | R | R | R | R |
| 4Dh |  | R | R | R | R | R | R | R | R |
| 4Eh |  | R | R | R | R | R | R | R | R |
| 4Fh |  | R | R | R | R | R | R | R | R |
| 50h | FORMAT TRACK | C | C | C | O | O | O | O | O |
| 51h | CONFIGURE STREAM | R | R | R | R | R | R | C | C |
| 52h |  | R | R | R | R | R | R | R | R |
| 53h |  | R | R | R | R | R | R | R | R |
| 54h |  | R | R | R | R | R | R | R | R |
| 55h |  | R | R | R | R | R | R | R | R |
| 56h |  | R | R | R | R | R | R | R | R |
| 57h |  | R | R | R | R | R | R | R | R |
| 58h |  | R | R | R | R | R | R | R | R |
| 59h |  | R | R | R | R | R | R | R | R |
| 5Ah |  | R | R | R | R | R | R | R | R |
| 5Bh |  | R | R | R | R | R | R | R | R |
| 5Ch | TRUSTED RECEIVE | R | R | R | R | R | R | R | Y |
| 5Dh | TRUSTED RECEIVE DMA | R | R | R | R | R | R | R | Y |
| 5Eh | TRUSTED SEND | R | R | R | R | R | R | R | Y |
| 5Fh | TRUSTED SEND DMA | R | R | R | R | R | R | R | Y |
| 60h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 61h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 62h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 63h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 64h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 65h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 66h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 67h | SATA (reserved) | R | R | R | R | R | R | S | S |
| 68h |  | R | R | R | R | R | R | S | S |
| 69h |  | R | R | R | R | R | R | S | S |
| 6Ah |  | R | R | R | R | R | R | S | S |
| 6Bh |  | R | R | R | R | R | R | S | S |
| 6Ch |  | R | R | R | R | R | R | S | S |
| 6Dh |  | R | R | R | R | R | R | S | S |
| 6Eh |  | R | R | R | R | R | R | S | S |
| 6Fh |  | R | R | R | R | R | R | S | S |
| 70h | SEEK | C | C | C | C | C | C | O | O |
| 71h | SEEK | C | C | O | E | E | E | E | E |
| 72h | SEEK | C | C | O | E | E | E | E | E |
| 73h | SEEK | C | C | O | E | E | E | E | E |
| 74h | SEEK | C | C | O | E | E | E | E | E |
| 75h | SEEK | C | C | O | E | E | E | E | E |
| 76h | SEEK | C | C | O | E | E | E | E | E |
| 77h | SEEK | C | C | O | E | E | E | E | E |
| 78h | SEEK | C | C | O | E | E | E | E | E |
| 79h | SEEK | C | C | O | E | E | E | E | E |
| 7Ah | SEEK | C | C | O | E | E | E | E | E |
| 7Bh | SEEK | C | C | O | E | E | E | E | E |
| 7Ch | SEEK | C | C | O | E | E | E | E | E |
| 7Dh | SEEK | C | C | O | E | E | E | E | E |
| 7Eh | SEEK | C | C | O | E | E | E | E | E |
| 7Fh | SEEK | C | C | O | E | E | E | E | E |
| 80h | (vendor unique) | V | V | V | V | V | V | V | V |
| 81h | (vendor unique) | V | V | V | V | V | V | V | V |
| 82h | (vendor unique) | V | V | V | V | V | V | V | V |
| 83h | (vendor unique) | V | V | V | V | V | V | V | V |
| 84h | (vendor unique) | V | V | V | V | V | V | V | V |
| 85h | (vendor unique) | V | V | V | V | V | V | V | V |
| 86h | (vendor unique) | V | V | V | V | V | V | V | V |
| 87h | (vendor unique) / CFA TRANSLATE SECTOR | V | V | V | F | F | F | F | F |
| 88h | (vendor unique) | V | V | V | V | V | V | V | V |
| 89h | (vendor unique) | V | V | V | V | V | V | V | V |
| 8Ah | (vendor unique) | V | V | V | V | V | V | V | V |
| 8Bh | (vendor unique) | V | V | V | V | V | V | V | V |
| 8Ch | (vendor unique) | V | V | V | V | V | V | V | V |
| 8Dh | (vendor unique) | V | V | V | V | V | V | V | V |
| 8Eh | (vendor unique) | V | V | V | V | V | V | V | V |
| 8Fh | (vendor unique) | V | V | V | V | V | V | V | V |
| 90h | EXECUTE DEVICE DIAGNOSTICS | C | C | C | C | C | C | C | C |
| 91h | INITIALIZE DEVICE PARAMETERS | C | C | C | C | C | O | O | O |
| 92h | DOWNLOAD MICROCODE | R | C | C | C | C | C | C | C |
| 93h |  | R | R | R | R | R | R | R | R |
| 94h | STANDBY IMMEDIATE | C | C | C | E | E | E | E | E |
| 95h | IDLE IMMEDIATE | C | C | C | E | E | E | E | E |
| 96h | STANDBY | C | C | C | E | E | E | E | E |
| 97h | IDLE | C | C | C | E | E | E | E | E |
| 98h | CHECK POWER MODE | C | C | C | E | E | E | E | E |
| 99h | SLEEP | C | C | C | E | E | E | E | E |
| 9Ah | (vendor unique) | V | V | V | V | V | V | V | V |
| 9Bh |  | R | R | R | R | R | R | R | R |
| 9Ch |  | R | R | R | R | R | R | R | R |
| 9Dh |  | R | R | R | R | R | R | R | R |
| 9Eh |  | R | R | R | R | R | R | R | R |
| 9Fh |  | R | R | R | R | R | R | R | R |
| A0h | PACKET | R | R | C | C | C | C | C | C |
| A1h | IDENTIFY PACKET DEVICE | R | R | C | C | C | C | C | C |
| A2h | SERVICE | R | R | C | C | C | C | C | C |
| A3h |  | R | R | R | R | R | R | R | R |
| A4h |  | R | R | R | R | R | R | R | R |
| A5h |  | R | R | R | R | R | R | R | R |
| A6h |  | R | R | R | R | R | R | R | R |
| A7h |  | R | R | R | R | R | R | R | R |
| A8h |  | R | R | R | R | R | R | R | R |
| A9h |  | R | R | R | R | R | R | R | R |
| AAh |  | R | R | R | R | R | R | R | R |
| ABh |  | R | R | R | R | R | R | R | R |
| ACh |  | R | R | R | R | R | R | R | R |
| ADh |  | R | R | R | R | R | R | R | R |
| AEh |  | R | R | R | R | R | R | R | R |
| AFh |  | R | R | R | R | R | R | R | R |
| B0h | SMART | R | R | C | C | C | C | C | C |
| B1h | CFA DEVICE CONFIGURATION | R | R | R | R | R | C | C | C |
| B2h |  | R | R | R | R | R | R | R | R |
| B3h |  | R | R | R | R | R | R | R | R |
| B4h |  | R | R | R | R | R | R | R | R |
| B5h |  | R | R | R | R | R | R | R | R |
| B6h |  | R | R | R | R | R | R | R | R |
| B7h |  | R | R | R | R | R | R | R | R |
| B8h | CFA (reserved) | R | R | R | R | A | A | A | A |
| B9h | CFA (reserved) | R | R | R | R | A | A | A | A |
| BAh | CFA (reserved) | R | R | R | R | A | A | A | A |
| BBh | CFA (reserved) | R | R | R | R | A | A | A | A |
| BCh | CFA (reserved) | R | R | R | R | A | A | A | A |
| BDh | CFA (reserved) | R | R | R | R | A | A | A | A |
| BEh | CFA (reserved) | R | R | R | R | A | A | A | A |
| BFh | CFA (reserved) | R | R | R | R | A | A | A | A |
| C0h | (vendor unique) / CFA ERASE SECTORS | V | V | V | F | F | F | F | F |
| C1h | (vendor unique) | V | V | V | V | V | V | V | V |
| C2h | (vendor unique) | V | V | V | V | V | V | V | V |
| C3h | (vendor unique) | V | V | V | V | V | V | V | V |
| C4h | READ MULTIPLE | C | C | C | C | C | C | C | C |
| C5h | WRITE MULTIPLE | C | C | C | C | C | C | C | C |
| C6h | SET MULTIPLE MODE | C | C | C | C | C | C | C | C |
| C7h | READ DMA QUEUED | R | R | R | C | C | C | C | C |
| C8h | READ DMA | C | C | C | C | C | C | C | C |
| C9h | READ DMA WITHOUT RETRIES | C | C | C | C | O | O | O | O |
| CAh | WRITE DMA | C | C | C | C | C | C | C | C |
| CBh | WRITE DMA WITHOUT RETRIES | C | C | C | C | O | O | O | O |
| CCh | WRITE DMA QUEUED | R | R | R | C | C | C | C | C |
| CDh | CFA WRITE MULTIPLE WITHOUT ERASE | R | R | R | C | C | C | C | C |
| CEh | WRITE MULTIPLE FUA EXT | R | R | R | R | R | R | C | C |
| CFh |  | R | R | R | R | R | R | R | R |
| D0h |  | R | R | R | R | R | R | R | R |
| D1h | CHECK MEDIA CARD TYPE | R | R | R | R | R | C | C | C |
| D2h | MediaCardPassThru (reserved) | R | R | R | R | R | M | M | M |
| D3h | MediaCardPassThru (reserved) | R | R | R | R | R | M | M | M |
| D4h | MediaCardPassThru (reserved) | R | R | R | R | R | M | M | M |
| D5h |  | R | R | R | R | R | R | R | R |
| D6h |  | R | R | R | R | R | R | R | R |
| D7h |  | R | R | R | R | R | R | R | R |
| D8h |  | R | R | R | R | R | R | R | R |
| D9h |  | R | R | R | R | R | R | R | R |
| DAh | GET MEDIA STATUS | R | R | R | C | C | C | C | C |
| DBh | ACKNOWLEDGE MEDIA CHANGE | C | C | O | E | E | E | E | E |
| DCh | BOOT POST BOOT | C | C | O | E | E | E | E | E |
| DDh | BOOT PRE BOOT | C | C | O | E | E | E | E | E |
| DEh | MEDIA LOCK | C | C | C | C | C | C | C | C |
| DFh | MEDIA UNLOCK | C | C | C | C | C | C | C | C |
| E0h | STANDBY IMMEDIATE | C | C | C | C | C | C | C | C |
| E1h | IDLE IMMEDIATE | C | C | C | C | C | C | C | C |
| E2h | STANDBY | C | C | C | C | C | C | C | C |
| E3h | IDLE | C | C | C | C | C | C | C | C |
| E4h | READ BUFFER | C | C | C | C | C | C | C | C |
| E5h | CHECK POWER MODE | C | C | C | C | C | C | C | C |
| E6h | SLEEP | C | C | C | C | C | C | C | C |
| E7h | FLUSH CACHE | R | R | R | C | C | C | C | C |
| E8h | WRITE BUFFER | C | C | C | C | C | C | C | C |
| E9h | WRITE SAME | C | C | O | E | E | E | E | E |
| EAh | FLUSH CACHE EXT | R | R | R | R | R | C | C | C |
| EBh |  | R | R | R | R | R | R | R | R |
| ECh | IDENTIFY DEVICE | C | C | C | C | C | C | C | C |
| EDh | MEDIA EJECT | R | C | C | C | C | C | C | C |
| EEh | IDENTIFY DEVICE DMA | R | R | C | O | O | O | O | O |
| EFh | SET FEATURES | C | C | C | C | C | C | C | C |
| F0h | (vendor unique) | V | V | V | V | V | V | V | V |
| F1h | SECURITY SET PASSWORD | V | V | C | C | C | C | C | C |
| F2h | SECURITY UNLOCK | V | V | C | C | C | C | C | C |
| F3h | SECURITY ERASE PREPARE | V | V | C | C | C | C | C | C |
| F4h | SECURITY ERASE UNIT | V | V | C | C | C | C | C | C |
| F5h | SECURITY FREEZE LOCK | V | V | C | C | C | C | C | C |
| F6h | SECURITY DISABLE PASSWORD | V | V | C | C | C | C | C | C |
| F7h | (vendor unique) | V | V | V | V | V | V | V | V |
| F8h | READ NATIVE MAX ADDRESS | V | V | V | C | C | C | C | C |
| F9h | SET MAX ADDRESS | V | V | V | C | C | C | C | C |
| FAh | (vendor unique) | V | V | V | V | V | V | V | V |
| FBh | (vendor unique) | V | V | V | V | V | V | V | V |
| FCh | (vendor unique) | V | V | V | V | V | V | V | V |
| FDh | (vendor unique) | V | V | V | V | V | V | V | V |
| FEh | (vendor unique) | V | V | V | V | V | V | V | V |
| FFh | (vendor unique) | V | V | V | V | V | V | V | V |
| Key: C= a defined command. E= a retired command. O= Obsolete. R= Reserved, undefined in current specifications. V= Vendor specific commands. |  | A=Reserved for assignment by the CompactFlash Association F= If the device does not implement the CFA feature set, this command code is Vendor specific. M=Reserved for the Media Card Pass Through Command feature set. S=Reserved for Serial ATA Y=Reserved for Trusted Computing |  |  |  |  |  |  |  |

| Table 98 - Historical SET FEATURE Code Assignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Feature Code | Description | ATA1 | ATA2 | ATA3 | ATA4 | ATA5 | ATA6 | ATA7 | ATA8 |
| 01h | Enable 8-bit data transfers | C | C | O | E | F | F | F | F |
| 02h | Enable write cache | V | V | C | C | C | C | C | C |
| 03h | Set transfer mode based on value in sector count register | C | C | C | C | C | C | C | C |
| 04h | Enable all automatic defect reassignment | R | R | C | O | O | O | O | O |
| 05h | Enable advanced power management | R | R | R | C | C | C | C | C |
| 06h | Enable Power-Up in Standby feature set | R | R | R | R | C | C | C | C |
| 07h | Power-up in Standby feature set device spin-up | R | R | R | R | C | C | C | C |
| 09h | Reserved for Address offset reserved boot area method technical report | R | R | R | R | C | C | C | C |
| 0Ah | Enable CFA power mode 1 | R | R | R | R | C | C | C | C |
| 20h | Reserved for Technical Report (T13/DT1696) | R | R | R | R | R | R | T | T |
| 21h | Reserved for Technical Report (T13/DT1696) | R | R | R | R | R | R | T | T |
| 31h | Disable Media Status Notification | R | R | R | C | C | C | C | C |
| 33h | Disable retry | V | V | C | C | O | O | O | O |
| 42h | Enable Automatic Acoustic Management feature set | R | R | R | R | R | R | C | C |
| 43h | Set Maximum Host Interface Sector Times | R | R | R | R | R | R | C | C |
| 44h | Vendor unique length of ECC on read long/write long commands | C | C | C | O | O | O | O | O |
| 54h | Set cache segments to sector count register value | V | V | C | O | O | O | O | O |
| 55h | Disable read look-ahead feature | C | C | C | C | C | C | C | C |
| 5Dh | Enable release interrupt | R | R | R | C | C | C | C | C |
| 5Eh | Enable SERVICE interrupt | R | R | R | C | C | C | C | C |
| 66h | Disable reverting to power on defaults | C | C | C | C | C | C | C | C |
| 77h | Disable ECC | V | V | C | O | O | O | O | O |
| 81h | Disable 8-bit data transfers | C | C | O | E | F | F | F | F |
| 82h | Disable write cache | V | V | C | C | C | C | C | C |
| 84h | Disable all automatic defect reassignment | R | R | C | O | O | O | O | O |
| 85h | Disable advanced power management | R | R | R | C | C | C | C | C |
| 86h | Disable Power-Up in Standby feature set | R | R | R | R | C | C | C | C |
| 88h | Enable ECC | V | V | C | C | C | O | O | O |
| 89h | Reserved for Address offset reserved boot area method technical report | R | R | R | R | C | C | C | C |
| 8Ah | Disable CFA power mode 1 | R | R | R | R | C | C | F | F |
| 90h | Reserved for Serial ATA | R | R | R | R | R | R | S | S |
| 95h | Enable Media Status Notification | R | R | R | C | C | C | C | C |
| 99h | Enable retries | V | V | C | O | O | O | O | O |
| 9Ah | Set device maximum average current | R | R | C | O | O | O | O | O |
| AAh | Enable read look-ahead features | C | C | C | C | C | C | C | C |
| ABh | Set maximum prefetch using sector count register value | V | V | C | O | O | O | O | O |
| BBh | 4 bytes of ECC apply on read long/write long commands | C | C | C | O | O | O | O | O |
| C2h | Disable Automatic Acoustic Management feature set | R | R | R | R | R | R | C | C |
| CCh | Enable reverting to power on defaults | C | C | C | C | C | C | C | C |
| DDh | Disable release interrupt | R | R | R | C | C | C | C | C |
| DEh | Disable SERVICE interrupt | R | R | R | C | C | C | C | C |
| E0h | Vendor unique | R | R | R | R | R | R | O | O |
| E1h | Vendor unique | R | R | R | R | R | R | O | O |
| E2h | Vendor unique | R | R | R | R | R | R | O | O |
| F0h |  | R | R | R | R | A | A | A | A |
| F1h |  | R | R | R | R | A | A | A | A |
| F2h |  | R | R | R | R | A | A | A | A |
| F3h |  | R | R | R | R | A | A | A | A |
| F4h |  | R | R | R | R | A | A | A | A |
| F5h |  | R | R | R | R | A | A | A | A |
| F6h |  | R | R | R | R | A | A | A | A |
| F7h |  | R | R | R | R | A | A | A | A |
| F8h |  | R | R | R | R | A | A | A | A |
| F9h |  | R | R | R | R | A | A | A | A |
| FAh |  | R | R | R | R | A | A | A | A |
| FBh |  | R | R | R | R | A | A | A | A |
| FCh |  | R | R | R | R | A | A | A | A |
| FDh |  | R | R | R | R | A | A | A | A |
| FEh |  | R | R | R | R | A | A | A | A |
| FFh |  | R | R | R | R | A | A | A | A |
| Key: C= a defined command. E= a retired command. O= Obsolete. R= Reserved, undefined in current specifications. V= Vendor specific commands. |  | A=Reserved for assignment by the CompactFlash Association. F= If the device does not implement the CFA feature set, this command code is Vendor specific. M=Reserved for the Media Card Pass Through Command feature set. S=Reserved for Serial ATA. T=Reserved for Technical Report T13/DT1696 (Time-Limited Commands). |  |  |  |  |  |  |  |

- Design and programming considerations for large physical sector devices (informative)

  - Introduction

Since the inception of the ATA interface the smallest addressable unit of data has been the 512 byte sector.  In hard disk drives each sector has an associated error correcting code field to allow detection and correction of read errors.  Over time, error correcting code fields have been lengthened to provide greater detection and correction capability.  As a result, the proportion of device media devoted to ECC fields has risen.  Increasing the length of data sectors on the media increases the efficiency of ECC by enabling better error detection and correction using a smaller proportion of media.

  - Physical sectors

Because the 512 byte sector has been a constant since the beginning of ATA many software changes would be required if device logical sectors were made larger.  To preserve the legacy software that assumes a 512 byte sector, logical addressing based on 512 byte sectors has been retained.  Larger physical sectors are implemented as power of two multiples of  512 byte logical sectors, 1,2,4,8,16, etc.  For example, devices may have physical sectors that are 8 logical sectors long or 4096 bytes total.  It is not possible to create a logical sector that spans two physical sectors.

  - Unaligned write

While retaining the 512 byte logical sector maintains software compatibility it introduces a potential performance issue, unaligned write, which must be avoided.  A physical sector must be written to the media in a single operation.  To complete a write command that writes a fraction of a physical sector the device must read the entire physical sector into buffer memory update the buffer memory with the write data and then write the entire physical sector to the media.  This will incur a performance penalty of at least a drive revolution.

Write commands can begin mid physical sector and end mid physical sector resulting in two unaligned writes.  In this case the device has to read both the beginning and ending physical sector of the write into the buffer.

To avoid the performance penalty from an unaligned write all write operations must begin with the first sector of a physical sector and end with the last sector of a physical sector.

The first logical sector must be the first 512 bytes of the first physical sector on the device.  This allows a host to align write operations with the physical sectors.

Supporting unaligned write operations is optional, but highly recommended to maintain backward compatibility with software.  See IDENTIFY DEVICE 7.17.

Figure 10 illustrates an unaligned write on a device with 2048 byte physical sectors.  The first four logical sectors, LBA0 - LBA3, reside on physical sector 0.  To write only LBA3 the host sends a conventional write command and the data for LBA3.  On receipt of the write command the device seeks to the physical sector that contains LBA3, which is physical sector 0.  Physical sector 0 is read into the device buffer.  Then the new write data for LBA3 is placed in the buffer, overwriting a segment of the buffer.  The buffer data is then written to the media, physical sector 0.

Figure 10 - Unaligned Write Example

  - SET MAX

Hosts which use the SET MAX command should set a value to the last logical sector of a physical sector to allow writes to the end of the user area without requiring an unaligned write.  Devices should accommodate a SET MAX setting to any LBA address to maintain compatibility.

  - Software compatibility

While the current specification allows devices to report up to 215 or 32,768 logical sectors per physical sector there are file system limitations in existing systems that restrict practical device implementations to 4096 bytes per physical sector.